List of stereotye to categorize subProfiles ShortCircuit Description Operation European Abstract Entsoe

# Concrete Classes

| CAEcEcoDuSnEtinnegrPgoy iSnht aring Reference Data Market Document assembly |  |
| --- | --- |
| model |  |
| An administrative point where Energy Supplier change can take place and for which commercial business processes are defined. Native Members administrativeStatus 0..1 String Specifies whether (or not) the AccountingPoint is active part of the imbalance settlement. Additional information: E.g., if still physically connected, but inactive in the market, the energy volume for the AccountingPoint will be part of the losses in the grid. energySharingEnergyDirection 0..1 String The Energy flow direction used in the Collective energy sharing process. energySharingParticipationFactor 0..1 Integer A value from 1 to 100, where 100 is the default value determining zthe participation of the final customer in the Collective En ergy Storing Unit (CESU). Lower values enable final customers to participate in more than one CESU. flexibilityContract 0..1 Boolean Indicates if there are contract(s) at this Accounting Point as a result of the demand side management or the load management for the Accounting Point. |  |

| gridAgreementType | 0..1 | String | Specification of type of grid contract, such as if the contract is directly between the Grid Company and the Grid Customer, or through the Energy Supplier. |
| --- | --- | --- | --- |
| meterReadingResolution | 0..1 | Duration | The length of time between the regular meter readings. |
| mRID | 0..1 | MeasurementPointID_String | A unique identification |

of the measurement point.

|  |  |  | In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification. |
| --- | --- | --- | --- |
|  |  |  | Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |
| settlementMethod | 0..1 | String | Specifies how the energy volumes are treated for settlement for the AccountingPoint, such as profiled or non- |

profiled.

| CAEsEseDtS Energy Sharing Reference Data Market Document assembly |  |
| --- | --- |
| model |  |
| asset is an energy data space specific attribute referring to the Asset from which data is being requested Native Members meterId 0..1 String meterId is an energy data space specific attribute that specifies the id of the meter that measures the asset. operatorId 0..1 String operatorId is an energy data space specific attribute that specifies the id operator of the asset refered to. type 0..1 String type is an energy data space specific attribute that specifies type of asset refered to. |  |

| CEEnEerDgSyCEonmermgyuSnhitayring Reference Data Market Document assembly |  |  |  |  |
| --- | --- | --- | --- | --- |
| model |  |  |  |  |
| Native Members |  |  |  |  |
| DateFrom | 0..1 | ESMP_DateTime |  |  |
| mRID | 0..1 | String | Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970- 552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |  |

| CEESERDDSMEDn_eErngvyeSlohpareing Reference Data Market Document assembly |  |
| --- | --- |
| model |  |
| The envelope entity is a non-CIM object used in an energy data space environment to serve as a central top- level entity connecting the MarketDocumentHeader (energy data space specific context information block) with |  |

the regular CIM MarketDocument.

Native Members

MarketDocument

[0..1]

ESRDMD_MarketDocument

MessageDocumentHeader

[0..1]

MessageDocumentHeader

| CEESERDDSMEDn_eMrgayrkSehtDaroincgumReefnetrence Data Market Document assembly |  |  |  |  |
| --- | --- | --- | --- | --- |
| model |  |  |  |  |
| An electronic document containing the information necessary to satisfy the requirements of a given business process. Native Members |  |  |  |  |
| createdDateTime | 0..1 | ESMP_DateTime | The date and time of the creation of the document. |  |
| description | 0..1 | String | The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. |  |
| mRID | 0..1 | ID_String | The unique identification of the document being exchanged within a business process flow. In the ESMP context, the "model authority" is defined as a party (originator of the exchange) that provides an identification in the context of a business exchange such as document identification, ... |  |

Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context.

Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this.

For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements.

receiver_MarketParticipant.marketRole.type 0..1	MarketRoleKind_String The identification

of the role played by a market player.

--- The MarketParticipant associated with an electronic document header.

--- The role associated with a MarketParticipant.

receiver_MarketParticipant.mRID	0..1	PartyID_String	The identification

of a party in the energy market.

In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement

point, resources (generator, lines, substations, etc.) identification.

Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context.

Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this.

For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements.

--- The MarketParticipant associated with an electronic document header.

receiver_MarketParticipant.name	0..1	String	The name is any

free human readable and possibly non unique text naming the object.

--- The MarketParticipant associated with an electronic document header.

revisionNumber	0..1	ESMPVersion_String	The identification of the version that distinguishes one evolution of a document from another.

sender_MarketParticipant.marketRole.type	0..1	MarketRoleKind_String The identification

of the role played by a market player.

--- The MarketParticipant associated with an electronic document header.

--- The role associated with a MarketParticipant.

sender_MarketParticipant.mRID	0..1	PartyID_String	The identification

of a party in the energy market.

In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification.

Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context.

Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this.

For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that

| identify CIM object elements. --- The MarketParticipant associated with an electronic document header |  |  |  |
| --- | --- | --- | --- |
| sender_MarketParticipant.name | 0..1 | String | The name is any |
|  |  |  | free human |
|  |  |  | readable and |
|  |  |  | possibly non |
|  |  |  | unique text |
|  |  |  | naming the |
|  |  |  | object. |
|  |  |  | --- The |
|  |  |  | MarketParticipant |
|  |  |  | associated with |
|  |  |  | an electronic |
|  |  |  | document header |
| type | 0..1 | MessageKind_String | The coded type of |
|  |  |  | a document. The |
|  |  |  | document type |
|  |  |  | describes the |
|  |  |  | principal |
|  |  |  | characteristic of |
|  |  |  | the document. |

.

.

| CMEeEsDsaSgEenDeorcguymSehnartiHnegaRdeefrerence Data Market Document assembly |  |
| --- | --- |
| model |  |
| MessageDocumentHeader is an energy data space specific entity with the creationDateTime of this document. Native Members creationDateTime 0..1 ESMP_DateTime The date and time the MessageDocumenHeader was created. MetaInformation [0..1] MetaInformation |  |

| CMEeEtaDInSfoErnmeragtyioSnharing Reference Data Market Document assembly |  |
| --- | --- |
| model |  |
| MetaInformation is an energy data space specific entity referring to the information concerning the technical and functional context of the MarketDocument. Native Members connectionId 0..1 String connectionId is an energy data |  |

|  |  |  | space specific attribute referring to the established connection used for the communication of this document. |
| --- | --- | --- | --- |
| dataNeedId | 0..1 | String | dataNeedId is an energy data |
|  |  |  | space specific attribute holding |
|  |  |  | the type of data need requested, |
|  |  |  | like ‘last 24 hours at 1 hour |
|  |  |  | interval”. |
| dataSourceId | 0..1 | String | dataSourceId is an energy data |
|  |  |  | space specific attribute referring |
|  |  |  | to the datasource from which the |
|  |  |  | requested data is being queried. |
| defaultValues | 0..1 | String | defaultValues is a energy data |
|  |  |  | space specific attribute to hold |
|  |  |  | relevant default values. The |
|  |  |  | format to use can freely be |
|  |  |  | chosen based on the |
|  |  |  | implementation. Most commonly |
|  |  |  | you would expect a semi-colon |
|  |  |  | separated name value pair, like: |
|  |  |  | "minPower=value; |
|  |  |  | maxPower=value; |
|  |  |  | avgPower=value". |
| documentType | 0..1 | String | documentType is an energy data |
|  |  |  | space specific attribute referring |
|  |  |  | to the type of document, |
|  |  |  | comparable to the |
|  |  |  | MessageTypeList in the CIM part |
|  |  |  | of the message. |
|  |  |  | Note: |
|  |  |  | This is applicable if you have the |
|  |  |  | need to know the content without |
|  |  |  | having to open the complete |
|  |  |  | document. DocumentType might |
|  |  |  | contain messages not (yet) part |
|  |  |  | of the MessageTypeList. |
| finalCustomerId | 0..1 | String | The finalCustomerId is an energy |
|  |  |  | data space specific attribute |
|  |  |  | identifying the actor doing the |
|  |  |  | request for data or data sharing. |
| regionConnector | 0..1 | String | regionConnector is an energy |
|  |  |  | data space specific attribute that |
|  |  |  | specifies the regional connector |
|  |  |  | being used to retrieve the data |
|  |  |  | requested. |

| regionCountry | 0..1 | String | regionCountry is an energy data space specific attribute that specifies the country where the regional connector is located. |
| --- | --- | --- | --- |
| requestPermissionId | 0..1 | String | The requestPermissionId is |
|  |  |  | energy data space specific |
|  |  |  | identification attribute holding an |
|  |  |  | unique identifier issued by the |
|  |  |  | energy data space to allow for |
|  |  |  | tracing of the permission through |
|  |  |  | serveral message exchanges. |
|  |  |  | Note: the requestPermissionId is |
|  |  |  | NOT the identifier issued by the |
|  |  |  | Identity Service Provider as |
|  |  |  | specified in the EU Implementing |
|  |  |  | Act on access to Metering and |
|  |  |  | Consumption Data. |
| Asset | [0..1] | Asset |  |

| CMEeEteDrSReEandeirnggy Sharing Reference Data Market Document assembly |  |
| --- | --- |
| model |  |
| Set of values obtained from the meter. Native Members mRID 0..1 ResourceID_String The unique identification of the meter reading. In the ESMP context, the "model authority" is defined as a party (originator of the exchange) that provides an identification in the context of a business exchange. Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970- 552 Edition 1, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non |  |

unique text naming the object.

| CPEroEcDeSssEnergy Sharing Reference Data Market Document assembly |  |
| --- | --- |
| model |  |
| The formal identification of the business process in which a flow of information is exchanged. Native Members classificationType 0..1 ClassificationKind_String The classification mechanism used to group a set of objects together within a business process. The grouping may be of a detailed or a summary nature. processType 0..1 ProcessKind_String The identification of the nature of process that the document addresses. |  |

# Enumerations

CodingSchemeTypeList	ESMPEnumerations

Codification scheme used to identify the coding scheme used for the set of coded values to identify specific objects.

EIC	The coding scheme is the Energy Identification Coding Scheme (EIC), maintained by ENTSO-E.

## Coding scheme which are part of GS1 system

The coding schemes maintained by GS1.

## Andorra National coding scheme

The National coding scheme of the country in question.

## Albania National coding scheme

The National coding scheme of the country in question.

## Armenia National coding scheme

The National coding scheme of the country in question.

## Austria National coding scheme

The National coding scheme of the country in question.

## Azerbaijan National coding scheme

The National coding scheme of the country in question.

## Bosnia and Herzegovina National coding scheme

The National coding scheme of the country in question.

## Belgium National coding scheme

The National coding scheme of the country in question.

## Bulgaria National coding scheme

The National coding scheme of the country in question.

## Switzerland National coding scheme

The National coding scheme of the country in question.

## Serbia and Montenegro National coding scheme

The National coding scheme of the country in question.

## Czech Republic National coding scheme

The National coding scheme of the country in question.

## Germany National coding scheme

The National coding scheme of the country in question.

## Denmark National coding scheme

The National coding scheme of the country in question.

## Estonia National coding scheme

The National coding scheme of the country in question.

## Spain National coding scheme

The National coding scheme of the country in question.

## Finland National coding scheme

The National coding scheme of the country in question.

## France National coding scheme

The National coding scheme of the country in question.

## United Kingdom National coding scheme

The National coding scheme of the country in question.

## Georgia National coding scheme

The National coding scheme of the country in question.

## Gibraltar National coding scheme

The National coding scheme of the country in question.

## Greece National coding scheme

The National coding scheme of the country in question.

## Croatia National coding scheme

The National coding scheme of the country in question.

## Hungary National coding scheme

The National coding scheme of the country in question.

## Ireland National coding scheme

The National coding scheme of the country in question.

## Italy National coding scheme

The National coding scheme of the country in question.

## Kyrgyzstan National coding scheme

The National coding scheme of the country in question.

## Kazakhstan National coding scheme

The National coding scheme of the country in question.

## Liechtenstein National coding scheme

The National coding scheme of the country in question.

## Lithuania National coding scheme

The National coding scheme of the country in question.

## Luxembourg National coding scheme

The National coding scheme of the country in question.

## Latvia National coding scheme

The National coding scheme of the country in question.

## Morocco National coding scheme

The National coding scheme of the country in question.

## Moldavia National coding scheme

The National coding scheme of the country in question.

## Macedonia National coding scheme

The National coding scheme of the country in question.

## Netherlands National coding scheme

The National coding scheme of the country in question.

## Nordic Regional coding scheme

The coding scheme of the Nordic region which covers Denmark, Finland, Norway and Sweden.

## Norway National coding scheme

The National coding scheme of the country in question.

## Poland National coding scheme

The National coding scheme of the country in question.

Portugal National coding The National coding scheme of the country in question.

## scheme

Romania National coding scheme

The National coding scheme of the country in question.

## Russian Federation National coding scheme

The National coding scheme of the country in question.

## Sweden National coding scheme

The National coding scheme of the country in question.

## Slovenia National coding scheme

The National coding scheme of the country in question.

## Slovakia National coding scheme

The National coding scheme of the country in question.

## Turkey National coding scheme

The National coding scheme of the country in question.

## Ukraine National coding scheme

The National coding scheme of the country in question.

CGM	The coding scheme used for Common Grid Model Exchange Standard (CGMES).

## Cyprus National coding scheme

The National coding scheme of the country in question.

ISO	The coding scheme for the preceding attribute is a code maintained by International Organization for Standardization (ISO), such as “ISO 3166 English country names and code elements”.

RoleTypeList

Identification of the role played by a party.

ESMPEnumerations

Trade responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Consumption responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Combined power exchange (not to be used)

This role is no longer in the ENTSO-E Harmonised Role Model Document.

System operator

Refer to role model definitions in the ENTSO-E Harmonised Role Model

## Imbalance settlement responsible

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Production responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Transmission capacity allocator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Balance responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Metered data aggregator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Billing agent	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Market operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Energy supplier	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Consumer	Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Control area operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Control block operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Coordination centre operator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Grid access provider	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Grid operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Meter administrator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Party connected to grid	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Producer	Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Profile maintenance party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Meter operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Metered data collector	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Metered data responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Metering point administrator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Resource Provider	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Scheduling coordinator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Capacity Trader	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Interconnection Trade Responsible

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Nomination Validator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Market information aggregator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

A party that collects information from different sources and assembles it to provide a summary of the market.

Information receiver	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

A party, not necessarily a market participant, which receives information

about the market.

Reserve Allocator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

A party that informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders.

MOL Responsible	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

A party that Informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders.

Capacity Coordinator	A party, acting on behalf of the SOs involved, responsible for establishing a

coordinated Offered Capacity and/or NTC and/or ATC between several Scheduling Areas.

## Reconciliation Accountable

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

A party that is financially accountable for the reconciled volume of energy products for a profiled local metering point.

## Reconciliation Responsible

A party that is responsible for reconciling, within a metering grid area, the volumes used in the imbalance settlement process for profiled metering points and the actual metered quantities.

Data provider	A party that is responsible for providing information to a central authority.

## Local Issuing Office (LIO)

A party that is responsible for operating a Local Issuing Office (LIO).

## Central Issuing Office (CIO)

A party that is responsible for operating a Central Issuing Office (CIO).

EIC Participant	A party that participates in the EIC environment.

Weather analyser	A party that analyses the current and forecast weather situation and

establishes a prognosis of its impact on the renewable energy environment as well as the overall load.

## Regional Security Coordinator (RSC)

The RSC as defined in the System Operation guideline.

## Energy Service Company (ESCO)

A party offering energy-related services to the Party Connected to Grid, but not directly active in the energy value chain or the physical infrastructure itself. The ESCO may provide insight services as well as energy management services.

## Balancing Service Provider

A party with reserve-providing units or reserve-providing groups able to provide balancing services to LFC Operators.

Energy trader	A party that is selling or buying energy.

LFC Operator	A party that is responsible for the Load Frequency Control of its LFC Area or

LFC Block.

## Transmission System Operator (TSO)

The Transmission System Operator (TSO) is responsible for the transport of electricity on the extra high-voltage and high-voltage interconnected system. This is a market participant and not a role in Harmonised Role Model.

## Distribution System Operator (DSO)

Distribution System Operator (DSO) is responsible for transport of electricity on high-voltage (optionally), medium-voltage and low-voltage distribution systems. This is a market participant and not a role in Harmonised Role Model.

## Resource Capacity Mechanism Operator

Resource capacity mechanism operator is the party responsible to operate the resource capacity mechanism in a member state. It can either be the TSO or an independent party.

Resource aggregator	A party that aggregates resources for usage by a service provider for energy

market services.

Cost sharing calculator	A party responsible for the mapping of the costs between areas and parties.

Settlement responsible	A party responsible for settling the costs.

## Coordinated Capacity Calculator

Coordinated Capacity Calculator is the entity or entities with the task of calculating transmission capacity, at regional level or above.

## Flexibility service provider

A party that offers flexibility services based on acquired (aggregated) resources.

Data access provider	A party responsible for facilitating access to data.

## Metered data administrator

A party responsible for storing and distributing validated measured data.

Permission administrator A party responsible for administrating a register of data access permissions

for a set of metering points, making this information available to final customers and eligible parties in the sector, on request.

MessageTypeList

ESMPEnumerations

The coded type of a document. The message type describes the principal characteristic of a document. This enumeration is used in the XML instances based on IEC 62325.

## Balance responsible schedule

A schedule which has been prepared by a balance responsible party providing planned schedule information.

## Allocated capacity schedule

A schedule which has been prepared by a capacity allocator providing allocated capacity.

Balance area schedule	A schedule that provides the planned schedule information for a balance

area.

## System Operator area schedule

A compilation of all external schedules concerning two System Operator areas or a connector concerning two System Operator of all balance responsible parties.

## Control block area schedule

A compilation of all the exchange programs of all control areas for one control block with all neighbouring control areas of a neighbouring control block.

## Coordination center area schedule

A compilation of the exchange programs of all exchange blocks divided into UCTE south and north.

## Intermediate confirmation report

An intermediate confirmation report that may be produced between final cutoffs.

## Final confirmation report

A final confirmation report that is produced after a final cutoff.

Finalised schedule	A compilation of a set of schedules that have been finalized after a given

cutoff.

Regulation data report	A compilation of the time series employed on a given day to ensure the

balance of the system.

## Aggregated energy data report

A compilation of the time series of all the meter readings or their equivalent for a given period.

Imbalance report	The report containing the complete situation of a given period for a party and

including the schedules, regulation data and actual or calculated readings.

Interconnection Capacity Document for cross-border capacity exchanges.

## Resource Provider Resource Schedule

A document providing the schedules for resource objects submitted by a resource provider.

## Acquiring System Operator Reserve Schedule

A document providing reserve purchases submitted by an Acquiring System Operator.

Anomaly Report	A document providing anomaly information for the receiving party to correct.

Acknowledgement Document A document providing acknowledgement information.

Confirmation report	A document providing confirmation information.

Capacity for Resale	A document providing information about capacity for resale.

## Approved Capacity Transfer

A document to approve a capacity transfer.

## Capacity transfer notification

A document notifying a capacity transfer.

## Transmission rights portfolio

A document providing the portfolio of the transmission capacity rights of a market participant.

Allocations	A document providing the capacity allocations for a border.

Bid document	A document providing bid information.

## Allocation result document

A document providing the allocation results of an auction.

Capacity document	A document providing capacity information.

Rights document	A document providing transmission capacity rights information.

## Generation availability schedule

This document contains information related to energy availability.

Cross border schedule	This document contains the cross border schedules for all the borders of a

given country where energy is exchanged.

Agreed capacity	The capacity agreed between parties.

Proposed capacity	The capacity proposed for agreement between parties.

System vertical load	The sum of all flows out of the transmission grid via directly connected

transformers and lines to distribution grids and end consumers as known by the System Operator.

Escalation document	A document which requesting the escalation of a situation.

## Trouble shooting document

A document providing trouble shooting information for the resolution of a problem.

Deactivation document	A document providing deactivation information.

Reserve tender document	The document that is used for the tendering for reserves within the ERRP

process.

## Reserve Allocation Result Document

The document used to provide the results of a Reserve auction.

SATCR activation	The document is used to provide the activation of reserves through the

SATCR process.

DATCR activation	The document is used to provide the activation of reserves through the

DATCR process.

Activation response	The document is used to provide a response to a request to activate

reserves.

Tender reduction	The document is used to provide information concerning the reduction of a

previously submitted tender.

MOL Document	The document is used to provide Merit Order List information.

Price Document	The document is used to provide market price information.

## Measurement Value Document

The document is used to provide measurement information from measurement devices.

SOAM Document	The document is used to provide system operator accounting data for

matching.

SOVA Document	The document is used to provide system operator validated accounting data.

CCVA Document	The document is used to provide coordination center validated accounting

data.

## Daily settlement document

The document is used to provide daily settlement information.

Weekly settlement	The document is used to provide weekly settlement information.

## document

Capacity Auction Specification Document

The document is used to provide capacity auction specification information.

## Market Coupling Results Document

The document is used to provide the results of a market coupling auction.

## Outage publication Document

The document is used to provide the outage information for publication.

## Forced generation outage Document

A document providing information on forced generation outages.

## Summarised Market Schedule

A compilation of all external schedules concerning two Market Balance Areas of all balance responsible parties.

## Compensation Program Schedule

A schedule that provides the schedule information for the compensation of unintended deviation.

## Load Frequency Control Program Schedule

A schedule that provides the schedule information for the Load Frequency Control Program of a Control Area or a Control Block.

## Timeframe Independent Schedule

A compilation of all external Timeframe Independent Schedules concerning two System Operators.

Information request	An information request being made concerning some specific information.

## status request for a position independently from a specific process

A status request concerning the position of an object independently of any ongoing processes.

## Estimated Net Transfer Capacity

The estimated net transfer capacity for a given border.

Compensation rights	The capacity rights that have been allocated as compensation.

Redispatch notice	A notice to confirm the actions agreed between System Operators to resolve a congestion situation through redispatch.

## Tender reduction response

A response to a tender reduction request that provides corrections to the initial document.

System total load	Total load', including losses without power used for energy storage, is equal

to generation deducted with exports, added with imports and deducted with power used for energy storage.

Final MOL	A document providing the information concerning the situation of the MOL at the end of an activation period.

## Resource Provider Schedule for production/consumption

A document providing the schedules for production/consumption for resource objects submitted by a resource provider.

## Installed generation per type

A document providing the installed generation per generation type.

Wind and solar forecast	A document providing the forecast of wind and solar generation.

Load forecast margin	A document providing the load forecast margin for a period.

Generation forecast	A document providing the generation forecast for a period.

## Reservoir filling information

A document providing information concerning the filling of reservoirs.

Actual generation	A document providing the actual generation for a period.

## Wind and solar generation

A document providing the generation of wind and solar energy for a period.

## Actual generation per type

A document providing the actual generation per generation type for a period.

Load unavailability	A document providing the unavailability of units providing load on the

network.

## Production unavailability

A document providing the unavailability of production units providing energy to the network.

## Transmission unavailability

A document providing the unavailability of transmission capacity on the network.

## Offshore grid infrastructure unavailability

A document providing the unavailability of an offshore grid infrastructure to the network.

## Generation unavailability

A document providing the unavailability of generation units providing energy to the network.

Contracted reserves	A document providing the reserves contracted for a period.

Accepted offers	A document providing the offers of reserves that have been accepted for a

period.

## Activated balancing quantities

A document providing the quantities of reserves that have been activated for balancing.

## Activated balancing prices

A document providing the prices of the reserves that have been activated for balancing.

Imbalance prices	A document providing the prices of reserves due to imbalance for a period.

Imbalance volume	A document providing the volume of the imbalance for a period.

Financial situation	A document providing the financial situation for reserves.

Cross border balancing	A document providing the cross border balancing requirements for a period.

## Contracted reserve prices

A document providing the price of reserves contracted for a period.

## Interconnection network expansion

A document providing information on the expansion of the interconnection network.

Counter trade notice	A document providing information on counter trades for a period.

Congestion costs	A document providing the cost of congestion for a period.

DC link capacity	A document providing the DC links for a period.

Non EU allocations	A document providing allocations made to non EU member states.

Configuration document	A document providing configuration information.

## Redispatch activation document

A document enabling the activation of a redispatch notice.

## Detailed activation history document

A document enabling a detailed history of activations.

## Aggregated activation history document

A document enabling an aggregated history of activations.

HVDC Link constraints	A document providing the information concerning the maximum and

minimum active power flow through each link can limited.

HVDC Configuration	A document providing the information concerning the power set point. The

mode in which the HVDC is managed.

HVDC Schedule	A document providing the information for operating DC links.

EIC code request	A document providing the information requesting a new EIC code.

EIC code information	A document providing EIC information in a central registry exchange or

information to an EIC participant.

EIC code publication	A document providing EIC publication information in a web site publication of

a limited set of information.

## Critical network element determination

A document providing all the elements necessary for the capacity coordinator to determine the transfer capacity and the associated critical network elements.

## Critical network element publication

A document providing all the elements necessary for the market information aggregator and TSOs to know the critical network elements which limit the transfer capacity.

Flow based domain	A document providing the capacity domain and its limits available for the

TSO.

## Flow based domain publication

A document providing the capacity domain and its limits available for the market.

## Flow based domain market impact publication

A document providing the capacity domain and its impacts on the market to be published.

## Anonymized flow based parameters publication

A document providing all the relevant flow based parameters in case of flow based capacity allocation.

## Critical network element market impact publication

A document providing the critical network elements and its impacts on the market to be published.

Weather document	An estimation or prediction in advance of the weather by analysis of

meteorological data and the results of what actually happened with the weather.

## Energy prognosis document

A document to provide the prognosis of energy production/load for a given period.

## Network constraint document

A document providing the network constraint situations used for the load flow studies. A network constraint situation includes contingencies, monitored elements and remedial actions.

## Aggregated netted external market schedule document

A document used to report aggregated netted external market schedules for a given border.

## Aggregated netted external TSO schedule document

A document used to report aggregated netted external TSO schedules for a given border.

## Reporting status market document

A document used to report the status of aggregated netted external market schedules, aggregated netted external TSO schedules and compensation program schedules on a given border.

## Reporting information market document

A document used to report the information concerning aggregated netted external schedules, aggregated netted external market schedules, aggregated netted external TSO schedules, compensation program schedules, netted area position schedules and netted area AC position schedules to an interested party.

## Status request for a reporting information market document

A document requesting the provision of a reporting information document.

Reserve need document	Used by a TSO to send its reserve needs.

## Generation and load shift keys document

A document providing the values of the generation and load shift keys to be used on network model.

Offers to be activated	A document containing the outcome of the process, with the list of offers that

are to be activated by the TSO concerned and the results for its reserve needs.

Clearing price	A document containing the outcome of the process, with the clearing prices

for a period.

Security analysis report A document providing a report on a performed security analysis.

## Aggregated netted external schedule document

A document used to report aggregated netted external schedules for a given border.

External TSO schedule	A document used to report external TSO schedules for a given border or

interconnector.

## Move of scheduled production

A document indication a move of scheduled production.

PS&LC results document	A document providing Pole Splitting & Loss Calculation results.

## Notification data market document

A document used to notify data to any information receiver.

## Additional Constraint document

A document describing additional constraints for a capacity calculation process.

## Operational state document

A document used for exchanging operational states for grid assets.

## Published offered capacity

A document providing the most recent values of offered capacity.

Market result document	Published prices and volumes

## Area Configuration document

A document containing the definition of areas.

## Area Composition document

A document containing the relations between areas, i.e which Metering Grid Areas a Bidding Zone composed of.

Connected Areas document A document containing which other areas an area is connected to i.e. which

Metering Grid Areas a Metering Grid Area is connected to.

Settlement document	A document providing settlement information.

## Imbalance prognosis document

A document to provide the prognosis of energy imbalances for a given period.

Complete set of bids	Submission of complete set of bids. If there are existing bids, they should be

replaced.

Merged MOL notice	A notice providing information on the MOL merging process.

K-factor document	A document providing K-factor values.

## Settlement coordination document

A document providing settlement information for coordination between different parties.

## Financial settlement document

A document providing financial settlement information.

## Bid availability document

A document providing the detailed reasons for changing the availability or volume of a bid.

## Resource capacity unit document

A document providing information about resource capacity units.

Other market information

A document providing other market information.

Permission document

A document that contains detailed information about permissions.

Requested capacity

A document providing requested capacity (e.g. ATC, CZCL) value.

Voltage control activation document

A document providing the number of generating units to activate for voltage control, by means of their operation in synchronous condenser mode.

Transmission inventory document

A document providing a transmission inventory.

Transmission inventory

A document providing statistics on the transmission grid.

The Time Series content provides aggregated information.

Summary type

The Time Series content provides detailed information.

Detail type

Indicates the classification mechanism used to group a set of objects together. The grouping may be of a detailed or a summary nature.

ESMPEnumerations

ClassificationTypeList

ProcessTypeList	ESMPEnumerations

Indicates the nature of process that the document addresses.

Day ahead	The information provided concerns a day ahead process.

Intra day incremental	The information provided concerns an intra day schedule.

Inter-area transit	The information provided concerns an inter area transit schedule. The rules

governing this process are market dependent

System operation closure The information provided concerns the closure of a given period of both scheduled and regulation information.

Metered data aggregation The information provided concerns the aggregation process of metered

information.

Imbalance settlement	The information provided concerns the imbalance settlement for a given

period for a balance responsible party or parties.

Capacity allocation	The information provided concerns the capacity allocation process.

Central reconciliation	The process carried out to finalise the imbalance settlement based on actual

metered values against provisional values from profiled metering points.

## Released capacity allocation

The process concerns the notification of capacity rights that are being released.

## Proposed capacity allocation

The process concerns the proposed capacity to be allocated for a given border.

## Agreed capacity allocation

The process concerns the capacity that has been agreed for allocation for a border.

Long term	The process concerns scheduling all schedules except daily and intraday contracts.

## Post scheduling adjustment

The process concerns the adjustments made to previous schedules after the period of execution.

Forecast	The data contained in the document are to be handled in short, medium, long term forecasting process.

Capacity determination	The process of determining the capacity for use.

Realised	The process for the treatment of realised data as opposed to forecast data.

Schedule day	The process concerns the day ahead, intraday and eventually ex-post

scheduling in a single document. The schedule will be transferred within the total position including historic information.

Intraday total	This process concerns an intraday schedule which contains the accumulated day ahead and intraday current position.

Intraday accumulated	This process concerns a single intraday schedule process where only

intraday evolutions occur through version changes.

SOMA process	System Operator meter alignment process.

SOVM process	System Operator validated measurement process.

RGCE accounting process	The document provides ENTSO-E Regional Group Continental Europe

accounting process information.

CCSR RGCE Settlement	The process concerns the control center settlement report for the whole of

the ENTSO-E Regional Group Continental Europe.

CBSR Settlement	The process concerns the control block settlement report.

CASR Settlement	The process concerns the control area settlement report.

Outage information	The process concerns TSO publication of outages on its power system.

Reserve resource process The process being described is for general reserve resources.

Primary reserve process	The process being described is for primary reserves.

## Secondary reserve process

The process being described is for secondary reserves.

Tertiary reserve process The process being described is for tertiary reserves.

Week ahead	The process being described is for the week ahead.

Month ahead	The process being described is for the month ahead.

Year ahead	The process being described is for the year ahead.

Contracted	The process being described is for contracted information.

Network information	The process being described is for network information. Creation	The process being described is for the creation of information. Modification	The process being described is for the modification of information. Deactivation process	The process being described is for deactivation of information.

Synchronisation process	The process being described is for the synchronisation of information.

Intraday process	The process being described is for intraday process.

Redispatch process	The process being described is for redispatch activation.

## Activation history process

The process being described is for the provision of an activation history.

## Flow based domain constraint day-ahead

The information provided concerns the flow-based process in day ahead.

## Flow based domain constraint intraday

The information provided concerns the flow-based process in intraday.

Two days ahead	Two days ahead.

Replacement reserve	A process being described is for replacement reserves (RR) to restore or

support the required level of frequency restoration reserves.

## Manual frequency restoration reserve

A process being described is for manual frequency restoration reserves (mFRR).

## Day-ahead capacity determination

The process run at the day-ahead timeframe to determine the capacity for use.

## Intraday capacity determination

The process run at the intraday timeframe to determine the capacity for use.

## Long term capacity determination

The process run at the long term timeframe to determine the capacity for use.

## Automatic frequency restoration reserve

A process being described is for automatic frequency restoration reserves (aFRR).

## Frequency containment reserve

A process being described is for frequency containment reserves (FCR).

## Common Grid Model (CGM) merging process

The process for merging individual grid models to form the common grid model.

## Coordinated security analysis

The process being described is used for coordinated security analysis.

Exchange of master data	A process for exchanging master data.

## Frequency restoration reserve

The process being described is for general frequency restoration reserve.

FSKAR settlement	The information provided concerns the Financial Settlement of K?f, ACE and

ramping period settlement for a given period.

mFRR capacity market	Processes related to the Reserve option market to assure that there are

enough available reserves for the manual FRR market.

Internal trade reporting The process related to internal trade reporting.

## Scheduled activation mFRR

mFRR being subject to scheduled activation.

Direct activation mFRR	mFRR being subject to direct activation.

Registration	A process related to the registration and management of object information.

Imbalance Netting	The process described is for imbalance netting.

## Criteria application for instantaneous frequency

The process describes criteria application for instantaneous frequency.

## Criteria application for frequency restoration

The process describes criteria application for frequency restoration.

Cost sharing	The process describes the cost sharing process for costly remedial actions.

Central selection aFRR	aFRR subject to central selection of bids for activation.

Local selection aFRR	aFRR subject to local selection of bids for activation.

## Common grid model alignment

The process for aligning bidding zone net positions.

## Pan-European verification function

The process for verifying bidding zone net positions.

## Outage planning coordination

The process being described is used for outage planning coordination.

Short term adequacy	The process being described is used for short term adequacy calculations.

## Coordinated capacity calculation

The process being described is used for coordinated capacity calculation.

Access to metered data	Access to meter readings of electricity consumption from or electricity feed

into the grid.

## Permission administration

Administration of permissions to access and/or use data.

# Compound types

MeasurementPointID_String

ESMPDataTypes

The coded identification of a domain covering a number of related objects, such as metering point, accounting point, etc.

In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for measurement point identification.

Datatypes

| codingScheme | 1..1 | CodingSchemeTypeList | DomainQualification. |
| --- | --- | --- | --- |
| value | 1..1 | String | Main Core value Space. |

Main Core value Space.

DateTime

1..1

value

ESMP_DateTime	ESMPDataTypes

In ESMP, the dateTime shall be expressed in UTC as YYYY-MM-DDThh:mm:ssZ.

Main Core value Space.

String

1..1

value

In the ESMP context, the code is defined either by:

- an emitting company that provides an agreed identification unique within a business context such as capacity auction identification, market agreement identification, etc.

- a party (originator of the exchange) that provides a unique identification in the framework of a business exchange such as document identification, time series identification, bid identification, ...

ESMPDataTypes

ID_String

A code to uniquely distinguish one occurrence of an entity from another.

Main Core value Space.

RoleTypeList

1..1

value

ESMPDataTypes

MarketRoleKind_String

The identification of the role played by a party.

PartyID_String

The identification of an actor in the energy market.

ESMPDataTypes

In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for market participant identification.

ResourceID_String

The identification of a resource object in the energy market.

ESMPDataTypes

In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for resources (generator, lines, substations, etc.) identification.

| codingScheme | 1..1 | CodingSchemeTypeList | DomainQualification. |
| --- | --- | --- | --- |
| value | 1..1 | String | Main Core value Space. |

Main Core value Space.

String

1..1

value

A code that distinguishes one evolution of an identified object from another. Information about a specific object may be sent several times, each transmission being identified by a different version number.

ESMPDataTypes

ESMPVersion_String

In ESMP, the coded value is restricted to digits.

Main Core value Space.

MessageTypeList

1..1

value

ESMPDataTypes

MessageKind_String

The coded type of a document.

| codingScheme | 1..1 | CodingSchemeTypeList | DomainQualification. |
| --- | --- | --- | --- |
| value | 1..1 | String | Main Core value Space. |

ClassificationKind_String

ESMPDataTypes

The coded identification of the classification mechanism used to group a set of objects together. The grouping may be of a detailed or a summary nature.

value

1..1

ClassificationTypeList

Main Core value Space.

Main Core value Space.

ProcessTypeList

1..1

value

ESMPDataTypes

ProcessKind_String

The coded identification of the nature of process.

ESMPDataTypes_ESMPCompound.png %d×%d pixels