List of stereotye to categorize subProfiles ShortCircuit Description Operation European Abstract Entsoe

ECnEvAEDssSe_tReferenceEnergyCurveMinMaxOperatingEnvelope assembly model

EnvAsset is an energy data space specific attribute referring to the Asset from which data is being requested

Concrete Classes

| Native Members |  |  |  |
| --- | --- | --- | --- |
| meterId | 0..1 | String | meterId is an energy data space |
|  |  |  | specific attribute that specifies the id |
|  |  |  | of the meter that measures the asset. |
| operatorId | 0..1 | String | operatorId is an energy data space |
|  |  |  | specific attribute that specifies the id |
|  |  |  | operator of the asset refered to. |
| type | 0..1 | String | type is an energy data space specific |
|  |  |  | attribute that specifies type of asset |
|  |  |  | refered to. |

[0..1] MetaInformation

MetaInformation

The date and time the MessageDocumenHeader was created.

0..1	ESMP_DateTime

creationDateTime

MCeEsEsaDgSe_DRoecfeurmenecnetEHneeardgeyrCurveMinMaxOperatingEnvelope assembly model

MessageDocumentHeader is an energy data space specific entity with the creationDateTime of this document.

Native Members

MCeEtaEIDnfSo_rRmeafteiorennceEnergyCurveMinMaxOperatingEnvelope assembly model

MetaInformation is an energy data space specific entity referring to the information concerning the technical and functional context of the MarketDocument.

Native Members

connectionId

0..1	String

connectionId is an energy data space specific attribute referring to the established connection used for the communication of this document.

| dataNeedId | 0..1 | String | dataNeedId is an energy data space specific attribute holding the type of data need requested, like ‘last 24 hours at 1 hour interval”. |
| --- | --- | --- | --- |
| dataSourceId | 0..1 | String | dataSourceId is an energy data space |
|  |  |  | specific attribute referring to the |
|  |  |  | datasource from which the requested |
|  |  |  | data is being queried. |
| defaultValues | 0..1 | String | defaultValues is a energy data space |
|  |  |  | specific attribute to hold relevant |
|  |  |  | default values. The format to use can |
|  |  |  | freely be chosen based on the |
|  |  |  | implementation. Most commonly you |
|  |  |  | would expect a semi-colon separated |
|  |  |  | name value pair, like: |
|  |  |  | "minPower=value; maxPower=value; |
|  |  |  | avgPower=value". |
| documentType | 0..1 | String | documentType is an energy data |
|  |  |  | space specific attribute referring to |
|  |  |  | the type of document, comparable to |
|  |  |  | the MessageTypeList in the CIM part |
|  |  |  | of the message. |
|  |  |  | Note: |
|  |  |  | This is applicable if you have the |
|  |  |  | need to know the content without |
|  |  |  | having to open the complete |
|  |  |  | document. DocumentType might |
|  |  |  | contain messages not (yet) part of the |
|  |  |  | MessageTypeList. |
| finalCustomerId | 0..1 | String | The finalCustomerId is an energy |
|  |  |  | data space specific attribute |
|  |  |  | identifying the actor doing the request |
|  |  |  | for data or data sharing. |
| regionConnector | 0..1 | String | regionConnector is an energy data |
|  |  |  | space specific attribute that specifies |
|  |  |  | the regional connector being used to |
|  |  |  | retrieve the data requested. |
| regionCountry | 0..1 | String | regionCountry is an energy data |
|  |  |  | space specific attribute that specifies |
|  |  |  | the country where the regional |
|  |  |  | connector is located. |
| requestPermissionId | 0..1 | String | The requestPermissionId is energy |
|  |  |  | data space specific identification |
|  |  |  | attribute holding an unique identifier |
|  |  |  | issued by the energy data space to |
|  |  |  | allow for tracing of the permission |
|  |  |  | through serveral message |

exchanges.

Note: the requestPermissionId is NOT the identifier issued by the Identity Service Provider as specified in the EU Implementing Act on access to Metering and Consumption Data.

EnvAsset

[0..1] EnvAsset

A sequential value representing the relative position within a given time interval.

1..1	Position_Integer

position

PCoEinEtDS_ReferenceEnergyCurveMinMaxOperatingEnvelope assembly model

The identification of the values being addressed within a specific interval of time.

Native Members

| QCuEaEnDtitSy_ReferenceEnergyCurveMinMaxOperatingEnvelope assembly model Description of quantities needed in the data exchange. The type of the quantity is described either by the role of the association or the quantityType attribute. The quality attribute provides the information about the quality of the quantity (measured, estimated, etc.). Native Members |  |  |
| --- | --- | --- |
| quality 0..1 | Quality_String | The description of the quality of the quantity. |
| quantity 1..1 | Decimal | The quantity |
|  |  | value. |
|  |  | The association |
|  |  | role provides the |
|  |  | information abou |
|  |  | what is |
|  |  | expressed. |
| quantityDefinition.commodityFlowOccurence 0..1 | CommodityDirectionKind | The direction of |
|  |  | flow, such as tha |
|  |  | of water or |
|  |  | electricity, for the |
|  |  | quantity. |
|  |  | Quantities can be |
|  |  | for |
|  |  | measurements, |
|  |  | or expected |
|  |  | future values, |
|  |  | e.g. forecast, |
|  |  | schedule, |

t

t

instruction.

quantityDefinition.measure_Unit.name	1..1	MeasurementUnitKind_String The identification

of the formal code for a measurement unit (UN/ECE Recommendation 20).

quantityDefinition.mRID	0..1	String	Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122,

for the mRID. The use of UUID is strongly recommended. For CIMXML

data files in RDF syntax conforming to IEC 61970-552,

the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements.

quantityDefinition.quantityQualifier	0..1	String	Role or characteristic of the quantity type. For example: average measured value, minimum measured value, maximum measured value, forecast expected value, forecast lower bound, forecast upper bound, lower operational limit, upper operational limit.

RCEECEMDMSO_REe_fMeraernkceetDEonecrugmyCenurtveMinMaxOperatingEnvelope assembly model

An electronic document containing the information necessary to satisfy the requirements of a given business process.

| quantityDefinition.quantityType | 0..1 | String | Describes an |
| --- | --- | --- | --- |
|  |  |  | observable |
|  |  |  | property that can |
|  |  |  | be measured and |
|  |  |  | quantified |
|  |  |  | numerically. |
|  |  |  | Electricity related |
|  |  |  | examples are |
|  |  |  | ActivePower, |
|  |  |  | ReactivePower, |
|  |  |  | ActiveEnergy, |
|  |  |  | ReactiveEnergy. |
| type | 0..1 | String | The description of the type of the quantity. |

[0..1] MessageDocumentHeader

MessageDocumentHeader

[0..1] RECMMOE_MarketDocument

RECMMOE_MarketDocument

RCEECEMDMSO_REe_fEenrevnecloepEenergyCurveMinMaxOperatingEnvelope assembly model

The envelope entity is a non-CIM object used in an energy data space environment to serve as a central top-level entity connecting the MarketDocumentHeader (energy data space specific context information block) with the regular CIM MarketDocument.

Native Members

| Native Members |  |  |  |
| --- | --- | --- | --- |
| createdDateTime | 0..1 | ESMP_DateTime | The date and time |
|  |  |  | of the creation of |
|  |  |  | the document. |
| description | 0..1 | String | The description is |
|  |  |  | a free human |
|  |  |  | readable text |
|  |  |  | describing or |
|  |  |  | naming the |
|  |  |  | object. It may be |
|  |  |  | non unique and |
|  |  |  | may not correlate |
|  |  |  | to a naming |
|  |  |  | hierarchy. |

| mRID | 1..1 | ID_String | The unique identification of the document being exchanged within a business process flow. |
| --- | --- | --- | --- |
|  |  |  | In the ESMP context, the "model authority" is defined as a party (originator of the exchange) that provides an identification in the context of a business exchange such as document identification, ... |
|  |  |  | Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |
| process.processType | 0..1 | ProcessKind_String | The identification of the nature of process that the document addresses. --- The Process associated with an electronic document header that is valid for the whole document. |
| receiver_MarketParticipant.marketRole.type | 1..1 | MarketRoleKind_String | The identification |

of the role played by a market player.

--- The MarketParticipant associated with an electronic document header.

--- The role associated with a MarketParticipant.

receiver_MarketParticipant.mRID	1..1	PartyID_String	The identification

of a party in the energy market.

In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification.

Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context.

Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this.

For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements.

--- The

|  |  |  | MarketParticipant associated with an electronic document header. |
| --- | --- | --- | --- |
| receiver_MarketParticipant.name | 0..1 | String | The name is any free human readable and possibly non unique text naming the object. --- The MarketParticipant associated with an electronic document header. |
| revisionNumber | 0..1 | ESMPVersion_String | The identification of the version that distinguishes one evolution of a document from another. |
| sender_MarketParticipant.marketRole.type | 1..1 | MarketRoleKind_String | The identification of the role played by a market player. |
|  |  |  | --- The MarketParticipant associated with an electronic document header. --- The role associated with a MarketParticipant. |
| sender_MarketParticipant.mRID | 1..1 | PartyID_String | The identification of a party in the energy market. |
|  |  |  | In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources |

|  |  |  | (generator, lines, substations, etc.) identification. |
| --- | --- | --- | --- |
|  |  |  | Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. --- The MarketParticipant associated with an electronic document header. |
| sender_MarketParticipant.name | 0..1 | String | The name is any free human readable and possibly non unique text naming the object. --- The MarketParticipant associated with an electronic document header. |
| time_Period.timeInterval | 0..1 | ESMP_DateTimeInterval | The start and end date and time for a given interval. |
|  |  |  | --- The time interval that is associated with an electronic document and which is valid for the whole document. |

SCeEriEesD_SP_eRrieofedrenceEnergyCurveMinMaxOperatingEnvelope assembly model

The identification of the period of time corresponding to a given time interval and resolution.

TCimEeESDeSr_ieRse_fSeererniecseEnergyCurveMinMaxOperatingEnvelope assembly model

A set of similar physical or conceptual objects defined for the same period or point of time.

SCeEriEesDS_ReferenceEnergyCurveMinMaxOperatingEnvelope assembly model

A set of similar physical or conceptual objects defined for the same period or point of time.

Native Members

type

0..1	MessageKind_String

The coded type of a document. The document type describes the principal characteristic of the document.

| Native Members |  |  |  |
| --- | --- | --- | --- |
| resolution | 1..1 | Duration | The definition of the number of units of time that compose an individual step within a period. |
| timeInterval | 1..1 | ESMP_DateTimeInterval | The start and end time of the period. |

| Native Members |  |  |  |
| --- | --- | --- | --- |
| businessType | 0..1 | BusinessKind_String | The identification of the nature of the time series. |
| curveType | 0..1 | CurveType_String | The identification of the coded representation of the type of curve being described. |
| flowDirection.direction | 1..1 | DirectionKind_String | The coded identification of the direction of energy flow. |
|  |  |  | --- The flow direction |

|  |  |  | associated with a TimeSeries. |
| --- | --- | --- | --- |
| mRID | 0..1 | ID_String | A unique identification of the time series. |
|  |  |  | In the ESMP context, the "model authority" is defined as a party (originator of the exchange) that provides a unique identification in the context of a business exchange such as time series identification, bid identification, ... |
|  |  |  | Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |
| registeredResource.description | 0..1 | String | The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. --- The identification of a resource associated with a TimeSeries. |
| registeredResource.mRID | 1..1 | ResourceID_String | The unique identification of a resource. |
|  |  |  | In the ESMP context, the "model authority" is defined as an |

|  |  |  | authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification. |
| --- | --- | --- | --- |
|  |  |  | Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. --- The identification of a resource associated with a TimeSeries. |
| registeredResource.name | 0..1 | String | The name is any free human readable and possibly non unique text naming the object. --- The identification of a resource associated with a TimeSeries. |
| resourceTimeSeries.value1ScheduleType | 0..1 | CommodityDirectionKind | Value1ScheduleType documents two key features for a specific resource operating mode: (a) the energy flow convention for a positive number as either production or consumption of service and (b) the sense of how the resource is planned to be used or was used as either a target or setpoint or as a relative change |

to a defined level.

# Enumerations

The content of the object is temporary. The code is typically used when there is communication trouble to a meter and a more correct value is expected later.

Temporary

The contents of the object are calculated. The code is typically used when a value is calculated based on several other known values.

Calculated

The contents of the object are calculated based on incomplete data.

Incomplete

The contents of the object are as provided.

As provided

The contents of the object are estimated. The code is typically used when measured values are missing and an estimate is made based on historical data.

Estimated

The contents of the object are not available.

Not available

The contents of the object have been adjusted.

Adjusted

ESMPEnumerations

QualityTypeList

The quality of an object.

CommodityDirectionKind	CEEDS_ESMP_DataTypes

CommodityDirectionKind is an enumeration that documents two key features for a specific DER operating mode: (a) the energy flow convention for a positive number as either production or consumption of service and (b) the sense of how the bid/offer amount is used as either a target or setpoint or as a relative change to a defined level.

production	The resource operates as a generation resource, where a positive amount

corresponds to an injection of energy and/or grid support services to the grid. The amount of energy is an absolute level understood to be a production setpoint.

Negative values are acceptable and represent an energy withdrawal setpoint. For energy storage resources, a positive amount represents a setpoint in the discharging state.

consumption	The resource operates as a load resource, where a positive amount corresponds to a withdrawal of energy and/or support services from the grid. The amount of energy is an absolute level understood to be a consumption setpoint. Negative values are acceptable and represent an energy injection setpoint. For energy storage resources, a positive amount represents a setpoint in the charging state.

consumptionIncrease	The resource operates as a variable load, where a positive amount corresponds

to a relative load increase. The level is relative, generally as an upward offset to a reference level such as the current consumption or a calculated baseline consumption. Negative values are acceptable and represent a load reduction offset. For energy storage resources, a positive amount represents a faster rate of charge, a state change from discharging (or offline) to charge, or a slower rate of discharge.

consumptionDecrease	The resource operates as a curtailable load, where a positive amount corresponds to a relative load reduction. The level is relative, generally as an downward offset to a reference level such as the current consumption or a calculated baseline consumption. Negative values are acceptable and represent a load increase offset. For energy storage resources, a positive amount represents a slower rate of charge, a state change from charging (or offline) to discharging, or a faster rate of discharge.

productionIncrease	The resource operates primarily as a variable generation resource, where a

positive amount corresponds to a relative generation increase.

productionDecrease	The resource operates primarily as a variable generation resource, where a

positive amount corresponds to a relative generation reduction.

UnitOfMeasureTypeList	ESMPEnumerations

(synonym MeasurementUnit) The unit of measure that is applied to a quantity. The measurement units shall be in compliance with UN/ECE Recommendation 20.

ampere	The unit of electrical current in the International system of Units (SI) equivalent to one Coulomb per second.

megavolt-ampere	MVA unit as per UN/CEFACT recommendation 20.

One	A unit for dimensionless quantities, also called quantities of dimension one.

cubic hectometres	A unit of volume equal to one million cubic metres.

kilovolt	kV unit as per UN/CEFACT recommendation 20.

kilometre	km unit as per UN/CEFACT recommendation 20.

kilovolt ampere reactive A unit of electrical reactive power represented by a current of one thousand

amperes flowing due to a potential difference of one thousand volts where the sine of the phase angle between them is 1. The unity power factor is expressed in thousands of a volt ampere reactive.

gigawatt hour	GWh unit as per UN/CEFACT recommendation 20.

kilowatt hour	A total amount of electrical energy transferred or consumed in one hour.

kilowatt	A unit of bulk power flow, which can be defined as the rate of energy transfer

/consumption when a current of 1000 amperes flows due to a potential of 1000 volts at unity power factor expressed in thousands of a watt.

## megavolt ampere reactive hours

Total amount of reactive power across a power system.

megavolt ampere reactive A unit of electrical reactive power represented by a current of one thousand

amperes flowing due to a potential difference of one thousand volts where the sine of the phase angle between them is 1.

megawatt	A unit of bulk power flow, which can be defined as the rate of energy transfer

/consumption when a current of 1000 amperes flows due to a potential of 1000 volts at unity power factor expressed in millions of a watt.

cubic metres per second	The volume flow rate of cubic metre per second.

cubic metre	A Cubic metre.

metre	The length of a metre.

megawatt hours	The total amount of bulk energy transferred or consumed.

percent	A unit of proportion equal to 0.01.

gigawatt	GW unit as per UN/CEFACT recommendation 20.

watt	The watt is the International System of Units (SI) standard unit of power (energy per unit time), the equivalent of one joule per second.

meter per second	A unit of measurement of the speed expressed in m/s.

degree (unit of angle)	A unit of measurement of angles expressed in a 0 to 360 degree gradient.

watt per square meter	A unit of measurement of the density of heat flow rate expressed in watt per

square meter.

Celsius	A unit of measurement of temperature expressed in degree Celsius.

OKTA unit	A unit of measurement of the cloudiness expressed in OKTA or OCTA, i.e. A unit of count defining the number of eighth-parts as a measure of the celestial dome cloud coverage.

hectopascal	A unit of measurement of the pressure expressed in hectopascal.

## millimeter

A unit of measurement of length expressed in millimeter.

K (Kelvin)

Temperature unit refer ISO 80000-5 (Quantities and units, Part 5: Thermodynamics).

Hertz

HTZ unit as per UN/CEFACT recommendation 20.

Megawatt per Hertz

A unit of energy expressed as the load change in million watts that will cause a frequency shift of one hertz.

millihertz

A unit of frequency equal to 0.001 cycle per second.

minute

A period of time equal to sixty seconds.

second

A period of time equal to one second.

ProcessTypeList	ESMPEnumerations

Indicates the nature of process that the document addresses.

Day ahead	The information provided concerns a day ahead process.

Intra day incremental	The information provided concerns an intra day schedule.

Inter-area transit	The information provided concerns an inter area transit schedule. The rules

governing this process are market dependent

System operation closure The information provided concerns the closure of a given period of both scheduled and regulation information.

Metered data aggregation The information provided concerns the aggregation process of metered

information.

Imbalance settlement	The information provided concerns the imbalance settlement for a given period for a balance responsible party or parties.

Capacity allocation	The information provided concerns the capacity allocation process.

Central reconciliation	The process carried out to finalise the imbalance settlement based on actual

metered values against provisional values from profiled metering points.

## Released capacity allocation

The process concerns the notification of capacity rights that are being released.

## Proposed capacity

allocation

The process concerns the proposed capacity to be allocated for a given border.

## Agreed capacity allocation

The process concerns the capacity that has been agreed for allocation for a border.

Long term	The process concerns scheduling all schedules except daily and intraday contracts.

## Post scheduling adjustment

The process concerns the adjustments made to previous schedules after the period of execution.

Forecast	The data contained in the document are to be handled in short, medium, long term forecasting process.

Capacity determination	The process of determining the capacity for use.

Realised	The process for the treatment of realised data as opposed to forecast data.

Schedule day	The process concerns the day ahead, intraday and eventually ex-post scheduling in a single document. The schedule will be transferred within the total position including historic information.

Intraday total	This process concerns an intraday schedule which contains the accumulated day ahead and intraday current position.

Intraday accumulated	This process concerns a single intraday schedule process where only intraday evolutions occur through version changes.

SOMA process	System Operator meter alignment process.

SOVM process	System Operator validated measurement process.

RGCE accounting process	The document provides ENTSO-E Regional Group Continental Europe accounting process information.

CCSR RGCE Settlement	The process concerns the control center settlement report for the whole of the

ENTSO-E Regional Group Continental Europe.

CBSR Settlement	The process concerns the control block settlement report.

CASR Settlement	The process concerns the control area settlement report.

Outage information	The process concerns TSO publication of outages on its power system.

Reserve resource process The process being described is for general reserve resources.

Primary reserve process	The process being described is for primary reserves.

## Secondary reserve process

The process being described is for secondary reserves.

Tertiary reserve process The process being described is for tertiary reserves.

Week ahead	The process being described is for the week ahead.

Month ahead	The process being described is for the month ahead.

Year ahead	The process being described is for the year ahead.

Contracted	The process being described is for contracted information.

Network information	The process being described is for network information. Creation	The process being described is for the creation of information. Modification	The process being described is for the modification of information. Deactivation process	The process being described is for deactivation of information.

Synchronisation process	The process being described is for the synchronisation of information.

Intraday process	The process being described is for intraday process.

Redispatch process	The process being described is for redispatch activation.

## Activation history process

The process being described is for the provision of an activation history.

## Flow based domain constraint day-ahead

The information provided concerns the flow-based process in day ahead.

## Flow based domain constraint intraday

The information provided concerns the flow-based process in intraday.

Two days ahead	Two days ahead.

Replacement reserve	A process being described is for replacement reserves (RR) to restore or support the required level of frequency restoration reserves.

Manual frequency	A process being described is for manual frequency restoration reserves (mFRR).

## restoration reserve

Day-ahead capacity determination

The process run at the day-ahead timeframe to determine the capacity for use.

## Intraday capacity determination

The process run at the intraday timeframe to determine the capacity for use.

## Long term capacity determination

The process run at the long term timeframe to determine the capacity for use.

## Automatic frequency restoration reserve

A process being described is for automatic frequency restoration reserves (aFRR).

## Frequency containment reserve

A process being described is for frequency containment reserves (FCR).

## Common Grid Model (CGM) merging process

The process for merging individual grid models to form the common grid model.

## Coordinated security analysis

The process being described is used for coordinated security analysis.

Exchange of master data	A process for exchanging master data.

## Frequency restoration reserve

The process being described is for general frequency restoration reserve.

FSKAR settlement	The information provided concerns the Financial Settlement of K?f, ACE and

ramping period settlement for a given period.

mFRR capacity market	Processes related to the Reserve option market to assure that there are enough available reserves for the manual FRR market.

Internal trade reporting The process related to internal trade reporting.

## Scheduled activation mFRR

mFRR being subject to scheduled activation.

Direct activation mFRR	mFRR being subject to direct activation.

Registration	A process related to the registration and management of object information.

Imbalance Netting	The process described is for imbalance netting.

## Criteria application for instantaneous frequency

The process describes criteria application for instantaneous frequency.

Criteria application for The process describes criteria application for frequency restoration.

## frequency restoration

Cost sharing	The process describes the cost sharing process for costly remedial actions.

Central selection aFRR	aFRR subject to central selection of bids for activation.

Local selection aFRR	aFRR subject to local selection of bids for activation.

## Common grid model alignment

The process for aligning bidding zone net positions.

## Pan-European verification function

The process for verifying bidding zone net positions.

## Outage planning coordination

The process being described is used for outage planning coordination.

Short term adequacy	The process being described is used for short term adequacy calculations.

## Coordinated capacity calculation

The process being described is used for coordinated capacity calculation.

Access to metered data	Access to meter readings of electricity consumption from or electricity feed into

the grid.

## Permission administration

Administration of permissions to access and/or use data.

RoleTypeList

Identification of the role played by a party.

ESMPEnumerations

Trade responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Consumption responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Combined power exchange (not to be used)

This role is no longer in the ENTSO-E Harmonised Role Model Document.

System operator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Imbalance settlement responsible

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Production responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Transmission capacity allocator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

## Balance responsible party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Metered data aggregator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Billing agent	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Market operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Energy supplier	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Consumer	Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Control area operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Control block operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Coordination centre operator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Grid access provider	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Grid operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Meter administrator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Party connected to grid	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Producer	Refer to role model definitions in the ENTSO-E Harmonised Role Model

## Profile maintenance party

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Meter operator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Metered data collector	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Metered data responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Metering point administrator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Resource Provider	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Scheduling coordinator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

Capacity Trader	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Interconnection Trade Responsible

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

Nomination Validator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

## Market information aggregator

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

A party that collects information from different sources and assembles it to provide a summary of the market.

Information receiver	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

A party, not necessarily a market participant, which receives information about the market.

Reserve Allocator	Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document.

A party that informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders.

MOL Responsible	Refer to role model definitions in the ENTSO-E Harmonised Role Model

A party that Informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders.

Capacity Coordinator	A party, acting on behalf of the SOs involved, responsible for establishing a

coordinated Offered Capacity and/or NTC and/or ATC between several Scheduling Areas.

## Reconciliation Accountable

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document.

A party that is financially accountable for the reconciled volume of energy products for a profiled local metering point.

## Reconciliation Responsible

A party that is responsible for reconciling, within a metering grid area, the volumes used in the imbalance settlement process for profiled metering points and the actual metered quantities.

Data provider	A party that is responsible for providing information to a central authority.

## Local Issuing Office (LIO)

A party that is responsible for operating a Local Issuing Office (LIO).

## Central Issuing Office (CIO)

A party that is responsible for operating a Central Issuing Office (CIO).

EIC Participant	A party that participates in the EIC environment.

Weather analyser	A party that analyses the current and forecast weather situation and establishes a prognosis of its impact on the renewable energy environment as well as the overall load.

## Regional Security Coordinator (RSC)

The RSC as defined in the System Operation guideline.

## Energy Service Company (ESCO)

A party offering energy-related services to the Party Connected to Grid, but not directly active in the energy value chain or the physical infrastructure itself. The ESCO may provide insight services as well as energy management services.

## Balancing Service Provider

A party with reserve-providing units or reserve-providing groups able to provide balancing services to LFC Operators.

Energy trader	A party that is selling or buying energy.

LFC Operator	A party that is responsible for the Load Frequency Control of its LFC Area or LFC Block.

## Transmission System Operator (TSO)

The Transmission System Operator (TSO) is responsible for the transport of electricity on the extra high-voltage and high-voltage interconnected system. This is a market participant and not a role in Harmonised Role Model.

## Distribution System Operator (DSO)

Distribution System Operator (DSO) is responsible for transport of electricity on high-voltage (optionally), medium-voltage and low-voltage distribution systems. This is a market participant and not a role in Harmonised Role Model.

## Resource Capacity Mechanism Operator

Resource capacity mechanism operator is the party responsible to operate the resource capacity mechanism in a member state. It can either be the TSO or an independent party.

Resource aggregator	A party that aggregates resources for usage by a service provider for energy

market services.

Cost sharing calculator	A party responsible for the mapping of the costs between areas and parties.

Settlement responsible	A party responsible for settling the costs.

## Coordinated Capacity Calculator

Coordinated Capacity Calculator is the entity or entities with the task of calculating transmission capacity, at regional level or above.

## Flexibility service provider

A party that offers flexibility services based on acquired (aggregated) resources.

Data access provider	A party responsible for facilitating access to data.

## Metered data administrator

A party responsible for storing and distributing validated measured data.

Permission administrator A party responsible for administrating a register of data access permissions for a

set of metering points, making this information available to final customers and eligible parties in the sector, on request.

CodingSchemeTypeList	ESMPEnumerations

Codification scheme used to identify the coding scheme used for the set of coded values to identify specific objects.

EIC	The coding scheme is the Energy Identification Coding Scheme (EIC), maintained by ENTSO-E.

## Coding scheme which are part of GS1 system

The coding schemes maintained by GS1.

## Andorra National coding scheme

The National coding scheme of the country in question.

## Albania National coding scheme

The National coding scheme of the country in question.

## Armenia National coding scheme

The National coding scheme of the country in question.

## Austria National coding scheme

The National coding scheme of the country in question.

## Azerbaijan National coding scheme

The National coding scheme of the country in question.

## Bosnia and Herzegovina National coding scheme

The National coding scheme of the country in question.

## Belgium National coding scheme

The National coding scheme of the country in question.

## Bulgaria National coding scheme

The National coding scheme of the country in question.

## Switzerland National coding scheme

The National coding scheme of the country in question.

## Serbia and Montenegro National coding scheme

The National coding scheme of the country in question.

## Czech Republic National coding scheme

The National coding scheme of the country in question.

## Germany National coding scheme

The National coding scheme of the country in question.

## Denmark National coding scheme

The National coding scheme of the country in question.

## Estonia National coding scheme

The National coding scheme of the country in question.

## Spain National coding scheme

The National coding scheme of the country in question.

## Finland National coding scheme

The National coding scheme of the country in question.

## France National coding scheme

The National coding scheme of the country in question.

## United Kingdom National coding scheme

The National coding scheme of the country in question.

## Georgia National coding scheme

The National coding scheme of the country in question.

## Gibraltar National coding scheme

The National coding scheme of the country in question.

## Greece National coding scheme

The National coding scheme of the country in question.

## Croatia National coding scheme

The National coding scheme of the country in question.

## Hungary National coding scheme

The National coding scheme of the country in question.

## Ireland National coding scheme

The National coding scheme of the country in question.

## Italy National coding scheme

The National coding scheme of the country in question.

## Kyrgyzstan National coding scheme

The National coding scheme of the country in question.

## Kazakhstan National coding scheme

The National coding scheme of the country in question.

## Liechtenstein National coding scheme

The National coding scheme of the country in question.

## Lithuania National coding scheme

The National coding scheme of the country in question.

## Luxembourg National coding scheme

The National coding scheme of the country in question.

## Latvia National coding scheme

The National coding scheme of the country in question.

## Morocco National coding scheme

The National coding scheme of the country in question.

## Moldavia National coding scheme

The National coding scheme of the country in question.

## Macedonia National coding scheme

The National coding scheme of the country in question.

## Netherlands National coding scheme

The National coding scheme of the country in question.

## Nordic Regional coding scheme

The coding scheme of the Nordic region which covers Denmark, Finland, Norway and Sweden.

## Norway National coding scheme

The National coding scheme of the country in question.

## Poland National coding scheme

The National coding scheme of the country in question.

## Portugal National coding scheme

The National coding scheme of the country in question.

## Romania National coding scheme

The National coding scheme of the country in question.

## Russian Federation National coding scheme

The National coding scheme of the country in question.

## Sweden National coding scheme

The National coding scheme of the country in question.

## Slovenia National coding scheme

The National coding scheme of the country in question.

## Slovakia National coding scheme

The National coding scheme of the country in question.

## Turkey National coding scheme

The National coding scheme of the country in question.

## Ukraine National coding scheme

The National coding scheme of the country in question.

CGM	The coding scheme used for Common Grid Model Exchange Standard (CGMES).

## Cyprus National coding scheme

The National coding scheme of the country in question.

ISO	The coding scheme for the preceding attribute is a code maintained by International Organization for Standardization (ISO), such as “ISO 3166 English country names and code elements”.

MessageTypeList

ESMPEnumerations

The coded type of a document. The message type describes the principal characteristic of a document. This enumeration is used in the XML instances based on IEC 62325.

Balance responsible schedule

A schedule which has been prepared by a balance responsible party providing planned schedule information.

Allocated capacity schedule

A schedule which has been prepared by a capacity allocator providing allocated capacity.

Balance area schedule

A schedule that provides the planned schedule information for a balance area.

System Operator area schedule

A compilation of all external schedules concerning two System Operator areas or a connector concerning two System Operator of all balance responsible parties.

## Control block area schedule

A compilation of all the exchange programs of all control areas for one control block with all neighbouring control areas of a neighbouring control block.

## Coordination center area schedule

A compilation of the exchange programs of all exchange blocks divided into UCTE south and north.

## Intermediate confirmation report

An intermediate confirmation report that may be produced between final cutoffs.

## Final confirmation report

A final confirmation report that is produced after a final cutoff.

Finalised schedule	A compilation of a set of schedules that have been finalized after a given cutoff.

Regulation data report	A compilation of the time series employed on a given day to ensure the balance of the system.

## Aggregated energy data report

A compilation of the time series of all the meter readings or their equivalent for a given period.

Imbalance report	The report containing the complete situation of a given period for a party and

including the schedules, regulation data and actual or calculated readings.

Interconnection Capacity Document for cross-border capacity exchanges.

## Resource Provider Resource Schedule

A document providing the schedules for resource objects submitted by a resource provider.

## Acquiring System Operator Reserve Schedule

A document providing reserve purchases submitted by an Acquiring System Operator.

Anomaly Report	A document providing anomaly information for the receiving party to correct.

Acknowledgement Document A document providing acknowledgement information.

Confirmation report	A document providing confirmation information.

Capacity for Resale	A document providing information about capacity for resale.

## Approved Capacity Transfer

A document to approve a capacity transfer.

## Capacity transfer notification

A document notifying a capacity transfer.

## Transmission rights

portfolio

A document providing the portfolio of the transmission capacity rights of a market participant.

Allocations	A document providing the capacity allocations for a border.

Bid document	A document providing bid information.

## Allocation result document

A document providing the allocation results of an auction.

Capacity document	A document providing capacity information.

Rights document	A document providing transmission capacity rights information.

## Generation availability schedule

This document contains information related to energy availability.

Cross border schedule	This document contains the cross border schedules for all the borders of a given

country where energy is exchanged.

Agreed capacity	The capacity agreed between parties.

Proposed capacity	The capacity proposed for agreement between parties.

System vertical load	The sum of all flows out of the transmission grid via directly connected

transformers and lines to distribution grids and end consumers as known by the System Operator.

Escalation document	A document which requesting the escalation of a situation.

## Trouble shooting document

A document providing trouble shooting information for the resolution of a problem.

Deactivation document	A document providing deactivation information.

Reserve tender document	The document that is used for the tendering for reserves within the ERRP

process.

## Reserve Allocation Result Document

The document used to provide the results of a Reserve auction.

SATCR activation	The document is used to provide the activation of reserves through the SATCR

process.

DATCR activation	The document is used to provide the activation of reserves through the DATCR

process.

Activation response	The document is used to provide a response to a request to activate reserves.

Tender reduction	The document is used to provide information concerning the reduction of a

previously submitted tender.

MOL Document	The document is used to provide Merit Order List information.

Price Document	The document is used to provide market price information.

## Measurement Value Document

The document is used to provide measurement information from measurement devices.

SOAM Document	The document is used to provide system operator accounting data for matching.

SOVA Document	The document is used to provide system operator validated accounting data.

CCVA Document	The document is used to provide coordination center validated accounting data.

## Daily settlement document

The document is used to provide daily settlement information.

## Weekly settlement document

The document is used to provide weekly settlement information.

## Capacity Auction Specification Document

The document is used to provide capacity auction specification information.

## Market Coupling Results Document

The document is used to provide the results of a market coupling auction.

## Outage publication Document

The document is used to provide the outage information for publication.

## Forced generation outage Document

A document providing information on forced generation outages.

## Summarised Market Schedule

A compilation of all external schedules concerning two Market Balance Areas of all balance responsible parties.

## Compensation Program Schedule

A schedule that provides the schedule information for the compensation of unintended deviation.

## Load Frequency Control Program Schedule

A schedule that provides the schedule information for the Load Frequency Control Program of a Control Area or a Control Block.

## Timeframe Independent Schedule

A compilation of all external Timeframe Independent Schedules concerning two System Operators.

Information request	An information request being made concerning some specific information.

## status request for a position independently from a specific process

A status request concerning the position of an object independently of any ongoing processes.

## Estimated Net Transfer Capacity

The estimated net transfer capacity for a given border.

Compensation rights	The capacity rights that have been allocated as compensation.

Redispatch notice	A notice to confirm the actions agreed between System Operators to resolve a

congestion situation through redispatch.

## Tender reduction response

A response to a tender reduction request that provides corrections to the initial document.

System total load	Total load', including losses without power used for energy storage, is equal to

generation deducted with exports, added with imports and deducted with power used for energy storage.

Final MOL	A document providing the information concerning the situation of the MOL at the

end of an activation period.

## Resource Provider Schedule for production/consumption

A document providing the schedules for production/consumption for resource objects submitted by a resource provider.

## Installed generation per type

A document providing the installed generation per generation type.

Wind and solar forecast	A document providing the forecast of wind and solar generation.

Load forecast margin	A document providing the load forecast margin for a period.

Generation forecast	A document providing the generation forecast for a period.

## Reservoir filling information

A document providing information concerning the filling of reservoirs.

Actual generation	A document providing the actual generation for a period.

## Wind and solar generation

A document providing the generation of wind and solar energy for a period.

## Actual generation per type

A document providing the actual generation per generation type for a period.

Load unavailability	A document providing the unavailability of units providing load on the network.

## Production unavailability

A document providing the unavailability of production units providing energy to the network.

## Transmission unavailability

A document providing the unavailability of transmission capacity on the network.

## Offshore grid infrastructure unavailability

A document providing the unavailability of an offshore grid infrastructure to the network.

## Generation unavailability

A document providing the unavailability of generation units providing energy to the network.

Contracted reserves	A document providing the reserves contracted for a period.

Accepted offers	A document providing the offers of reserves that have been accepted for a period.

## Activated balancing quantities

A document providing the quantities of reserves that have been activated for balancing.

## Activated balancing prices

A document providing the prices of the reserves that have been activated for balancing.

Imbalance prices	A document providing the prices of reserves due to imbalance for a period.

Imbalance volume	A document providing the volume of the imbalance for a period.

Financial situation	A document providing the financial situation for reserves.

Cross border balancing	A document providing the cross border balancing requirements for a period.

## Contracted reserve prices

A document providing the price of reserves contracted for a period.

## Interconnection network expansion

A document providing information on the expansion of the interconnection network.

Counter trade notice	A document providing information on counter trades for a period.

Congestion costs	A document providing the cost of congestion for a period.

DC link capacity	A document providing the DC links for a period.

Non EU allocations	A document providing allocations made to non EU member states.

Configuration document	A document providing configuration information.

## Redispatch activation document

A document enabling the activation of a redispatch notice.

## Detailed activation history document

A document enabling a detailed history of activations.

## Aggregated activation history document

A document enabling an aggregated history of activations.

HVDC Link constraints	A document providing the information concerning the maximum and minimum

active power flow through each link can limited.

HVDC Configuration	A document providing the information concerning the power set point. The mode

in which the HVDC is managed.

HVDC Schedule	A document providing the information for operating DC links.

EIC code request	A document providing the information requesting a new EIC code.

EIC code information	A document providing EIC information in a central registry exchange or

information to an EIC participant.

EIC code publication	A document providing EIC publication information in a web site publication of a

limited set of information.

## Critical network element determination

A document providing all the elements necessary for the capacity coordinator to determine the transfer capacity and the associated critical network elements.

## Critical network element publication

A document providing all the elements necessary for the market information aggregator and TSOs to know the critical network elements which limit the transfer capacity.

Flow based domain	A document providing the capacity domain and its limits available for the TSO.

## Flow based domain publication

A document providing the capacity domain and its limits available for the market.

## Flow based domain market impact publication

A document providing the capacity domain and its impacts on the market to be published.

## Anonymized flow based parameters publication

A document providing all the relevant flow based parameters in case of flow based capacity allocation.

## Critical network element market impact publication

A document providing the critical network elements and its impacts on the market to be published.

Weather document	An estimation or prediction in advance of the weather by analysis of

meteorological data and the results of what actually happened with the weather.

## Energy prognosis document

A document to provide the prognosis of energy production/load for a given period.

## Network constraint document

A document providing the network constraint situations used for the load flow studies. A network constraint situation includes contingencies, monitored elements and remedial actions.

## Aggregated netted external market schedule document

A document used to report aggregated netted external market schedules for a given border.

## Aggregated netted external TSO schedule document

A document used to report aggregated netted external TSO schedules for a given border.

## Reporting status market document

A document used to report the status of aggregated netted external market schedules, aggregated netted external TSO schedules and compensation program schedules on a given border.

## Reporting information market document

A document used to report the information concerning aggregated netted external schedules, aggregated netted external market schedules, aggregated netted external TSO schedules, compensation program schedules, netted area position schedules and netted area AC position schedules to an interested party.

## Status request for a reporting information market document

A document requesting the provision of a reporting information document.

Reserve need document	Used by a TSO to send its reserve needs.

## Generation and load shift keys document

A document providing the values of the generation and load shift keys to be used on network model.

Offers to be activated	A document containing the outcome of the process, with the list of offers that are

to be activated by the TSO concerned and the results for its reserve needs.

Clearing price	A document containing the outcome of the process, with the clearing prices for a

period.

Security analysis report A document providing a report on a performed security analysis.

## Aggregated netted external schedule document

A document used to report aggregated netted external schedules for a given border.

External TSO schedule	A document used to report external TSO schedules for a given border or

interconnector.

## Move of scheduled production

A document indication a move of scheduled production.

PS&LC results document	A document providing Pole Splitting & Loss Calculation results.

## Notification data market document

A document used to notify data to any information receiver.

## Additional Constraint document

A document describing additional constraints for a capacity calculation process.

## Operational state document

A document used for exchanging operational states for grid assets.

## Published offered capacity

A document providing the most recent values of offered capacity.

Market result document	Published prices and volumes

## Area Configuration document

A document containing the definition of areas.

## Area Composition document

A document containing the relations between areas, i.e which Metering Grid Areas a Bidding Zone composed of.

Connected Areas document A document containing which other areas an area is connected to i.e. which

Metering Grid Areas a Metering Grid Area is connected to.

Settlement document	A document providing settlement information.

## Imbalance prognosis document

A document to provide the prognosis of energy imbalances for a given period.

Complete set of bids	Submission of complete set of bids. If there are existing bids, they should be

replaced.

Merged MOL notice	A notice providing information on the MOL merging process.

K-factor document	A document providing K-factor values.

## Settlement coordination document

A document providing settlement information for coordination between different parties.

## Financial settlement document

A document providing financial settlement information.

## Bid availability document

A document providing the detailed reasons for changing the availability or volume of a bid.

## Resource capacity unit document

A document providing information about resource capacity units.

Other market information A document providing other market information.

Permission document	A document that contains detailed information about permissions.

Requested capacity	A document providing requested capacity (e.g. ATC, CZCL) value.

## Voltage control activation document

A document providing the number of generating units to activate for voltage control, by means of their operation in synchronous condenser mode.

## Transmission inventory document

A document providing a transmission inventory.

Transmission inventory	A document providing statistics on the transmission grid.

BusinessTypeList	ESMPEnumerations

The exact business nature identifying the principal characteristic of a time series.

Production	The nature of the business being described is production details.

Internal trade	The nature of the business being described is internal trade details.

## External trade explicit capacity

The nature of the business being described is external trade details between two areas with limited capacity requiring a capacity agreement identification.

Consumption	The nature of the business being described is consumption details.

External trade total	The nature of the business being described is external trade total.

## External trade without explicit capacity

The nature of the business being described is external trade details between two areas without requiring capacity allocation information.

## Net Production / Consumption

Net production/consumption - where signed values will be used.

With the following rules: In area=Out area, In party=Out party, + means production and - means consumption.

Net internal trade	Net internal trade - where the direction from out party (seller) to in party (buyer) is positive and the opposite direction is negative (with minus signs).

## IPP (Independent Power Producer)

A time series concerning the production schedule from an IPP.

Tertiary control	A time series concerning tertiary reserve. Primary control	A time series concerning primary reserve. Secondary control	A time series concerning secondary reserve.

Load profile	A time series concerning a load profile as calculated by a metered data

aggregator.

Aggregated energy data	A time series concerning adjusted metered readings received from a metered data collector and aggregated and validated by a metered data aggregator.

Losses	A time series concerning losses that have been calculated for a tieline or an area.

Transits (CBT)	A time series concerning inter area transit flows determined for CBT requirements.

Settlement deviation	A time series concerning the imbalance energy calculated by an imbalance

settlement responsible.

## Technical constraint deviation

A time series defining the imbalance between schedules accepted by the system operator due to technical constraints and schedules declared by the balance responsible party.

Balance energy deviation A time series defining the imbalance between the schedule of a balance

responsible party that has been corrected by the system operator after using balance energy bids and the schedule that was accepted by the system operator due to technical constraints.

Imbalance volume	A time series defining the imbalance between the actual meter readings and the

schedule of the balance responsible party corrected by the system operator after using balance energy bids.

Unintended energy	A timeseries concerning the volume of an unintended cross-border exchange of

energy.

Frequency control	A time series concerning primary and secondary reserve.

Balance management	A time series concerning energy balancing services.

Total trade	A time series concerning the total of both the internal and external trades.

## General Capacity Information

A time series providing the total capacity available on a TSO border.

## Available transfer capacity (ATC)

Available transfer capacity for cross-border exchanges.

## Net transfer capacity (NTC)

Net transfer capacity for cross-border exchanges.

Control Area Program	A time series providing the total exchanges between two TSOs (including the

commercial transactions, the compensation program and the losses compensation program).

Note this definition might change when UCTE brings forward its coding requirements.

## Already allocated capacity (AAC)

The already allocated capacity is the total amount of allocated transmission rights.

## Internal inter area trade

A trade that occurs between internal areas within a market balance area.

Offered Capacity	The time series provides the offered capacity.

## Capacity transfer notification

The time series provides information concerning the notification of the transfer of capacity to another market participant.

Authorised AAC	The time series in question provides the amount of transmission capacity rights to be nominated.

Capacity rights	The time series in question provides the capacity rights allocated for a given

border.

Minimum authorised AAC	The time series in question provides the minimum amount of transmission

capacity rights to be nominated.

Maximum authorised AAC	The time series in question provides the maximum amount of transmission

capacity rights to be nominated.

Installed generation	The time series in question provides the installed generation.

Available generation	The time series in question provides the available generation.

## Interconnection Trade Responsible Designation

The Time series in question provides the designation of the ITR that may nominate the capacity in question.

Released AAC	The already allocated capacity (AAC) that has been released for resale.

## Requested capacity (with price)

The time series in question provides information concerning the requested capacity including price information.

## Requested capacity (without price)

The time series in question provides information concerning the requested capacity but excludes price information.

Compensation program	Compensation of unintentional deviation is performed by exporting to / importing

from the interconnected system during the compensation period by means of schedules as calculated during the accounting of unintentional deviations.

## Schedule activated reserves

The cross border or internal reserves that are to be activated through schedule nomination.

## System Operator redispatching

The cross border redispatching between System Operators that are to be activated through schedule nomination.

Market capacity price	The price of the capacity offered on a given market.

## Market capacity price differential

The difference between the price of capacity in a Market Balance Area receiving the capacity (In Area) and the price of capacity in a Market Balance Area providing the capacity (Out Area), i.e. In Area Price - Out Area price.

Inflow	The volume of water that flows into a reservoir in a given interval.

Water extraction	The volume of water that can be extracted from a reservoir in a given interval.

Turbined water	The volume of water that can be turbined in a plant in a given interval.

Water spillage	The volume of water that is not turbined going through the spillway in a given

interval.

Planned maintenance	Maintenance has been planned for the object in question with a forecast ending

date.

Unplanned outage	An unplanned outage has occurred on the object in question.

## Use it Or Sell it (UIOSI) pricing

The time series provides information on the capacity resold in the "use it or sell it" process and its corresponding price.

## Compensation for auction cancellation where capacity is for resale

The time series provides information on the compensation of the capacity for resale following an auction cancellation.

Resale pricing	For each Physical Transmission Rights holder, this document contains the resold capacity and its corresponding price.

## Curtailed capacity compensation

The time series provides information to compensate a party when curtailment is applied on the capacity obtained in a previous auction, resale or transfer.

## Use it Or Sell it (UIOSI) compensation

The time series provides information on the compensation for the capacity following an auction cancellation.

Minimum possible	The time series provides a schedule of minimum possible values for a Resource

Object. The nature of the flow could be defined by the attribute Direction.

Maximum available	The time series provides a schedule of maximum available values for a Resource Object. The nature of the flow could be defined by the attribute Direction.

Spot price	The time series provides the market spot prices from an auction.

Minimum ATC	The Available Transmission Capacity that must be guaranteed because of

regulatory constraints.

Meter Measurement data	The data as provided for a meter measurement source.

## Accounting Point Relevant data

The metered data that is to be considered relevant for accounting purposes.

Energy flow	Energy flow information.

## Power plant energy Schedule

Energy flow scheduled for a power plant.

## Compensation Requirements for the compensation period

The time series provides the compensation requirements for a given compensation period.

Market coupling results	The time series provides the results of a market coupling auction.

Production, unavailable	Production capacity that normally would be available, but due to maintenance or

similar is temporarily unavailable.

## Supplementary available generation

The supplementary generation that is available.

## Interruptible consumption

The consumption that may be interrupted on request.

## Summarised Market Balance Area Schedule

A time series providing the total exchanges based on commercial transactions between two Market Balance Areas.

## Load Frequency Control Program Schedule

A time series providing the schedule information for the Load Frequency Control Program.

## Timeframe Independent Schedule

A time series providing the total exchanges of Timeframe Independent Schedules between two System Operators.

Consumption curtailment	A time series providing the amount of voluntary consumption curtailed by the

energy supplier of an end-consumer.

Production, dispatchable The nature of the business being described is dispatchable production details, i.e.

generation output that can be changed by a request (activation order) of the TSO according with the applicable Market Rules.

## Consumption, dispatchable

The nature of the business being described is dispatchable consumption details,

i.e. consumption output that can be changed by a request (activation order) of the TSO according with the applicable Market Rules.

## Production, non- dispatchable

The nature of the business being described is non-dispatchable production details, i.e. generation output that cannot be modified by an activation order.

## Consumption, non- dispatchable

The nature of the business being described is non-dispatchable consumption details, i.e. consumption output that cannot be modified by an activation order.

## Total Transfer Capacity (TTC)

The Total Transfer Capacity is the maximum exchange program between two areas compatible with operational security standards applicable at each system if future network conditions, generation and load patterns were perfectly known in advance.

## Mutual Emergency Assistance Service (MEAS)

The cross border Mutual Emergency Assistance Service between System Operators that are to be activated through schedule nomination.

Auction cancelation	The time series covers auction cancellation right.

Nomination curtailment	The time series covers nomination curtailment rights

Internal redispatch	Redispatch to relieve Market Balance Area internal congestion.

## Control area balance energy

A sum of secondary, tertiary control as well as other energy that was used to balance a control area.

Balancing energy price	Price of energy used to balance.

## Economised secondary reserve

The activated secondary reserve that had been economised due to pooled reserve management.

Spinning reserve	The extra generating capacity that is available by increasing the production of

generators that are already connected to the power system.

Solar	The business being described concerns solar power.

positive forecast margin The business being described concerns a positive forecast margin. Negative forecast margin The business being described concerns a negative forecast margin. Wind generation	The business being described concerns wind generation.

Solar generation	The business being described concerns solar generation.

## Frequency containment reserve

The business being described concerns frequency containment reserve.

## Automatic frequency restoration reserve

The business being described concerns automatic frequency restoration reserve.

## Manual frequency restoration reserve

The business being described concerns manual frequency restoration reserve.

Replacement reserve	The business being described concerns replacement reserve.

Financial information	The business being described concerns financial information.

## Interconnector network evolution

The business being described concerns interconnector network evolution.

## Interconnector network dismantling

The business being described concerns interconnector network dismantling.

Counter trade	The business being described concerns counter trades.

Congestion costs	The business being described concerns congestion costs.

## Capacity allocated (including price)

The business being described concerns capacity allocation and includes price information.

DC link constraint	The business being described concerns DC link constraints.

Auction revenue	The business being described concerns auction revenue.

Total nominated capacity The business being described concerns the total nominated capacity.

Net position	The business being described concerns net position. Congestion income	The business being described concerns congestion income. Production unit	The business being described concerns a production unit.

## Rounded market coupling results

Rounded outputs of the market coupling to be sent to TSOs and Market Participants.

Allocation Revenue	The time series provides information on the revenue generated by the allocations.

Production deviation	A time series concerning the imbalance energy between the metered production

and the schedules calculated by an imbalance settlement responsible.

Consumption deviation	A time series concerning the imbalance energy between metered consumption

and the forecasted consumption calculated by an imbalance settlement responsible.

Transmission asset	The business being described concerns a transmission asset. Consumption unit	The business being described concerns a consumption unit. In-feed ATC	Available Transfer Capacity at the in-feed side of a DC tieline. Out-feed ATC	Available Transfer Capacity at the out-feed side of a DC tieline.

## Balance up regulation price

A time series concerning balance regulation market prices for up regulation.

## Balance down regulation price

A time series concerning balance regulation market prices for down regulation.

Main direction	A time series concerning the direction of balance regulations.

## Consumption imbalance price

A time series concerning imbalance prices for consumption.

## Production sales imbalance price

A time series concerning imbalance prices for production sales.

## Production purchase imbalance price

A time series concerning imbalance prices for production purchase.

## Average balance price between MBAs

A time series concerning the average prices between Market Balance Areas.

Pumped	A time series concerning the electricity consumption related to pumping.

## Large installation consumption

A time series concerning consumption from large installation.

## Metering Grid Area (MGA) imbalance

A time series concerning imbalance between reported consumption, production and exchange in a Metering Grid Area.

HVDC Link settings	The time series in question provides HVDC Link settings.

## Transmission Reliability Margin (TRM)

A time series concerning Transmission Reliability Margin (TRM).

## Imbalance component for a pool

This information is used to provide to a pool manager the combined imbalance of

all the pool participants.

Area Control error (ACE) The sum of the instantaneous difference between the actual and the set-point

value of the measured total power value and Control Program including Virtual Tie-Lines for the power interchange of a LFC Area or a LFC Block and the frequency bias given by the product of the K-Factor of the LFC Area or the LFC Block and the Frequency Deviation.

## Area Control Error after Imbalance Netting

A time series concerning the Area Control Error after applying the imbalance netting energy correction.

## Implicit and explicit trade total

The sum of cross border schedules based on implicit and explicit trades including long term, yearly, monthly, weekly, daily processes.

## Production units own consumption

The consumption of one or more production units.

Constraint situation	The timeseries describes the constraint situation for a given TimeInterval.

A constraint situation can be:

- composed of a list of network elements in outage associated for each outage to a list of network elements on which remedial actions have been carried out accordingly to contingency process

- or it can be an external constraint.

Initial domain	The timeseries describe the full flow based domain for a given TimeInterval.

Critical network elements are displayed in details and their impact on the market is quantified.

## Flow based domain adjusted to long term schedules

The timeseries describe the full flow based domain for a given TimeInterval adjusted to the latest update of the schedules.

Critical network elements are displayed in details and their impact on the market is quantified.

## Network element constraint

The timeSeries describes limiting elements which are overloaded.

## Calculation opposition (Red Flag)

The timeSeries describes a party who is opposed to the calculation result and imposes its transfer capacity value.

## Base case proportional shift key

The GSK or LSK are proportional to the base case generation or load.

## Proportional to participation factors shift key

The GSK or LSK are proportional to the participation factors.

## Proportional to the remaining capacity shift key

The GSK is proportional to the remaining available capacity.

Merit order shift key	The GSK is proportional to a merit order list.

Wind speed	The TimeSeries provides information on the wind speed.

Wind direction	The TimeSeries provides information on the wind direction.

Solar irradiance	The TimeSeries provides information on the power per unit area produced by the sun in the form of electromagnetic radiation.

Air temperature	The TimeSeries provides information on the air temperature.

Cloudiness	The TimeSeries provides information on the cloudiness, i.e. the level of coverage of the sky with clouds.

Air humidity	The TimeSeries provides information on the level of humidity of the air.

Atmospheric pressure	The TimeSeries provides information on the atmospheric pressure.

Precipitation	The TimeSeries provides information on the amount of rain, snow, etc. that falls

on the ground.

## Network constraint situation that constraints the market

The TimeSeries describes the network elements, that constraints the market, to be taken into account to simulate a network constraint during the network load flow studies. The network situation includes the contingencies, the remedial actions, the monitored network elements and the potential additional constraints.

Contingency	The TimeSeries describes the network elements part of the contingency to be

simulated for a given TimeInterval.

Remedial Action	The TimeSeries describes a set of remedial actions for a given TimeInterval.

## Monitored Network Element

The TimeSeries describes the network elements to be monitored during the network load flow studies.

Busbar	The TimeSeries describes the network elements that composed a busbar.

Network Element	The TimeSeries describes network elements.

SPS	The TimeSeries describes the network elements managed by a Special Protection System (automation).

## Aggregated netted external market schedule

The aggregated netted external market schedules for a given border.

## Aggregated netted external TSO schedule

The aggregated netted external TSO schedules for a given border.

Aggregated netted	The aggregated netted external schedules for a given border.

## external schedule

Netted area AC position	The AC position for a given area.

Netted area position	The netted aggregation of all AC external schedules of an area plus the

aggregated External Netted Schedules of related HVDC links of an area.

## Interconnection shift key

The shift key series describes the amount of power to be shifted from a border area.

DC flow with losses	DC flow with losses refers to the values at the importing end of the DC line.

DC flow without losses	DC flow without losses refers to the values at the exporting end of the DC line.

## minimum value of netted area position

That value which a netted area position must not fall below for a given area.

## maximum value of netted area position

That value which a netted area position must not exceed for a given optimisation area.

maximum value of DC flow That value which a balanced DC flow must not exceed for a given DC line on

exporting end. When aligning DC flows CGMA algorithm will respect this constraint.

minimum value of DC flow That value which a balanced DC flow must not fall below for a given DC line on

exporting end. Currently this business type is only included for consistency reasons. It is always set to 0. This constraint might, however, be used in future. When aligning DC flows the CGMA algorithm will respect this constraint.

indicative AC flow	It is the hypothetical flow on the aggregate of all AC tie lines of an electrical border between two optimisation areas. It results from the adjustments to the preliminary netted area positions of all optimisation areas made by the CGMA algorithm.

Indicative AC flows are an artefact of the CGMA algorithm, and do not correspond to physical flows

Offer	The time series provides an offer to provide reserves.

Need	The time series provides a requirement for reserves.

## Opportunity costs or benefits

The time series describes any opportunity costs or benefits.

## Financial compensation or penalties

The time series describes any financial compensation or penalties

Global radiation The total short-wave radiation from the Global radiation is the total short-wave

radiation from the sky falling onto a horizontal surface on the ground. It includes both the direct solar radiation and the diffuse radiation resulting from reflected or scattered sunlight.

Diffuse radiation	Radiation resulting from reflected or scattered sunlight.

Direct solar radiation	Radiation resulting from direct sunlight

Outage (OUT)	Outage process: Element is out of operation due to planned maintenance or due

to an unplanned/forced outage. Outage may be used as a synonym for unavailability.

## Special switching state (SSS)

Outage Process: This state applies to grid elements which are in operation in an exceptional state (e.g. separated nodes operation).

Testing (TEST)	Outage process: TESTING means any element status is possible - ON or OUT.

This status applies either between first connection and final commissioning of the relevant asset, or directly following maintenance of the relevant asset.

## Auxiliary busbar operation

Outage process: Element is in operation but connected via auxiliary busbar

Automatic reclosing	Outage process: Protection function Automatic reclosing is switched off for electric line

Busbar protection	Protection function busbar protection is switched off

Phase Shift Angle	The maximum phase shift angle allowed between two network elements.

## Base Case Network Situation

The TimeSeries describes the network elements to be taken into account to simulate a base case network situation during the network load flow studies, without any contingency.

Inter-TSO assistance	Cross border assistance schedule between TSOs not interconnected directly.

FlexibleNeed	The business type indicates that the need is optional.

GLSK Limitation	A constraint related to a GLSK maximum or minimum limitation in the production

or/and consumption shift.

## Capacity ramping limitation

A constraint related to a ramping limitation on the capacity offered at a given border.

interconnector capacity	The maximum capacity that can be exchanged on an interconnector, excluding

external factor on both ends.

Must Run	A time series concerning must run generation.

Procured capacity	An accepted offer of balancing capacity.

Used capacity	The used cross-zonal balancing capacity.

Estimated costs	Estimated costs of the process.

Estimated benefits	Estimated benefits of the process.

Load Shedding	A time series concerning a load shedding used to avoid failure of the power

system.

Remaining Capacity	A time series concerning the remaining capacity.

## Indicator of generation capacity adequacy

Indicator of adequacy, it indicates if there is final generation remaining capacity after SMTA calculation.

## Income from price divergence without congestions

The time series describes income due to price divergence without congestion between bidding zones.

Push-button	The cross-border Push-button service between System Operators. Intertripping	The cross-border Intertripping service between System Operators. Emergency instruction	The cross-border Emergency instruction service between System Operators.

Ramp management	The schedule resulting from cross-border Ramp management service between

System Operators.

Profile smoothing	The schedule resulting from cross-border Profile smoothing service between

System Operators.

## Emergency reallocation deselection

The schedule resulting from cross-border Emergency reallocation deselection service between System Operators.

SO-SO-trade	The generic cross border trade between System Operators.

Production reduction	A time series providing the volume of production reduced by an energy provider / producer / supplier.

Maximum power exchange	The timeseries provides the maximum admissible power flow between two bidding zones respecting operational security limits taking into account N-1 criterion.

## Maximum power exchange after remedial actions

The timeseries provides the maximum admissible power flow between two bidding zones after remedial actions.

## Network constraint situation that cannot

The TimeSeries describes the network elements, that cannot limit the market, to

limit the market	be taken into account to simulate a network constraint during the network load

flow studies. The network situation includes the contingencies, the remedial

actions, the monitored network elements and the potential additional constraints.

## Flat participation for all generators or loads

Flat GSK factors of all generators or loads, independently of the size.

## Proportional to installed capacity of generators

Generators participate relative to their maximum (installed) capacity (MW).

## Market price and total volume

A time series concerning market price and total volume.

Import price	A time series concerning import price (the volume-weighted price average of all

accepted bids).

## Capacity allocated (excluding price)

The business being described concerns capacity allocation and excludes price information.

## Common Grid Model Equipment

The timeseries provides equipment related to the Common Grid Model (CGM).

## Exchanged balancing reserve capacity

The balancing reserve capacity exchanged between areas.

## Shared balancing reserve capacity

The balancing reserve capacity shared between areas.

## Share of reserve capacity

A time series concerning the share of reserve capacity.

Actual reserve capacity	A timeseries concerning actual reserve capacity.

K-factor	K-factor as stated in the SO GL Art. 2 (45). It is also known as Frequency Bias.

## Frequency Containment Reserve-Normal (FCR-N)

FCR-N is a reserve that is automatically activated in both directions around a set point when the frequency varies between 50.10 Hz and 49.90 Hz after an imbalance.

## Frequency Containment Reserve-Disturbance (FCR-D)

FCR-D is a reserve that is automatically activated when the frequency falls below

49.90 Hz or rises above 50.1 Hz after an imbalance.

## Internal trade difference

A time series concerning internal trade difference, within an area, such as a Bidding Zone or Scheduling Area. The internal trade difference is the difference between trades reported from an out party (seller) and an in party (buyer).

Small scale production	Production from small scale production plants.

## System price

The system price is an unconstrained market clearing reference price. It is calculated without any congestion restrictions by setting capacities to infinity.

Wind gust	An increase in the speed of the wind lasting for a short period.

Area imbalance	A time series concerning imbalance between planned consumption, production

and exchange in an Area.

Unintended energy price	A timeseries concerning the price of the unintended cross-border exchange of

energy.

## Frequency containment process energy

A timeseries containing the volume of energy resulting from the frequency containment process.

## Frequency containment process energy price

A timeseries containing the energy price from the frequency containment process.

Ramping period energy	A timeseries containing the volume of energy exchanged as a result of ramping

between different ANES values.

## Ramping period energy price

A timeseries containing the price of the energy exchanged as a result of ramping between different ANES values.

Frequency deviation	A timeseries concerning the difference between the actual and the nominal

frequency of a synchronous area.

Day-Ahead market price	A timeseries concerning Day-Ahead market prices.

Conditional bid	Standard product bid that is conditional on bids submitted outside of common

platform.

Thermal limit	The current causing a given network element to work outside of the range of safe operating temperatures.

Frequency Limit	A constraint related to the containment of frequency deviations within a given

area.

Voltage limit	The maximum or minimum permissible voltage within normal operation state of a given network element.

Current limit	The maximum permissible current within normal operation state of a given

network element.

## Short circuit current limit

The maximum permissible short-circuit current within normal operation state of a given network element.

Dynamic stability limit	A maximum permissible load ensuring the control of oscillations in the grid and

avoiding the loss of synchronism.

Disconnection	A timeseries describing disconnection of a TSO from a common platform.

## Intended energy with positive price

A timeseries concerning the amount of intended energy with prices higher than zero (and including zero).

## Intended energy with negative price

A timeseries concerning the amount of intended energy with prices lower than zero (excluding zero).

Decopuling	A time series describing decoupling of an area.

Resource capacity unit	A timeseries containing information about resource capacity units.

## Resource entry capacity data

A timeseries containing the resource capacity that can be allocated to an eligible resource capacity operator from another area.

## Resource capacity obligation data

A timeseries containing the resource capacity operator obligation to guarantee delivery.

Available energy	A timeseries concerning the available energy. Production curtailment	A timeseries concerning the curtailment of production. Rounding error	A timeseries describing a rounding error.

Metered frequency	The timeseries provides information about metered frequency.

## Adjusted TTC to the nominal criteria

The exchange program between two areas which guarantees that the Margin Available for Cross-Zonal Trade (MACZT) fulfils the nominal criteria at least on the most limiting Critical Network Element with Contingency (CNEC) which limits the transfer capacity.

## Adjusted TTC to the nominal criteria with TSOs limitation

The exchange program between two areas which allows the Margin Available for Cross-Zonal Trade (MACZT) on, at least, the most limiting Critical Network Element with Contingency (CNEC) to get closer to the nominal criteria fulfilment with a limited impact on the rest of the network.

## Frequency deviation larger than standard deviation

Total time in which the absolute value of the instantaneous frequency deviation was larger than the standard frequency deviation.

## Frequency deviation larger than maximum deviation

Total time in which the absolute value of the instantaneous frequency deviation was larger than the maximum instantaneous frequency deviation.

## Frequency deviation not returned to 50%

Number of events in which the absolute value of the instantaneous frequency deviation of the synchronous area exceeded 200 % of the standard frequency

deviation as stated in SO GL (EU) regulation Art 131.1.a.vi.

## Frequency deviation not returned to restoration range

Number of events in which the absolute value of the instantaneous frequency deviation of the synchronous area exceeded 200 % of the standard frequency deviation.

## Frequency deviation outside recovery range

Number of events for which the absolute value of the instantaneous frequency deviation was outside of the frequency recovery range.

Frequency	A time series describing measurement frequency.

Mean value	A time series describing mean values. Standard deviation	A time series describing standard deviation. Percentile	A time series describing percentiles.

## Measured frequency resolution

A time series describing the resolution of a measured frequency.

Accuracy	A time series describing measurement accuracy.

## FRCE outside level 1 range

The number of time intervals in which the average value of the FRCE was outside the Level 1 FRCE range as stated in SO GL. (EU) regulation Art 131.1.b.i.

## FRCE outside level 2 range

The number of time intervals in which the average value of the FRCE was outside the Level 2 FRCE range as stated in SO GL (EU) regulation. Art 131.1.b.i.

## FRCE exceeded 60% of FRR capacity

The number of events for which the FRCE exceeded 60 % of the reserve capacity on FRR as stated in SO GL (EU) regulation Art 131.1.b.ii.

## FRCE exceeded steady state deviation

The number of events for which the absolute value of the FRCE exceeded the maximum steady-state frequency deviation.

## FRCE calculation and accuracy descriptor

A time series describing how FRCE is calculated and its accuracy.

Forecasted capacity	A time series describing forecasted capacity.

## Minimum available capacity

A time series describing minimum available capacity.

## Average available capacity

A time series describing average available capacity.

## Maximum available capacity

A time series describing maximum available capacity.

## Frequency and accuracy descriptor

A time series describing how system frequency and accuracy are determined.

## Long-Term internal redispatch

A time series describing long-term redispatch to relieve Scheduling Area internal congestion.

Other unavailability	This is an unplanned unavailability. Not considered by market participants as a

planned maintenance.

Faster than standard FAT Bids that can support a ?Full Activation Time? (FAT) that is faster than standard

FAT. Fast activation can be done for bids with activation time shorter than the minimum requirement for the standard product. When circumstances call for it, the TSO can order activation of such bids on a shorter notice.

## Faster than standard deactivation time

Fast deactivation can be done for bids with activation time shorter than the minimum requirement for the standard product. When circumstances call for it, the TSO can order activation of such bids on a shorter notice.

Slower than standard FAT Bids that can support a Full Activation Time (FAT) that is slower than standard

FAT.

## Remedial action cost summary

A timeseries summarizing all incurred costs and/or revenues per party related to the activated remedial actions eligible to Cost Sharing.

Settlement result	A timeseries representing the cost sharing settlement results between parties (i.e. which party will be paying and/or receiving money for the concerned period).

## Reserved cross zonal capacity

The reserved cross zonal capacity for system operator needs (EBGL art. 38(1)(b) and art. 41.).

Energy reserves	A timeseries describing energy reserves.

## Combined dynamic constraint

The time series describes a combined dynamic constraint, being defined as a limit on the sum of power flows on a set of network elements or partial flows on a set of network elements for the purpose to respect dynamic stability limits.

Anonymised constraint	The time series describes an anonymised constraint.

## Synchronous condenser activation

Activates a number of generators or other units in synchronous condenser mode.

## Permission administration details

A timeseries describing permission details such as authorisations, licenses or related regulatory requirements.

## Maximum capacity value of AC border flow

Maximum flow on an AC border between two areas (capacity)

Initial forecast value	Initial flow on an AC border between two areas (forecast)

of AC border flow

This curve is a restriction of the curve type A04, i.e. overlapping breakpoints. The restriction is that a single Period is allowed.

Non-overlapping breakpoint

The curve is made of successive Intervals of time of variable duration (size), where the end date and end time of each interval are equal to the start date and start time of the next Interval.

Overlapping breakpoint

The curve is made of successive Intervals of time (Blocks) of variable duration (size), where the end date and end time of each Block are equal to the start date and start time of the next Interval. For the last Block the end date and end time of the last Interval would be equal to EndDateTime of TimeInterval.

Variable sized Block

The curve is made of successive instants of time (Points).

Point

The curve is made of successive Intervals of time (Blocks) of constant duration (size), where the size of the Blocks is equal to the Resolution of the Period.

Sequential fixed size block

ESMPEnumerations

CurveTypeList

The type of curve being defined in the time series.

The direction at a given instant in time is considered to be stable.

Stable

Up and Down signifies that the UP and Down values are equal.

UP and DOWN

Down signifies that the available power can be used by the Purchasing area to decrease energy.

DOWN

Up signifies that the available power can be used by the Purchasing area to increase energy.

UP

ESMPEnumerations

DirectionTypeList

The coded identification of the direction of energy flow.

# Compound types

ESMP_DateTimeInterval

ESMPDataTypes

This datatype enables to express the start date and time, and the end date and time of a time interval with a specific pattern. This pattern is the YYYY-MM-DDThh:mmZ.

| start | 1..1 | YMDHM_DateTime | The start date and time of the interval with a minute resolution. |
| --- | --- | --- | --- |
| end | 1..1 | YMDHM_DateTime | The end date and time of the interval with a minute resolution. |

Datatypes

Main Core value Space.

1..1	DateTime

value

ESMPDataTypes

ESMP_DateTime

In ESMP, the dateTime shall be expressed in UTC as YYYY-MM-DDThh:mm:ssZ.

Main Core value Space.

1..1	Integer

value

An integer value, this value is used as a sequential value representing the relative position of an entity within a space such as a time interval.

ESMPDataTypes

Position_Integer

Main Core value Space.

1..1	QualityTypeList

value

ESMPDataTypes

Quality_String

The coded identification of the quality of the information.

Main Core value Space.

1..1	UnitOfMeasureTypeList

value

The coded identification of a unit of measure that is applied to a quantity. The measurement units shall be in compliance with UN/ECE Recommendation 20.

ESMPDataTypes

MeasurementUnitKind_String

ID_String

A code to uniquely distinguish one occurrence of an entity from another.

ESMPDataTypes

In the ESMP context, the code is defined either by:

- an emitting company that provides an agreed identification unique within a business context such as capacity

PartyID_String

The identification of an actor in the energy market.

ESMPDataTypes

In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for market participant identification.

Main Core value Space.

1..1	ProcessTypeList

value

ESMPDataTypes

ProcessKind_String

The coded identification of the nature of process.

auction identification, market agreement identification, etc.

- a party (originator of the exchange) that provides a unique identification in the framework of a business exchange such as document identification, time series identification, bid identification, ...

value

1..1	String

Main Core value Space.

Main Core value Space.

1..1	RoleTypeList

value

ESMPDataTypes

MarketRoleKind_String

The identification of the role played by a party.

| codingScheme | 1..1 | CodingSchemeTypeList | DomainQualification. |
| --- | --- | --- | --- |
| value | 1..1 | String | Main Core value Space. |

Main Core value Space.

1..1	String

value

A code that distinguishes one evolution of an identified object from another. Information about a specific object may be sent several times, each transmission being identified by a different version number.

ESMPDataTypes

ESMPVersion_String

In ESMP, the coded value is restricted to digits.

YMDHM_DateTime

ESMPDataTypes

In ESMP, the date and time is expressed as "YYYY-MM-DDThh:mmZ", which conforms with ISO 8601 UTC time zone. This date and time is without the seconds.

ResourceID_String

The identification of a resource object in the energy market.

ESMPDataTypes

In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for resources (generator, lines, substations, etc.) identification.

Main Core value Space.

1..1	MessageTypeList

value

ESMPDataTypes

MessageKind_String

The coded type of a document.

value

1..1	DateTime

The date and time as "YYYY-MM- DDThh:mmZ", which conforms with the ISO 8601 UTC time zone.

Main Core value Space.

1..1	BusinessTypeList

value

ESMPDataTypes

BusinessKind_String

The coded identification of the business type.

Main Core value Space.

1..1	CurveTypeList

value

ESMPDataTypes

CurveType_String

The coded identification of the type of curve.

Main Core value Space.

1..1	DirectionTypeList

value

ESMPDataTypes

DirectionKind_String

The coded identification of the direction of energy flow.

| codingScheme | 1..1 | CodingSchemeTypeList | DomainQualification. |
| --- | --- | --- | --- |
| value | 1..1 | String | Main Core value Space. |