List of stereotye to categorize subProfiles ShortCircuit Description Operation European Abstract Entsoe

# Concrete Classes

| Cadastre Grid Hosting Capacity Document assembly model A singular complete record of all land parcels within some hierarchical level of administrative jurisdiction. The cadastre provides the ability to find the results of land parcel surveys typically used for land management and planning. Native Members (European) |  |  |  |
| --- | --- | --- | --- |
| energyIdentCodeEic (European) | 0..1 | String | The attribute is used for an exchange of the EIC code (Energy identification Code). The length of the string is 16 characters as defined by the EIC code. For details on EIC scheme please refer to ENTSO-E web site. |
| shortName (European) | 0..1 | String | The attribute is used for an exchange of a human |
|  |  |  | readable short name with length of the string 12 |
|  |  |  | characters maximum. |
| Native Members |  |  |  |
| administrationLevel | 0..1 | String | Administrative level is referring to the organisation that is administrative authority for managing the a hierarchical division of a geographic area into land, property groups. This typically established by government that has the responsibility of managing governance, services, taxation, and legal jurisdiction, such as municipalities, townships, counties, or regions. |
| aliasName | 0..1 | String | The aliasName is free text human readable name of the object alternative to IdentifiedObject.name. It may be non unique and may not correlate to a naming hierarchy. The attribute aliasName is retained because of backwards compatibility between CIM releases. It is however recommended to replace aliasName with the Name class as aliasName is planned for retirement at a future time. |
| cadastralReference | 0..1 | String | Local or national unique identification reference for the cadastral for land parcel, ensuring effective land management, ownership tracking, and legal compliance. |
| description | 0..1 | String | The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. |
| identifier | 0..1 | UUID | A universally unique object identifier. Used to uniquely identify persistent objects between CIM messages. |
| levelFourDivision | 0..1 | String | Level Four Division refers to a further subdivision of a Level Three Division within a cadastral hierarchy, typically used to represent specific units or sections within a shared property or structure, such as |

apartments, condominiums, or co-owned facilities. levelOneDivision 0..1 String Level One Division refers to the highest organizational or reference unit within a cadastral hierarchy, typically representing a primary division of land, such as a farm or property group, which can be further subdivided into smaller parcels. levelThreeDivision 0..1 String Level Three Division refers to a further subdivision of a Level Two Division within a cadastral hierarchy, typically used to represent specialized land units such as leased parcels, easements, or other secondary divisions associated with distinct legal or usage rights. levelTwoDivision 0..1 String Level Two Division refers to a subdivision of a Level One Parcel within a cadastral hierarchy, representing smaller, distinct land units typically used for individual ownership, usage, or development, such as specific plots or lots within a farm or property group. mRID 1..1 String Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non unique text naming the object.

Circle Grid Hosting Capacity Document assembly model A circular area in space, described by a center point and radius. Any circle object must have a center and an outer radius and may have an inner radius. If an inner radius is specified, the area within the inner radius is to be excluded from the area within the outer radius, forming a ring shape. Native Members innerRadius 0..1 ESMP_Length A circle may have an inner radius. If specified the area described by the inner radius is to be excluded from the area within the outer radius, forming a ring shape. outerRadius 0..1 ESMP_Length Every circle must have an outer radius which describes the outer limit of the circular area. CircleCenter_LinearCoordinate [0..1] LinearCoordinate A point representing the center of a circular area. Any circle object must have one CircleCenter. CircleCenterPositionPoint [0..1] PositionPoint A point representing the center of a circular area. Any circle object must have one CircleCenter.

EnvAsset Grid Hosting Capacity Document assembly model asset is an energy data space specific attribute referring to the Asset from which data is being requested

Native Members meterId 0..1 String meterId is an energy data space specific attribute that specifies the id of the meter that measures the asset. operatorId 0..1 String operatorId is an energy data space specific attribute that specifies the id operator of the asset refered to. type 0..1 String type is an energy data space specific attribute that specifies type of asset refered to.

Geometry Grid Hosting Capacity Document assembly model Geo-spatial representation of a physical location. A location may be represented as a Point, Line, Polygon, or Circle. Native Members coordinateSystem.crsUrn 0..1 String A Uniform Resource Name (URN) for the coordinate reference system (crs) used to define 'Location.PositionPoints'. An example would be the European Petroleum Survey Group (EPSG) code for a coordinate reference system, defined in URN under the Open Geospatial Consortium (OGC) namespace as: urn:ogc:def:uom:EPSG::XXXX, where XXXX is an EPSG code (a full list of codes can be found at the EPSG Registry web site http://www.epsg-registry.org/). To define the coordinate system as being WGS84 (latitude, longitude) using an EPSG OGC, this attribute would be urn:ogc:def:uom:EPSG::4236. A profile should limit this code to a set of allowed URNs agreed to by all sending and receiving parties. --- The coordinate system used for a given geometry. mRID 1..1 String Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non unique text naming the object.

GHC_Envelope Grid Hosting Capacity Document assembly model The envelope entity is a non-CIM object used in an energy data space environment to serve as a central top-level entity connecting the MarketDocumentHeader (energy data space specific context information block) with the regular CIM MarketDocument. Native Members MessageDocumentHeader [0..1] MessageDocumentHeader GHC_MarketDocument [0..1] GHC_MarketDocument

| GHC_MarketDocument Grid Hosting Capacity Document assembly model An electronic document containing the information necessary to satisfy the requirements of a given business process. Native Members |  |  |  |
| --- | --- | --- | --- |
| createdDateTime | 0..1 | ESMP_DateTime | The date and time of the creation of the document. |
| description | 0..1 | String | The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. |
| legalInformationText | 0..1 | String | Text with the required legal information from the information provider (Connecting System operator=CSO) |
| legalInformationWebsites | 0..1 | String | URL pointing to the required legal information from the information provider (Connecting System operator=CSO) |
| mRID | 1..1 | ID_String | The unique identification of the document being exchanged within a business process flow. In the ESMP context, the "model authority" is defined as a party (originator of the exchange) that provides an identification in the context of a business exchange such as document identification, ... Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |
| publishedDateTime | 0..1 | DateTime | The date time the legal information is published |
| receiver_MarketParticipant.marketRole.type 1..1 |  | MarketRoleKind_String | The identification of the role played by a market player. --- The MarketParticipant associated with an electronic document header. --- The role associated with a |

MarketParticipant. receiver_MarketParticipant.mRID 1..1 PartyID_String The identification of a party in the energy market. In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification. Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. --- The MarketParticipant associated with an electronic document header. receiver_MarketParticipant.name 0..1 String The name is any free human readable and possibly non unique text naming the object. --- The MarketParticipant associated with an electronic document header. revisionNumber 0..1 ESMPVersion_String The identification of the version that distinguishes one evolution of a document from another. sender_MarketParticipant.marketRole.type 1..1 MarketRoleKind_String The identification of the role played by a market player. --- The MarketParticipant associated with an electronic document header. --- The role associated with a MarketParticipant. sender_MarketParticipant.mRID 1..1 PartyID_String The identification of a party in the energy market. In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification. Master resource identifier issued by a model authority. The mRID

is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. --- The MarketParticipant associated with an electronic document header. sender_MarketParticipant.name 0..1 String The name is any free human readable and possibly non unique text naming the object. --- The MarketParticipant associated with an electronic document header.

HostingCapacity Grid Hosting Capacity Document assembly model A structure for a set of information on grid hosting capacity. Native Members capacityDataDetails 0..1 String Informational field for further information on timing (e.g. is time for capacity available for connection a snapshot or is it the time when connection can be performed) description of the SO description of the data provided, description of the meaning of the different colours firstConnectionYear 0..1 Integer The year that the capacity is applicable. identifier 0..1 UUID A universally unique object identifier. Used to uniquely identify persistent objects between CIM messages. mRID 1..1 String Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non unique text naming the object. publicationDate 0..1 Date The date when the data was produced or when it can be available

HostingCapacityDetail Grid Hosting Capacity Document assembly model

| A structure for grid hosting capacity information at a certain year and duration for a certain customerconnectiontype. Native Members |  |  |  |
| --- | --- | --- | --- |
| appliesFromYear | 0..1 | Integer | The year that the capacity is first applicable. |
| capacityAvailableForConnection | 0..1 | CEEDS_ActivePower | Information on firm |
|  |  | capacity available for |  |
|  |  | new connections in |  |
|  |  | MW. Requirement for |  |
|  |  | grid hosting capacity |  |
|  |  | information in (EU) |  |
|  |  | 2019/943 and (EU) |  |
|  |  | 2019/944 |  |
| capacityUnderConnectionRequest | 0..1 | CEEDS_ActivePower | Information on |
|  |  | connections requests |  |
|  |  | in |  |
|  |  | MW. Requirement for |  |
|  |  | grid hosting capacity |  |
|  |  | information in (EU) |  |
|  |  | 2019/943 and (EU) |  |
|  |  | 2019/944 |  |
| customerConnectionType | 1..1 | CustomerConnectionTypeKind | Information on |
|  |  | customer connection |  |
|  |  | types for which grid |  |
|  |  | hosting capacity |  |
|  |  | information is |  |
|  |  | applicable. |  |
|  |  | Possible values are: |  |
|  |  | Consumption |  |
|  |  | Production |  |
|  |  | Energy storage |  |
|  |  | Hybrid park |  |
| customerSubConnectionType | 1..1 | CustomerSubConnectionTypeKind | Information on |
|  |  | customer connection |  |
|  |  | types (subgroups) for |  |
|  |  | which grid hosting |  |
|  |  | capacity information is |  |
|  |  | applicable. |  |
|  |  | Possible values are: |  |
|  |  | Solar |  |
|  |  | Wind |  |
|  |  | EV charger |  |
|  |  | Industry |  |
| customerSubSubConnectionType | 1..1 | CustomerSubSubConnectionTypeKind Information on |  |
|  |  | customer connection |  |
|  |  | types (subsubgroups) |  |
|  |  | for which grid hosting |  |
|  |  | capacity information is |  |
|  |  | applicable. |  |
|  |  | Possible values are: |  |
|  |  | EV charger light vehicle |  |
|  |  | EV charger heavy duty |  |

| vehicle Industry - High usage Industry - Average usage |  |  |  |
| --- | --- | --- | --- |
| durationApplicable | 0..1 | Integer | The period in years |
|  |  |  | starting from |
|  |  |  | appliesYearFrom that |
|  |  |  | the capacity is |
|  |  |  | available. |
|  |  |  | So: |
|  |  |  | appliesYearFrom=2028 |
|  |  |  | durationApplicable=2 |
|  |  |  | resulting period: 2028- |
|  |  |  | 2030 |
| FlexibleConnectionCapacity | 0..1 | CEEDS_ActivePower | Information on |
|  |  |  | possibility of flexible |
|  |  |  | connection capacity in |
|  |  |  | MW. Requirement for |
|  |  |  | grid hosting capacity |
|  |  |  | information in (EU) |
|  |  |  | 2019/943 and (EU) |
|  |  |  | 2019/944 |
| levelOfFlexibility | 0..1 | String | Description of flexible |
|  |  |  | connection agreement |
|  |  |  | (e.g. " 2 percent of |
|  |  |  | yearly energy", "20 |
|  |  |  | percent of capacity", |
|  |  |  | "seasonal difference of |
|  |  |  | capacity') |
| totalCapacity | 0..1 | CEEDS_ActivePower | Information on total capacity in MW. |
| withConsiderationOfCapacityFromOverlyingGrid 0..1 |  | Boolean | The capacity shown is |
|  |  |  | with (TRUE) or without |
|  |  |  | (FALSE) consideration |
|  |  |  | of available capacity |
|  |  |  | from overlying grid. |

LinearCoordinate Grid Hosting Capacity Document assembly model Specifies a point on a line segment, including a sequence number and bulge. To visualize the line segment, each linear coordinate is to be connected in the order specified by the sequence attribute. Each linear coordinate must have a sequence value. Native Members bulge 0..1 Decimal Bulge is a factor that describes a curved line segment between two points. bulge=0 is a straight line: bulge=1 is a curve along the edge of a circle. A bulge can be associated only with the second point of a line segment. A negative bulge factor causes the line to bulge to the left as viewed from the first point to the second; a positive bulge causes the line to be curved to the right as viewed from the first point to the second.

relativeHeight.verticalOffset 0..1 ESMP_Length Distance above (positive value) or below (negative value) the specified vertical offset reference. relativeHeight.verticalOffsetReference 0..1 String The height from which vertical offset is measured. Examples include “Ground Level”, “Pole Top”, “Tower Top”, etc. sequenceNumber 1..1 Integer The order in which linear coordinates are to be connected to form the line described. Sequence is required for all linear coordinates. xPosition 0..1 String X axis position. yPosition 0..1 String Y axis position. zPosition 0..1 String (if applicable) Z axis position.

LineElement Grid Hosting Capacity Document assembly model A geometric line segment, described by 2 or more coordinate tuples with linear interpolation between them. A line must contain at least 2 linear coordinates. Native Members

Location Grid Hosting Capacity Document assembly model The place, scene, or point of something where someone or something has been, is, and/or will be at a given moment in time. It can be defined with one or more position points (coordinates) in a given coordinate system. Native Members mRID 1..1 String The unique identification of a location. In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification, location, etc. Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non unique text naming the object.

Site [0..1] Site

| MessageDocumentHeader Grid Hosting Capacity Document assembly model MessageDocumentHeader is an energy data space specific entity with the creationDateTime of this document. Native Members |  |  |
| --- | --- | --- |
| creationDateTime | 0..1 ESMP_DateTime | The date and time the MessageDocumenHeader was created. |
| MetaInformation | [0..1] MetaInformation |  |

| MetaInformation Grid Hosting Capacity Document assembly model MetaInformation is an energy data space specific entity referring to the information concerning the technical and functional context of the MarketDocument. Native Members |  |  |  |
| --- | --- | --- | --- |
| connectionId | 0..1 | String | connectionId is an energy data space specific attribute referring to the established connection used for the communication of this document. |
| dataNeedId | 0..1 | String | dataNeedId is an energy data space specific attribute holding the type of data need requested, like ‘last 24 hours at 1 hour interval”. |
| dataSourceId | 0..1 | String | dataSourceId is an energy data space specific attribute referring to the datasource from which the requested data is being queried. |
| defaultValues | 0..1 | String | defaultValues is a energy data space specific attribute to hold relevant default values. The format to use can freely be chosen based on the implementation. Most commonly you would expect a semi-colon separated name value pair, like: "minPower=value; maxPower=value; avgPower=value". |
| documentType | 0..1 | String | documentType is an energy data space specific attribute referring to the type of document, comparable to the MessageTypeList in the CIM part of the message. Note: This is applicable if you have the need to know the content without having to open the complete document. DocumentType might contain messages not (yet) part of the MessageTypeList. |
| finalCustomerId | 0..1 | String | The finalCustomerId is an energy data space specific attribute identifying the actor doing the request for data or data sharing. |
| regionConnector | 0..1 | String | regionConnector is an energy data space specific attribute that specifies the regional connector being used to retrieve the data requested. |

| regionCountry | 0..1 String | regionCountry is an energy data space specific attribute that specifies the country where the regional connector is located. |
| --- | --- | --- |
| requestPermissionId | 0..1 String | The requestPermissionId is energy data space specific identification attribute holding an unique identifier issued by the energy data space to allow for tracing of the permission through serveral message exchanges. Note: the requestPermissionId is NOT the identifier issued by the Identity Service Provider as specified in the EU Implementing Act on access to Metering and Consumption Data. |
| EnvAsset | [0..1] EnvAsset |  |

| PhysicalAddress Grid Hosting Capacity Document assembly model The structured geographic and administrative context of a physical place where an organisation, person, or activity is located. PhysicalAddress is used for precisely identifying, classifying, and locating entities for regulatory, logistical, or informational purposes. Native Members (European) |  |  |  |
| --- | --- | --- | --- |
| energyIdentCodeEic (European) | 0..1 | String | The attribute is used for an exchange of the EIC code (Energy identification Code). The length of the string is 16 characters as defined by the EIC code. For details on EIC scheme please refer to ENTSO-E web site. |
| shortName (European) | 0..1 | String | The attribute is used for an exchange of a human |
|  |  |  | readable short name with length of the string 12 |
|  |  |  | characters maximum. |
| Native Members |  |  |  |
| aliasName | 0..1 | String | The aliasName is free text human readable name of the object alternative to IdentifiedObject.name. It may be non unique and may not correlate to a naming hierarchy. The attribute aliasName is retained because of backwards compatibility between CIM releases. It is however recommended to replace aliasName with the Name class as aliasName is planned for retirement at a future time. |
| cityCode | 0..1 | String | City code. |
| cityName | 0..1 | String | Name of the city. |
| community | 1..1 | String | Community in which the locality is, and which is in the municipality, city or town.. For example, Anderston, Glasgow City, Scotland or another appropriate fourth- level administrative division |
| country | 0..1 | String | Name of the country in which the address is located. It is recommended to use either the 3-letter ISO 3166-1 alpha 3 code such as "SGP," or the full country name such as "Singapore." |

county 0..1 String Name of the second-level Administrative division of a country in which the address is located. The term used for the second-level Administrative division in a county varies from country to country and between first-level Administrative divisions within a country. For example, California has counties, Louisiana has parishes, British Columbia, Canada has regional districts, and Queensland Australia has shires and regions. Counties are usually made up of cities, towns, and rural populations. description 0..1 String The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. fullAddress 1..1 String A concatenated string specifying the complete address in a single string which may include delimiters. globalLocationNumber 1..1 String Global Location Number (GLN, sometimes also referred to as International Location Number or ILN) of the respective organization, person, or place. The GLN is a 13-digit number used to identify parties and physical locations. identifier 0..1 UUID A universally unique object identifier. Used to uniquely identify persistent objects between CIM messages. isicV4 1..1 String International Standard of Industrial Classification of All Economic Activities (ISIC), Revision 4 code for a particular organization, business person, or place. isWithinTownLimits 0..1 Boolean If TRUE, this street is within the legal geographical boundaries of the specified town (default). mRID 1..1 String Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non unique text naming the object. postalCode 0..1 String Postal code for the address. region 0..1 String Name of the first-level Administrative division of a country in which the address is located. For example, California, Quebec, and Queensland are states and provinces of the United States of America, Canada, and Australia, respectively. status 0..1 Status Current status of the address e.g. pending, approved streetCode 0..1 String (if applicable) Utilities often make use of external reference systems, such as those of the town-planner's

department or surveyor general's mapping system, that allocate global reference codes to streets. streetName 0..1 String Name of the street. streetNumber 0..1 String Designator of the specific location on the street. streetPrefix 0..1 String Prefix to the street name. For example: North, South, East, West. streetSuffix 0..1 String Suffix to the street name. For example: North, South, East, West. suiteNumber 0..1 String Number of the apartment or suite.

Polygon Grid Hosting Capacity Document assembly model One or more self-closing line segments representing a polygonal area. In order to be self-closing, the first and last linear coordinates of each line segment must be identical. A polygon must have one outer boundary and may have one or more inner boundaries. If an inner boundary is specified, the area within the inner boundary is to be excluded from the area within the outer boundary. Each line description within a polygon (both outer and inner boundaries) must contain at least 4 linear coordinates. Native Members

PositionPoint Grid Hosting Capacity Document assembly model Set of spatial coordinates that determine a point, defined in the coordinate system specified in 'Location.CoordinateSystem'. Use a single position point instance to describe a point-oriented location. Use a sequence of position points to describe a line-oriented object (physical location of non-point oriented objects like cables or lines), or area of an object (like a substation or a geographical zone - in this case, have first and last position point with the same values). Native Members relativeHeight.verticalOffset 0..1 ESMP_Length Distance above (positive value) or below (negative value) the specified vertical offset reference. relativeHeight.verticalOffsetReference 0..1 String The height from which vertical offset is measured. Examples include “Ground Level”, “Pole Top”, “Tower Top”, etc. xPosition 0..1 String X axis position. yPosition 0..1 String Y axis position. zPosition 0..1 String (if applicable) Z axis position.

ServiceArea Grid Hosting Capacity Document assembly model Native Members

identifier 0..1 UUID A universally unique object identifier. Used to uniquely identify persistent objects between CIM messages. licenseArea.mRID 0..1 String Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. licenseArea.name 0..1 String The name is any free human readable and possibly non unique text naming the object. mRID 1..1 String Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. name 0..1 String The name is any free human readable and possibly non unique text naming the object. stationName 0..1 String The human readable name of the station. UsagePointLocation [0..1] UsagePointLocation Location of this power system resource. Location [0..1] Location Location of this power system resource.

Site Grid Hosting Capacity Document assembly model Area of ground on which work can be executed, organisation can be hosted, and building (including substation and power line) can be located. Native Members PhysicalAddress [1..1] PhysicalAddress

UsagePointLocation Grid Hosting Capacity Document assembly model Location of an individual usage point. Native Members accessMethod 0..1 String Method for the service person to access this usage point location. For example, a description of where to obtain a key if the facility is unmanned and secured. description 0..1 String The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy.

| geoInfoReference | 0..1 | String | (if applicable) Reference to geographical information source, often external to the utility. |
| --- | --- | --- | --- |
| mRID | 1..1 | String | The unique identification of a location. In the ESMP context, the "model authority" is defined as an authorized issuing office that provides an agreed identification coding scheme for market participant, domain, measurement point, resources (generator, lines, substations, etc.) identification, location, etc. Master resource identifier issued by a model authority. The mRID is globally unique within an exchange context. Global uniqueness is easily achieved by using a UUID for the mRID. It is strongly recommended to do this. For CIMXML data files in RDF syntax, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |
| name | 0..1 | String | The name is any free human readable and possibly non unique text naming the object. |
| type | 0..1 | String | Classification by utility's corporate standards and practices, relative to the location itself (e.g., geographical, functional accounting, etc., not a given property that happens to exist at that location). |
| Site [0..1] Site |  |  |  |

| VoltageLevel Grid Hosting Capacity Document assembly model For a production unit or a generating unit, the high voltage level at the network side. For other network equipment the voltage level. Native Members |  |  |  |
| --- | --- | --- | --- |
| baseVoltage.nominalVoltage 1..1 |  | ESMP_Voltage | The power system resource's base voltage, expressed on a phase-to-phase (line-to-line) basis. Shall be a positive value and not zero. --- The base voltage used for all equipment within the voltage level. |
| highVoltageLimit | 1..1 | ESMP_Voltage | The bus bar's high voltage limit |
| identifier | 0..1 | UUID | A universally unique object identifier. Used to uniquely identify persistent objects between CIM messages. |
| lowVoltageLimit | 0..1 | ESMP_Voltage | The bus bar's low voltage limit |
| mRID | 1..1 | String | Master resource identifier issued by a model authority. The mRID is unique within an exchange context. Global uniqueness is easily achieved by using a UUID, as specified in IETF RFC 4122, for the mRID. The use of UUID is strongly recommended. For CIMXML data files in RDF syntax conforming to IEC 61970-552, the mRID is mapped to rdf:ID or rdf:about attributes that identify CIM object elements. |

name 0..1 String The name is any free human readable and possibly non unique text naming the object. UsagePointLocation [0..1] UsagePointLocation Location of this power system resource. Location [0..1] Location Location of this power system resource.

Enumerations

UnitSymbol ESMPEnumerations The coded representation of different units from IEC 61970. Ampere The unit of electrical current in the International system of Units (SI) equivalent to one Coulomb per second. One A unit for dimensionless quantities, also called quantities of dimension one. kV The symbol of kV MW The symbol of MW Percent A unit of proportion equal to 0.01. Ohm The symbol of Ohm Unit MVAr The symbol of MVAr Hertz HTZ unit as per UN/CEFACT recommendation 20 megavolt-ampere MVA unit as per UN/CEFACT recommendation 20 degree (unit of angle) A unit of measurement of angles expressed in a 0 to 360 degree gradient. grams per kilowatt hour It represents the mass rate of emissions per unit of work accomplished.

RoleTypeList ESMPEnumerations Identification of the role played by a party. Trade responsible party Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Consumption responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. party Combined power exchange This role is no longer in the ENTSO-E Harmonised Role Model Document. (not to be used)

System operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Imbalance settlement Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. responsible Production responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. party Transmission capacity Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. allocator Balance responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. party Metered data aggregator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Billing agent Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Market operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Energy supplier Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Consumer Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Control area operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Control block operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Coordination centre Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. operator Grid access provider Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Grid operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Meter administrator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Party connected to grid Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Producer Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Profile maintenance Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. party Meter operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Metered data collector Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Metered data responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Metering point Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. administrator

Resource Provider Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Scheduling coordinator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Capacity Trader Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Interconnection Trade Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Responsible Nomination Validator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Market information Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. aggregator A party that collects information from different sources and assembles it to provide a summary of the market. Information receiver Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. A party, not necessarily a market participant, which receives information about the market. Reserve Allocator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. A party that informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders. MOL Responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. A party that Informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders. Capacity Coordinator A party, acting on behalf of the SOs involved, responsible for establishing a coordinated Offered Capacity and/or NTC and/or ATC between several Scheduling Areas. Reconciliation Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Accountable A party that is financially accountable for the reconciled volume of energy products for a profiled local metering point. Reconciliation A party that is responsible for reconciling, within a metering grid area, the volumes used in the Responsible imbalance settlement process for profiled metering points and the actual metered quantities. Data provider A party that is responsible for providing information to a central authority. Local Issuing Office A party that is responsible for operating a Local Issuing Office (LIO). (LIO) Central Issuing Office A party that is responsible for operating a Central Issuing Office (CIO). (CIO) EIC Participant A party that participates in the EIC environment. Weather analyser A party that analyses the current and forecast weather situation and establishes a prognosis of its impact on the renewable energy environment as well as the overall load. Regional Security The RSC as defined in the System Operation guideline. Coordinator (RSC)

Energy Service Company A party offering energy-related services to the Party Connected to Grid, but not directly active in the (ESCO) energy value chain or the physical infrastructure itself. The ESCO may provide insight services as well as energy management services. Balancing Service A party with reserve-providing units or reserve-providing groups able to provide balancing services Provider to LFC Operators. Energy trader A party that is selling or buying energy. LFC Operator A party that is responsible for the Load Frequency Control of its LFC Area or LFC Block. Transmission System The Transmission System Operator (TSO) is responsible for the transport of electricity on the extra Operator (TSO) high-voltage and high-voltage interconnected system. This is a market participant and not a role in Harmonised Role Model. Distribution System Distribution System Operator (DSO) is responsible for transport of electricity on high-voltage Operator (DSO) (optionally), medium-voltage and low-voltage distribution systems. This is a market participant and not a role in Harmonised Role Model. Resource Capacity Resource capacity mechanism operator is the party responsible to operate the resource capacity Mechanism Operator mechanism in a member state. It can either be the TSO or an independent party. Resource aggregator A party that aggregates resources for usage by a service provider for energy market services. Cost sharing calculator A party responsible for the mapping of the costs between areas and parties. Settlement responsible A party responsible for settling the costs. Coordinated Capacity Coordinated Capacity Calculator is the entity or entities with the task of calculating transmission Calculator capacity, at regional level or above. Flexibility service A party that offers flexibility services based on acquired (aggregated) resources. provider Data access provider A party responsible for facilitating access to data. Metered data A party responsible for storing and distributing validated measured data. administrator Permission administrator A party responsible for administrating a register of data access permissions for a set of metering points, making this information available to final customers and eligible parties in the sector, on request.

CodingSchemeTypeList ESMPEnumerations Codification scheme used to identify the coding scheme used for the set of coded values to identify specific objects. EIC The coding scheme is the Energy Identification Coding Scheme (EIC), maintained by ENTSO-E. Coding scheme which are The coding schemes maintained by GS1. part of GS1 system Andorra National coding The National coding scheme of the country in question. scheme

Albania National coding The National coding scheme of the country in question. scheme Armenia National coding The National coding scheme of the country in question. scheme Austria National coding The National coding scheme of the country in question. scheme Azerbaijan National The National coding scheme of the country in question. coding scheme Bosnia and Herzegovina The National coding scheme of the country in question. National coding scheme Belgium National coding The National coding scheme of the country in question. scheme Bulgaria National coding The National coding scheme of the country in question. scheme Switzerland National The National coding scheme of the country in question. coding scheme Serbia and Montenegro The National coding scheme of the country in question. National coding scheme Czech Republic National The National coding scheme of the country in question. coding scheme Germany National coding The National coding scheme of the country in question. scheme Denmark National coding The National coding scheme of the country in question. scheme Estonia National coding The National coding scheme of the country in question. scheme Spain National coding The National coding scheme of the country in question. scheme Finland National coding The National coding scheme of the country in question. scheme France National coding The National coding scheme of the country in question. scheme United Kingdom National The National coding scheme of the country in question. coding scheme Georgia National coding The National coding scheme of the country in question. scheme Gibraltar National The National coding scheme of the country in question. coding scheme Greece National coding The National coding scheme of the country in question. scheme

Croatia National coding The National coding scheme of the country in question. scheme Hungary National coding The National coding scheme of the country in question. scheme Ireland National coding The National coding scheme of the country in question. scheme Italy National coding The National coding scheme of the country in question. scheme Kyrgyzstan National The National coding scheme of the country in question. coding scheme Kazakhstan National The National coding scheme of the country in question. coding scheme Liechtenstein National The National coding scheme of the country in question. coding scheme Lithuania National The National coding scheme of the country in question. coding scheme Luxembourg National The National coding scheme of the country in question. coding scheme Latvia National coding The National coding scheme of the country in question. scheme Morocco National coding The National coding scheme of the country in question. scheme Moldavia National coding The National coding scheme of the country in question. scheme Macedonia National The National coding scheme of the country in question. coding scheme Netherlands National The National coding scheme of the country in question. coding scheme Nordic Regional coding The coding scheme of the Nordic region which covers Denmark, Finland, Norway and Sweden. scheme Norway National coding The National coding scheme of the country in question. scheme Poland National coding The National coding scheme of the country in question. scheme Portugal National coding The National coding scheme of the country in question. scheme Romania National coding The National coding scheme of the country in question. scheme Russian Federation The National coding scheme of the country in question. National coding scheme

Sweden National coding The National coding scheme of the country in question. scheme Slovenia National coding The National coding scheme of the country in question. scheme Slovakia National coding The National coding scheme of the country in question. scheme Turkey National coding The National coding scheme of the country in question. scheme Ukraine National coding The National coding scheme of the country in question. scheme CGM The coding scheme used for Common Grid Model Exchange Standard (CGMES). Cyprus National coding The National coding scheme of the country in question. scheme ISO The coding scheme for the preceding attribute is a code maintained by International Organization for Standardization (ISO), such as “ISO 3166 English country names and code elements”.

CustomerConnectionTypeKind CEEDS_ESMP_CodeLists Consumption Production Energy storage The customer type relevant for the capacity data is Industry customer – energy storage Hybrid park

CustomerSubConnectionTypeKind CEEDS_ESMP_CodeLists Solar Wind EV charger The customer type relevant for the capacity data is EV chargers Light vehicles Industry The customer type relevant for the capacity data is Industry customer – high users

CustomerSubSubConnectionTypeKind CEEDS_ESMP_CodeLists EV charger light vehicle The customer type relevant for the capacity data is EV chargers Light vehicles EV charger heavy duty The customer type relevant for the capacity data is EV chargers Heavy Duty vehicle

Industry - High usage The customer type relevant for the capacity data is Industry customer – high users Industry - Average usage The customer type relevant for the capacity data is Industry customer – average users

Compound types

Status CEEDS_ESMP_DataTypes Current status information relevant to an entity. value 0..1 String dateTime 0..1 DateTime remark 0..1 String reason 0..1 String

Datatypes

ESMP_Length CEEDS_ESMP_DataTypes ConstantValue= MTR unit 1..1 UnitSymbol value 1..1 Float

UnitSymbol ESMPDataTypes A code to identify an analog measurement. A code to uniquely distinguish one occurrence of an entity from another. In the ESMP context, the code is defined by an emitting company that provides an agreed identification unique within a business context such as capacity auction identification, market agreement identification, etc. value 1..1 UnitSymbol Main Core value Space.

ESMP_DateTime ESMPDataTypes In ESMP, the dateTime shall be expressed in UTC as YYYY-MM-DDThh:mm:ssZ. value 1..1 DateTime Main Core value Space.

ID_String ESMPDataTypes A code to uniquely distinguish one occurrence of an entity from another.

In the ESMP context, the code is defined either by: - an emitting company that provides an agreed identification unique within a business context such as capacity auction identification, market agreement identification, etc. - a party (originator of the exchange) that provides a unique identification in the framework of a business exchange such as document identification, time series identification, bid identification, ... value 1..1 String Main Core value Space.

MarketRoleKind_String ESMPDataTypes The identification of the role played by a party. value 1..1 RoleTypeList Main Core value Space.

PartyID_String ESMPDataTypes The identification of an actor in the energy market. In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for market participant identification. codingScheme 1..1 CodingSchemeTypeList DomainQualification. value 1..1 String Main Core value Space.

ESMPVersion_String ESMPDataTypes In ESMP, the coded value is restricted to digits. A code that distinguishes one evolution of an identified object from another. Information about a specific object may be sent several times, each transmission being identified by a different version number. value 1..1 String Main Core value Space.

CEEDS_ActivePower CEEDS_ESMP_DataTypes value 1..1 Float Main core space value. The value is given in decimal format. ConstantValue= MAW unit 1..1 UnitSymbol The unit of the value expressed using the UN/CEFACT recommendations 20.

ESMP_Voltage ESMPDataTypes The coded identification of a voltage value. value 1..1 Float Main Core value Space. The value is expressed as a simple precision and no

mantisse. ConstantValue= KVT unit 1..1 UnitSymbol The unit of the value, the UN/CEFACT recommendation 20 is used as coding scheme of the unit.