List of stereotye to categorize subProfiles ShortCircuit Description Operation European Abstract Entsoe

Concrete Classes

CEEDSActivation_Envelope CEEDS_Activation assembly model The envelope entity is a non-CIM object used in an energy data space environment to serve as a central top-level entity connecting the MarketDocumentHeader (energy data space specific context information block) with the regular CIM MarketDocument. Native Members MessageDocumentHeader [0..1] MessageDocumentHeader MarketDocument [0..1] CEEDSActivation_MarketDocument

CEEDSActivation_MarketDocument CEEDS_Activation assembly model An electronic document containing the information necessary to satisfy the requirements of a given business process. Native Members activation_Time_Period.timeInterval 1..1 ESMP_DateTimeInterval The start and end date and time for a given interval. --- This information provides the start and end date and time of the activation time interval. createdDateTime 1..1 ESMP_DateTime The date and time of the creation of the document. domain.mRID 0..1 AreaID_String The unique identification of the domain. --- The identification of the domain that is covered in the document.

mRID 1..1 ID_String The unique identification of the document being exchanged within a business process flow. order_MarketDocument.mRID 0..1 ID_String The unique identification of the document being exchanged within a business process flow. order_MarketDocument.revisionNumber 0..1 ESMPVersion_String The identification of the version that distinguishes one evolution of a document from another. process.processType 0..1 ProcessKind_String The identification of the nature of process that the document addresses. --- The process dealt with in the document. receiver_MarketParticipant.marketRole.type 1..1 MarketRoleKind_String The identification of the role played by a market player. --- Document recipient. --- The role associated with a MarketParticipant. receiver_MarketParticipant.mRID 1..1 PartyID_String The identification of a party in the energy market. --- Document recipient. revisionNumber 1..1 ESMPVersion_String The identification of the version that distinguishes one evolution of a document from another.

sender_MarketParticipant.marketRole.type 1..1 MarketRoleKind_String The identification of the role played by a market player. --- Document owner. --- The role associated with a MarketParticipant. sender_MarketParticipant.mRID 1..1 PartyID_String The identification of a party in the energy market. --- Document owner. subject_MarketParticipant.marketRole.type 0..1 MarketRoleKind_String The identification of the role played by a market player. --- --- The role associated with a MarketParticipant. subject_MarketParticipant.mRID 0..1 PartyID_String The identification of a party in the energy market. type 1..1 MessageKind_String The coded type of a document. The document type describes the principal characteristic of the document.

EnvAsset CEEDS_Activation assembly model EnvAsset is an energy data space specific attribute referring to the Asset from which data is being requested Native Members meterId 0..1 String meterId is an energy data space specific attribute that specifies the id of the meter that measures the asset. operatorId 0..1 String operatorId is an energy data space specific attribute that specifies the

id operator of the asset refered to. type 0..1 String type is an energy data space specific attribute that specifies type of asset refered to.

MessageDocumentHeader CEEDS_Activation assembly model MessageDocumentHeader is an energy data space specific entity with the creationDateTime of this document. Native Members creationDateTime 0..1 ESMP_DateTime The date and time the MessageDocumenHeader was created. MetaInformation [0..1] MetaInformation

MetaInformation CEEDS_Activation assembly model MetaInformation is an energy data space specific entity referring to the information concerning the technical and functional context of the MarketDocument. Native Members connectionId 0..1 String connectionId is an energy data space specific attribute referring to the established connection used for the communication of this document. dataNeedId 0..1 String dataNeedId is an energy data space specific attribute holding the type of data need requested, like ‘last 24 hours at 1 hour interval”. dataSourceId 0..1 String dataSourceId is an energy data space specific attribute referring to the datasource from which the requested data is being queried. defaultValues 0..1 String defaultValues is a energy data space specific attribute to hold relevant default values. The format to use can freely be chosen based on the implementation. Most commonly you would expect a semi-colon separated name value

pair, like: "minPower=value; maxPower=value; avgPower=value". documentType 0..1 String documentType is an energy data space specific attribute referring to the type of document, comparable to the MessageTypeList in the CIM part of the message. Note: This is applicable if you have the need to know the content without having to open the complete document. DocumentType might contain messages not (yet) part of the MessageTypeList. finalCustomerId 0..1 String The finalCustomerId is an energy data space specific attribute identifying the actor doing the request for data or data sharing. regionConnector 0..1 String regionConnector is an energy data space specific attribute that specifies the regional connector being used to retrieve the data requested. regionCountry 0..1 String regionCountry is an energy data space specific attribute that specifies the country where the regional connector is located. requestPermissionId 0..1 String The requestPermissionId is energy data space specific identification attribute holding an unique identifier issued by the energy data space to allow for tracing of the permission through serveral message exchanges. Note: the requestPermissionId is NOT the identifier issued by the Identity Service Provider as specified in the EU Implementing Act on access to Metering and Consumption Data. EnvAsset [0..1] EnvAsset

Point CEEDS_Activation assembly model The identification of the values being addressed within a specific interval of time.

Native Members position 1..1 Position_Integer A sequential value representing the relative position within a given time interval. quantity 1..1 Decimal The principal quantity identified for a point.

Quantity CEEDS_Activation assembly model Description of quantities needed in the data exchange. The type of the quantity is described either by the role of the association or the quantityType attribute. The quality attribute provides the information about the quality of the quantity (measured, estimated, etc.). Native Members quality 0..1 Quality_String The description of the quality of the quantity. quantity 0..1 Decimal The quantity value. The association role provides the information about what is expressed. type 0..1 String The description of the type of the quantity.

Reason CEEDS_Activation assembly model The motivation of an act. Native Members code 1..1 ReasonCode_String The motivation of an act in coded form. text 0..1 ReasonText_String The textual explanation corresponding to the reason code.

Series_Period CEEDS_Activation assembly model The identification of the period of time corresponding to a given time interval and resolution.

Native Members resolution 1..1 Duration The definition of the number of units of time that compose an individual step within a period. timeInterval 1..1 ESMP_DateTimeInterval The start and end time of the period.

| TimeSeries CEEDS_Activation assembly model A set of time-ordered quantities being exchanged in relation to a product. Native Members |  |  |  |
| --- | --- | --- | --- |
| acquiring_Domain.mRID | 1..1 | AreaID_String | The unique |
|  |  | identification of |  |
|  |  | the domain. |  |
|  |  | --- The area |  |
|  |  | where the |  |
|  |  | product is being |  |
|  |  | delivered. |  |
| businessType | 1..1 | BusinessKind_String The identification of the nature of the time series. |  |
| connecting_Domain.mRID | 1..1 | AreaID_String | The unique |
|  |  | identification of |  |
|  |  | the domain. |  |
|  |  | --- The area |  |
|  |  | where the |  |
|  |  | product is being |  |
|  |  | extracted. |  |
| flowDirection.direction | 1..1 | DirectionKind_String | The coded |
|  |  | identification of |  |
|  |  | the direction of |  |
|  |  | energy flow. |  |
| marketObjectStatus.status | 1..1 | Status_String | The coded |
|  |  | condition or |  |
|  |  | position of an |  |
|  |  | object with |  |
|  |  | regard to its |  |
|  |  | standing. |  |
| measurement_Unit.name | 1..1 | MeasurementUnitKind_String The identification of the formal code for a |  |

| measurement |  |  |
| --- | --- | --- |
| unit (UN/ECE |  |  |
| Recommendation |  |  |
| 20). |  |  |
| --- The unit of |  |  |
| measurement |  |  |
| used for the |  |  |
| quantities |  |  |
| expressed within |  |  |
| the time series. |  |  |
| mRID 1..1 | ID_String | A unique identification of the time series. |
| registeredResource.mRID 0..1 | ResourceID_String | The unique |
|  |  | identification of |
|  |  | resource. |
|  |  | --- The |
|  |  | identification of |
|  |  | resource |
|  |  | associated with |
|  |  | TimeSeries. |
| resourceProvider_MarketParticipant.mRID 1..1 | PartyID_String | The identification |
|  |  | of a party in the |
|  |  | energy market. |
|  |  | --- The |
|  |  | identification of |
|  |  | the party putting |
|  |  | the product into |
|  |  | the in area. |

a

a a

Enumerations

CodingSchemeTypeList ESMPEnumerations Codification scheme used to identify the coding scheme used for the set of coded values to identify specific objects. EIC The coding scheme is the Energy Identification Coding Scheme (EIC), maintained by ENTSO-E. Coding scheme which are The coding schemes maintained by GS1. part of GS1 system Andorra National coding The National coding scheme of the country in question. scheme Albania National coding The National coding scheme of the country in question.

scheme Armenia National coding The National coding scheme of the country in question. scheme Austria National coding The National coding scheme of the country in question. scheme Azerbaijan National The National coding scheme of the country in question. coding scheme Bosnia and Herzegovina The National coding scheme of the country in question. National coding scheme Belgium National coding The National coding scheme of the country in question. scheme Bulgaria National coding The National coding scheme of the country in question. scheme Switzerland National The National coding scheme of the country in question. coding scheme Serbia and Montenegro The National coding scheme of the country in question. National coding scheme Czech Republic National The National coding scheme of the country in question. coding scheme Germany National coding The National coding scheme of the country in question. scheme Denmark National coding The National coding scheme of the country in question. scheme Estonia National coding The National coding scheme of the country in question. scheme Spain National coding The National coding scheme of the country in question. scheme Finland National coding The National coding scheme of the country in question. scheme France National coding The National coding scheme of the country in question. scheme United Kingdom National The National coding scheme of the country in question. coding scheme Georgia National coding The National coding scheme of the country in question. scheme

Gibraltar National The National coding scheme of the country in question. coding scheme Greece National coding The National coding scheme of the country in question. scheme Croatia National coding The National coding scheme of the country in question. scheme Hungary National coding The National coding scheme of the country in question. scheme Ireland National coding The National coding scheme of the country in question. scheme Italy National coding The National coding scheme of the country in question. scheme Kyrgyzstan National The National coding scheme of the country in question. coding scheme Kazakhstan National The National coding scheme of the country in question. coding scheme Liechtenstein National The National coding scheme of the country in question. coding scheme Lithuania National The National coding scheme of the country in question. coding scheme Luxembourg National The National coding scheme of the country in question. coding scheme Latvia National coding The National coding scheme of the country in question. scheme Morocco National coding The National coding scheme of the country in question. scheme Moldavia National coding The National coding scheme of the country in question. scheme Macedonia National The National coding scheme of the country in question. coding scheme Netherlands National The National coding scheme of the country in question. coding scheme Nordic Regional coding The coding scheme of the Nordic region which covers Denmark, Finland, scheme Norway and Sweden.

Norway National coding The National coding scheme of the country in question. scheme Poland National coding The National coding scheme of the country in question. scheme Portugal National coding The National coding scheme of the country in question. scheme Romania National coding The National coding scheme of the country in question. scheme Russian Federation The National coding scheme of the country in question. National coding scheme Sweden National coding The National coding scheme of the country in question. scheme Slovenia National coding The National coding scheme of the country in question. scheme Slovakia National coding The National coding scheme of the country in question. scheme Turkey National coding The National coding scheme of the country in question. scheme Ukraine National coding The National coding scheme of the country in question. scheme CGM The coding scheme used for Common Grid Model Exchange Standard (CGMES). Cyprus National coding The National coding scheme of the country in question. scheme ISO The coding scheme for the preceding attribute is a code maintained by International Organization for Standardization (ISO), such as “ISO 3166 English country names and code elements”.

ProcessTypeList ESMPEnumerations Indicates the nature of process that the document addresses. Day ahead The information provided concerns a day ahead process. Intra day incremental The information provided concerns an intra day schedule.

Inter-area transit The information provided concerns an inter area transit schedule. The rules governing this process are market dependent System operation closure The information provided concerns the closure of a given period of both scheduled and regulation information. Metered data aggregation The information provided concerns the aggregation process of metered information. Imbalance settlement The information provided concerns the imbalance settlement for a given period for a balance responsible party or parties. Capacity allocation The information provided concerns the capacity allocation process. Central reconciliation The process carried out to finalise the imbalance settlement based on actual metered values against provisional values from profiled metering points. Released capacity The process concerns the notification of capacity rights that are being released. allocation Proposed capacity The process concerns the proposed capacity to be allocated for a given border. allocation Agreed capacity The process concerns the capacity that has been agreed for allocation for a allocation border. Long term The process concerns scheduling all schedules except daily and intraday contracts. Post scheduling The process concerns the adjustments made to previous schedules after the adjustment period of execution. Forecast The data contained in the document are to be handled in short, medium, long term forecasting process. Capacity determination The process of determining the capacity for use. Realised The process for the treatment of realised data as opposed to forecast data. Schedule day The process concerns the day ahead, intraday and eventually ex-post scheduling in a single document. The schedule will be transferred within the total position including historic information. Intraday total This process concerns an intraday schedule which contains the accumulated day ahead and intraday current position.

Intraday accumulated This process concerns a single intraday schedule process where only intraday evolutions occur through version changes. SOMA process System Operator meter alignment process. SOVM process System Operator validated measurement process. RGCE accounting process The document provides ENTSO-E Regional Group Continental Europe accounting process information. CCSR RGCE Settlement The process concerns the control center settlement report for the whole of the ENTSO-E Regional Group Continental Europe. CBSR Settlement The process concerns the control block settlement report. CASR Settlement The process concerns the control area settlement report. Outage information The process concerns TSO publication of outages on its power system. Reserve resource process The process being described is for general reserve resources. Primary reserve process The process being described is for primary reserves. Secondary reserve The process being described is for secondary reserves. process Tertiary reserve process The process being described is for tertiary reserves. Week ahead The process being described is for the week ahead. Month ahead The process being described is for the month ahead. Year ahead The process being described is for the year ahead. Contracted The process being described is for contracted information. Network information The process being described is for network information. Creation The process being described is for the creation of information. Modification The process being described is for the modification of information. Deactivation process The process being described is for deactivation of information.

Synchronisation process The process being described is for the synchronisation of information. Intraday process The process being described is for intraday process. Redispatch process The process being described is for redispatch activation. Activation history The process being described is for the provision of an activation history. process Flow based domain The information provided concerns the flow-based process in day ahead. constraint day-ahead Flow based domain The information provided concerns the flow-based process in intraday. constraint intraday Two days ahead Two days ahead. Replacement reserve A process being described is for replacement reserves (RR) to restore or support the required level of frequency restoration reserves. Manual frequency A process being described is for manual frequency restoration reserves restoration reserve (mFRR). Day-ahead capacity The process run at the day-ahead timeframe to determine the capacity for use. determination Intraday capacity The process run at the intraday timeframe to determine the capacity for use. determination Long term capacity The process run at the long term timeframe to determine the capacity for use. determination Automatic frequency A process being described is for automatic frequency restoration reserves restoration reserve (aFRR). Frequency containment A process being described is for frequency containment reserves (FCR). reserve Common Grid Model (CGM) The process for merging individual grid models to form the common grid model. merging process Coordinated security The process being described is used for coordinated security analysis. analysis Exchange of master data A process for exchanging master data.

Frequency restoration The process being described is for general frequency restoration reserve. reserve FSKAR settlement The information provided concerns the Financial Settlement of K?f, ACE and ramping period settlement for a given period. mFRR capacity market Processes related to the Reserve option market to assure that there are enough available reserves for the manual FRR market. Internal trade reporting The process related to internal trade reporting. Scheduled activation mFRR being subject to scheduled activation. mFRR Direct activation mFRR mFRR being subject to direct activation. Registration A process related to the registration and management of object information. Imbalance Netting The process described is for imbalance netting. Criteria application for The process describes criteria application for instantaneous frequency. instantaneous frequency Criteria application for The process describes criteria application for frequency restoration. frequency restoration Cost sharing The process describes the cost sharing process for costly remedial actions. Central selection aFRR aFRR subject to central selection of bids for activation. Local selection aFRR aFRR subject to local selection of bids for activation. Common grid model The process for aligning bidding zone net positions. alignment Pan-European The process for verifying bidding zone net positions. verification function Outage planning The process being described is used for outage planning coordination. coordination Short term adequacy The process being described is used for short term adequacy calculations. Coordinated capacity The process being described is used for coordinated capacity calculation. calculation Access to metered data Access to meter readings of electricity consumption from or electricity feed into

the grid. Permission Administration of permissions to access and/or use data. administration

RoleTypeList ESMPEnumerations Identification of the role played by a party. Trade responsible party Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Consumption responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model party Document. Combined power exchange This role is no longer in the ENTSO-E Harmonised Role Model Document. (not to be used) System operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Imbalance settlement Refer to role model definitions in the ENTSO-E Harmonised Role Model responsible Document. Production responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model party Document. Transmission capacity Refer to role model definitions in the ENTSO-E Harmonised Role Model allocator Document. Balance responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model party Document. Metered data aggregator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Billing agent Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Market operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Energy supplier Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Consumer

Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Control area operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Control block operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Coordination centre Refer to role model definitions in the ENTSO-E Harmonised Role Model operator Document. Grid access provider Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Grid operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Meter administrator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Party connected to grid Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Producer Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Profile maintenance Refer to role model definitions in the ENTSO-E Harmonised Role Model party Document. Meter operator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Metered data collector Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Metered data responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Metering point Refer to role model definitions in the ENTSO-E Harmonised Role Model administrator Document. Resource Provider Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Scheduling coordinator Refer to role model definitions in the ENTSO-E Harmonised Role Model

Document. Capacity Trader Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Interconnection Trade Refer to role model definitions in the ENTSO-E Harmonised Role Model Responsible Document. Nomination Validator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. Market information Refer to role model definitions in the ENTSO-E Harmonised Role Model aggregator Document. A party that collects information from different sources and assembles it to provide a summary of the market. Information receiver Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. A party, not necessarily a market participant, which receives information about the market. Reserve Allocator Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. A party that informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders. MOL Responsible Refer to role model definitions in the ENTSO-E Harmonised Role Model Document. A party that Informs the market of reserve requirements, receives tenders against the requirements and in compliance with the prequalification criteria, determines what tenders meet requirements and assigns tenders. Capacity Coordinator A party, acting on behalf of the SOs involved, responsible for establishing a coordinated Offered Capacity and/or NTC and/or ATC between several Scheduling Areas. Reconciliation Refer to role model definitions in the ENTSO-E Harmonised Role Model Accountable Document. A party that is financially accountable for the reconciled volume of energy products for a profiled local metering point. Reconciliation A party that is responsible for reconciling, within a metering grid area, the Responsible volumes used in the imbalance settlement process for profiled metering points and the actual metered quantities. Data provider A party that is responsible for providing information to a central authority. Local Issuing Office A party that is responsible for operating a Local Issuing Office (LIO).

(LIO) Central Issuing Office A party that is responsible for operating a Central Issuing Office (CIO). (CIO) EIC Participant A party that participates in the EIC environment. Weather analyser A party that analyses the current and forecast weather situation and establishes a prognosis of its impact on the renewable energy environment as well as the overall load. Regional Security The RSC as defined in the System Operation guideline. Coordinator (RSC) Energy Service Company A party offering energy-related services to the Party Connected to Grid, but not (ESCO) directly active in the energy value chain or the physical infrastructure itself. The ESCO may provide insight services as well as energy management services. Balancing Service A party with reserve-providing units or reserve-providing groups able to provide Provider balancing services to LFC Operators. Energy trader A party that is selling or buying energy. LFC Operator A party that is responsible for the Load Frequency Control of its LFC Area or LFC Block. Transmission System The Transmission System Operator (TSO) is responsible for the transport of Operator (TSO) electricity on the extra high-voltage and high-voltage interconnected system. This is a market participant and not a role in Harmonised Role Model. Distribution System Distribution System Operator (DSO) is responsible for transport of electricity on Operator (DSO) high-voltage (optionally), medium-voltage and low-voltage distribution systems. This is a market participant and not a role in Harmonised Role Model. Resource Capacity Resource capacity mechanism operator is the party responsible to operate the Mechanism Operator resource capacity mechanism in a member state. It can either be the TSO or an independent party. Resource aggregator A party that aggregates resources for usage by a service provider for energy market services. Cost sharing calculator A party responsible for the mapping of the costs between areas and parties. Settlement responsible A party responsible for settling the costs. Coordinated Capacity Coordinated Capacity Calculator is the entity or entities with the task of Calculator calculating transmission capacity, at regional level or above.

Flexibility service A party that offers flexibility services based on acquired (aggregated) provider resources. Data access provider A party responsible for facilitating access to data. Metered data A party responsible for storing and distributing validated measured data. administrator Permission administrator A party responsible for administrating a register of data access permissions for a set of metering points, making this information available to final customers and eligible parties in the sector, on request.

MessageTypeList ESMPEnumerations The coded type of a document. The message type describes the principal characteristic of a document. This enumeration is used in the XML instances based on IEC 62325. Balance responsible A schedule which has been prepared by a balance responsible party providing schedule planned schedule information. Allocated capacity A schedule which has been prepared by a capacity allocator providing allocated schedule capacity. Balance area schedule A schedule that provides the planned schedule information for a balance area. System Operator area A compilation of all external schedules concerning two System Operator areas schedule or a connector concerning two System Operator of all balance responsible parties. Control block area A compilation of all the exchange programs of all control areas for one control schedule block with all neighbouring control areas of a neighbouring control block. Coordination center area A compilation of the exchange programs of all exchange blocks divided into schedule UCTE south and north. Intermediate An intermediate confirmation report that may be produced between final cutoffs. confirmation report Final confirmation A final confirmation report that is produced after a final cutoff. report Finalised schedule A compilation of a set of schedules that have been finalized after a given cutoff. Regulation data report A compilation of the time series employed on a given day to ensure the balance of the system.

Aggregated energy data A compilation of the time series of all the meter readings or their equivalent for report a given period. Imbalance report The report containing the complete situation of a given period for a party and including the schedules, regulation data and actual or calculated readings. Interconnection Capacity Document for cross-border capacity exchanges. Resource Provider A document providing the schedules for resource objects submitted by a Resource Schedule resource provider. Acquiring System A document providing reserve purchases submitted by an Acquiring System Operator Reserve Operator. Schedule Anomaly Report A document providing anomaly information for the receiving party to correct. Acknowledgement Document A document providing acknowledgement information. Confirmation report A document providing confirmation information. Capacity for Resale A document providing information about capacity for resale. Approved Capacity A document to approve a capacity transfer. Transfer Capacity transfer A document notifying a capacity transfer. notification Transmission rights A document providing the portfolio of the transmission capacity rights of a portfolio market participant. Allocations A document providing the capacity allocations for a border. Bid document A document providing bid information. Allocation result A document providing the allocation results of an auction. document Capacity document A document providing capacity information. Rights document A document providing transmission capacity rights information. Generation availability This document contains information related to energy availability. schedule

Cross border schedule This document contains the cross border schedules for all the borders of a given country where energy is exchanged. Agreed capacity The capacity agreed between parties. Proposed capacity The capacity proposed for agreement between parties. System vertical load The sum of all flows out of the transmission grid via directly connected transformers and lines to distribution grids and end consumers as known by the System Operator. Escalation document A document which requesting the escalation of a situation. Trouble shooting A document providing trouble shooting information for the resolution of a document problem. Deactivation document A document providing deactivation information. Reserve tender document The document that is used for the tendering for reserves within the ERRP process. Reserve Allocation The document used to provide the results of a Reserve auction. Result Document SATCR activation The document is used to provide the activation of reserves through the SATCR process. DATCR activation The document is used to provide the activation of reserves through the DATCR process. Activation response The document is used to provide a response to a request to activate reserves. Tender reduction The document is used to provide information concerning the reduction of a previously submitted tender. MOL Document The document is used to provide Merit Order List information. Price Document The document is used to provide market price information. Measurement Value The document is used to provide measurement information from measurement Document devices. SOAM Document The document is used to provide system operator accounting data for matching.

SOVA Document The document is used to provide system operator validated accounting data. CCVA Document The document is used to provide coordination center validated accounting data. Daily settlement The document is used to provide daily settlement information. document Weekly settlement The document is used to provide weekly settlement information. document Capacity Auction The document is used to provide capacity auction specification information. Specification Document Market Coupling Results The document is used to provide the results of a market coupling auction. Document Outage publication The document is used to provide the outage information for publication. Document Forced generation outage A document providing information on forced generation outages. Document Summarised Market A compilation of all external schedules concerning two Market Balance Areas of Schedule all balance responsible parties. Compensation Program A schedule that provides the schedule information for the compensation of Schedule unintended deviation. Load Frequency Control A schedule that provides the schedule information for the Load Frequency Program Schedule Control Program of a Control Area or a Control Block. Timeframe Independent A compilation of all external Timeframe Independent Schedules concerning two Schedule System Operators. Information request An information request being made concerning some specific information. status request for a A status request concerning the position of an object independently of any position independently ongoing processes. from a specific process Estimated Net Transfer The estimated net transfer capacity for a given border. Capacity Compensation rights The capacity rights that have been allocated as compensation. Redispatch notice A notice to confirm the actions agreed between System Operators to resolve a congestion situation through redispatch.

Tender reduction A response to a tender reduction request that provides corrections to the initial response document. System total load Total load', including losses without power used for energy storage, is equal to generation deducted with exports, added with imports and deducted with power used for energy storage. Final MOL A document providing the information concerning the situation of the MOL at the end of an activation period. Resource Provider A document providing the schedules for production/consumption for resource Schedule for objects submitted by a resource provider. production/consumption Installed generation per A document providing the installed generation per generation type. type Wind and solar forecast A document providing the forecast of wind and solar generation. Load forecast margin A document providing the load forecast margin for a period. Generation forecast A document providing the generation forecast for a period. Reservoir filling A document providing information concerning the filling of reservoirs. information Actual generation A document providing the actual generation for a period. Wind and solar A document providing the generation of wind and solar energy for a period. generation Actual generation per A document providing the actual generation per generation type for a period. type Load unavailability A document providing the unavailability of units providing load on the network. Production A document providing the unavailability of production units providing energy to unavailability the network. Transmission A document providing the unavailability of transmission capacity on the unavailability network. Offshore grid A document providing the unavailability of an offshore grid infrastructure to the infrastructure network. unavailability Generation A document providing the unavailability of generation units providing energy to unavailability the network.

Contracted reserves A document providing the reserves contracted for a period. Accepted offers A document providing the offers of reserves that have been accepted for a period. Activated balancing A document providing the quantities of reserves that have been activated for quantities balancing. Activated balancing A document providing the prices of the reserves that have been activated for prices balancing. Imbalance prices A document providing the prices of reserves due to imbalance for a period. Imbalance volume A document providing the volume of the imbalance for a period. Financial situation A document providing the financial situation for reserves. Cross border balancing A document providing the cross border balancing requirements for a period. Contracted reserve A document providing the price of reserves contracted for a period. prices Interconnection network A document providing information on the expansion of the interconnection expansion network. Counter trade notice A document providing information on counter trades for a period. Congestion costs A document providing the cost of congestion for a period. DC link capacity A document providing the DC links for a period. Non EU allocations A document providing allocations made to non EU member states. Configuration document A document providing configuration information. Redispatch activation A document enabling the activation of a redispatch notice. document Detailed activation A document enabling a detailed history of activations. history document Aggregated activation A document enabling an aggregated history of activations. history document

HVDC Link constraints A document providing the information concerning the maximum and minimum active power flow through each link can limited. HVDC Configuration A document providing the information concerning the power set point. The mode in which the HVDC is managed. HVDC Schedule A document providing the information for operating DC links. EIC code request A document providing the information requesting a new EIC code. EIC code information A document providing EIC information in a central registry exchange or information to an EIC participant. EIC code publication A document providing EIC publication information in a web site publication of a limited set of information. Critical network element A document providing all the elements necessary for the capacity coordinator to determination determine the transfer capacity and the associated critical network elements. Critical network element A document providing all the elements necessary for the market information publication aggregator and TSOs to know the critical network elements which limit the transfer capacity. Flow based domain A document providing the capacity domain and its limits available for the TSO. Flow based domain A document providing the capacity domain and its limits available for the publication market. Flow based domain market A document providing the capacity domain and its impacts on the market to be impact publication published. Anonymized flow based A document providing all the relevant flow based parameters in case of flow parameters publication based capacity allocation. Critical network element A document providing the critical network elements and its impacts on the market impact market to be published. publication Weather document An estimation or prediction in advance of the weather by analysis of meteorological data and the results of what actually happened with the weather. Energy prognosis A document to provide the prognosis of energy production/load for a given document period. Network constraint A document providing the network constraint situations used for the load flow document

studies. A network constraint situation includes contingencies, monitored elements and remedial actions. Aggregated netted A document used to report aggregated netted external market schedules for a external market schedule given border. document Aggregated netted A document used to report aggregated netted external TSO schedules for a external TSO schedule given border. document Reporting status market A document used to report the status of aggregated netted external market document schedules, aggregated netted external TSO schedules and compensation program schedules on a given border. Reporting information A document used to report the information concerning aggregated netted market document external schedules, aggregated netted external market schedules, aggregated netted external TSO schedules, compensation program schedules, netted area position schedules and netted area AC position schedules to an interested party. Status request for a A document requesting the provision of a reporting information document. reporting information market document Reserve need document Used by a TSO to send its reserve needs. Generation and load A document providing the values of the generation and load shift keys to be shift keys document used on network model. Offers to be activated A document containing the outcome of the process, with the list of offers that are to be activated by the TSO concerned and the results for its reserve needs. Clearing price A document containing the outcome of the process, with the clearing prices for a period. Security analysis report A document providing a report on a performed security analysis. Aggregated netted A document used to report aggregated netted external schedules for a given external schedule border. document External TSO schedule A document used to report external TSO schedules for a given border or interconnector. Move of scheduled A document indication a move of scheduled production. production PS&LC results document A document providing Pole Splitting & Loss Calculation results.

Notification data market A document used to notify data to any information receiver. document Additional Constraint A document describing additional constraints for a capacity calculation process. document Operational state A document used for exchanging operational states for grid assets. document Published offered A document providing the most recent values of offered capacity. capacity Market result document Published prices and volumes Area Configuration A document containing the definition of areas. document Area Composition A document containing the relations between areas, i.e which Metering Grid document Areas a Bidding Zone composed of. Connected Areas document A document containing which other areas an area is connected to i.e. which Metering Grid Areas a Metering Grid Area is connected to. Settlement document A document providing settlement information. Imbalance prognosis A document to provide the prognosis of energy imbalances for a given period. document Complete set of bids Submission of complete set of bids. If there are existing bids, they should be replaced. Merged MOL notice A notice providing information on the MOL merging process. K-factor document A document providing K-factor values. Settlement coordination A document providing settlement information for coordination between different document parties. Financial settlement A document providing financial settlement information. document Bid availability A document providing the detailed reasons for changing the availability or document volume of a bid. Resource capacity unit A document providing information about resource capacity units. document

Other market information A document providing other market information. Permission document A document that contains detailed information about permissions. Requested capacity A document providing requested capacity (e.g. ATC, CZCL) value. Voltage control A document providing the number of generating units to activate for voltage activation document control, by means of their operation in synchronous condenser mode. Transmission inventory A document providing a transmission inventory. document Transmission inventory A document providing statistics on the transmission grid.

QualityTypeList ESMPEnumerations The quality of an object. Adjusted The contents of the object have been adjusted. Not available The contents of the object are not available. Estimated The contents of the object are estimated. The code is typically used when measured values are missing and an estimate is made based on historical data. As provided The contents of the object are as provided. Incomplete The contents of the object are calculated based on incomplete data. Calculated The contents of the object are calculated. The code is typically used when a value is calculated based on several other known values. Temporary The content of the object is temporary. The code is typically used when there is communication trouble to a meter and a more correct value is expected later.

ReasonCodeTypeList ESMPEnumerations The coded motivation of an act. Errors not specifically This code is used to identify errors that have not been specifically addressed in identified the Reason code list. It can be used at any level and refers to the level for which it has been identified.

Message fully accepted The message has been fully accepted for application processing. Message fully rejected No part of the message has been accepted for application processing, e.g. Global position incomplete. Message contains errors Part of the message contents, i.e. certain time series, has been accepted for at the time series level application processing. It is necessary to look at the time series level to determine the time series that have been rejected. The time series is excluded from the global position. Time interval incorrect The schedule time interval is not within the contractual agreement or the period does not agree with the schedule time interval. Sender without valid The sender has no current valid contract with the TSO. The message contract consequently will be fully rejected. Schedule accepted The schedule of the recipient as presented has been completely accepted. Schedule partially The schedule of the recipient as presented has been partially accepted. It is accepted necessary to look at the time series level to determine the changes (time series rejected, modified, etc.). Schedule rejected The schedule of the recipient as presented has been totally rejected. The cause could be the non presentation of a counter part for the involved trades. Time series not matching Time series mismatches. Credit limit exceeded The contractual credit limit has been exceeded and as such the message has been rejected. Time series fully The time series has been fully rejected. In the case of a confirmation report, this rejected reason code is used in conjunction with either A26 or A30. Time series accepted The time series has been accepted but some time interval quantities have been with specific time rectified or zeroed out. interval errors In party/Out party There is no contract for the parties indicated or the rules for cross border invalid nominations are not being respected. The time series has been rejected. Area invalid The area is unknown or not allowed. The time series has been rejected. A24 not applicable This code is no longer applicable. A25 not applicable This code is no longer applicable.

Default time series The time series has been rejected and replaced with a default time series applied profile. This reason code may not be used in conjunction with A30. Cross border capacity The cross border capacity has been exceeded. The time series has been exceeded rejected or adjusted. Counterpart time series This provides an indication that the time series has not got a counterpart time missing series. In the case of an Intermediate Confirmation Report this is advising the recipient that the time series may be rejected at nomination closure if the counterpart time series is not received. In the case of a Final Confirmation Report this is informing the recipient that the time series has been rejected because the counterpart time series has not been forthcoming. Counterpart time series The time series has been rejected as it does not match that of the counterpart quantity differences who is considered by market rules to be correct. Imposed Time series from The nominated party's time series has replaced the current time series. This nominated party's time reason code may not be used in conjunction with A26. series (party identified in reason text) Resolution inconsistency The resolution is not coherent with the time interval, or resolution not valid. Quantity inconsistency The quantity is not coherent. For example a time period with the same version number but different quantities or an non permitted number of digits after the decimal point, etc. Quantity increased The quantity has been increased in order to satisfy minimum constraints. Quantity decreased The quantity has been decreased in order to satisfy congestion constraints. Default quantity applied The default quantity has been applied as the current quantity does not satisfy contractual obligations. Quantities must not be The quantity proposed is illegal since signed values are only permitted in signed values specific circumstances. A47 not applicable This code is no longer applicable. Modification reason In an intraday transmission, the reason for the modification is as follows (in the reason text). Position inconsistency A position is missing or too many. Senders time series There is an error in the sender time series version, i.e. it could be superior to

version conflict the message version or is inconsistent with the existing data. The time series has been rejected. Message identification The message identification is already in the receiving system. Or a higher or version conflict version already exists. Message rejected. Time series missing from A time series is not contained in a new version of the message. Message new version of message rejected. Receiving party The receiving party is incorrect. Message rejected. incorrect Global position not in The message does not balance out to zero. Market rules might require that the balance message is rejected. Time series The identification of the time series is duplicated or incorrect. Time series will identification conflict be rejected. Corresponding Time All corresponding time series must be netted. Time series rejected. series not netted Deadline limit The deadline for receiving schedule messages has passed. Message or time exceeded/Gate not open series rejected. One to one nomination There is a one to one nomination inconstancy with the in/out parties or areas. inconsistency Time series rejected. Not compliant to local The level in which this is identified is not in compliance with local market rules. market rules The level in question has been rejected. Inter-area transit The inter-area transit schedule exceeds the nominated schedule for the same schedule exceeds time interval. The inter-area transit schedule is rejected. nominated schedule Currency invalid The currency is not in compliance with ISO 4217. Invalid business type The business type does not belong to the valid set of business types for the process in question. Time Series modified The time series has been modified. Resource Object Invalid The Resource Object defined in the document is not valid. Reserve object Technical Reserve objects aggregated values are not within technical/prequalified limits limits exceeded Planned reserves do not Planned reserves do not correspond with contractual data.

correspond with contractual data Limit Data is not Limit Data is not available. available Reserve Object not Reserve Object is not qualified for reserve type. qualified for reserve type Mandatory attributes Mandatory attributes missing. missing Curtailment The capacity in question has been curtailed. Linked bid rejected due The bid in question has been rejected because an associated bid has been to associated bid unsuccessful. unsuccessful Original bid divided to The original bid quantity has been divided to enable it to be accepted. permit acceptance Bid accepted The bid in question has been accepted. Auction Status The information in the Reason Text provides auction status information. Right status information The information in the Reason Text provides status information concerning the transmission rights in question. Agreement identification There is an inconsistency between the contract type and the agreement inconsistency identification. Dependency matrix not There is an inconsistency between the document contents and the dependency respected matrix. Sender identification The identification of the sender or the sender/role combination is invalid. and/or role invalid Process type invalid The process type does not figure in the list of valid process types for this document. Domain invalid The domain does not figure in the list of valid domains for this document and process. Matching period invalid The period is not within the expected limits. In/Out area inconsistent The in and out area does not figure within the domain specified. with domain

Disagree with matching The matching results provided are not consistent. results Confirmation ignored due The report has been ignored since a higher version has been received. to higher version already received Confirmation without The report has been successfully matched without any changes. adjustment (time series have been matched without change) Confirmation with The report has been matched but required adjustment. adjustment (time series have been modified) For action (only in The report in question is only for action in an intermediate stage. intermediate confirmation - time series need mutual agreement and action) Time series matched The time series has been successfully matched. Time series ignored The time series has been ignored and not matched since it does not figure in a (note: this can only counterparty transmission. All are correctly equal to zero. apply to time series that are set to zero - see matching principles) Modification proposal The document is a proposal for change before finalization. (intermediate confirmation) Expected document not The document that is expected has not been received within the expected received timeframe. Not possible to send The document that is due cannot be transmitted within the required timeframe. document on time, but An estimated time of transmission is provided. estimated delivery time is provided Not possible to send The document that is due cannot be transmitted within the required timeframe. document on time, and The time of transmission of the document is unknown. furthermore no expected time of return to normal situation Document cannot be The receiving system cannot process that document in question. processed by receiving system Complementary Additional text is provided in order to further explain a condition, for example to information provide details of an outage. Technical constraint

A technical constraint has been applied. Force majeure Curtailment due to Force Majeure. A code that enables the identification of the curtailment curtailment reason for settlement purposes. Network security Curtailment due to network security reasons A code that enables the curtailment identification of the curtailment reason for settlement purposes. Auction cancelled The auction has been cancelled. Incomplete document The document is incomplete and cannot be processed. Accounting Point (tie- The document is incomplete as a time series for an accounting point is missing. line) Time Series missing Meter data Time series The document is incomplete as a time series for meter data is missing. missing Estimated values not The document is in its initial form and estimated values are not allowed. allowed in first transmission No quantity values No quantity values are allowed for a quality that is not available. allowed for a quality that is not available Time series accepted Time series accepted. Auction without bids The auction has terminated without any bids being submitted. The ReasonText being entered may provide the identification of the auction in question. Data not yet available It is not possible to perform the necessary action since the required data for this action is not yet available. Bid not accepted The bid in question has not been accepted. Initiator area problem The problem concerns the initiator area. Cooperating area problem The problem concerns the cooperating area. Communication status The status within the system indicates that the communication capability is currently active currently active. Communication status The status within the system indicates that the communication capability is currently inactive currently inactive.

Communication status The status within the system indicates that the communication capability is currently restricted currently restricted. Problem associated with The problem concerns both areas. both areas Tender unavailable in A tender that has been requested is no longer available in the MOL. MOL list Price based on The price is based on a preliminary exchange rate. preliminary exchange rate Failure A failure has occurred. Foreseen maintenance Maintenance is foreseen. Shutdown A shutdown has occurred. Official exchange rate The official exchange rate has been approved. approved System regulation The information provided regards a regulation for system purposes. Frequency regulation The information provided regards a regulation for frequency purposes. Load flow overload Situation in the grid, where loading of a certain grid element, e.g. overhead line, is above defined technical limits. Voltage level adjustment A TSO activity to maintain an acceptable voltage profile throughout the network. This is achieved by balancing of the respective reactive power requirements of the network and the customers. Emergency situation Curtailment due to emergency situation. A code that enables the identification curtailment of the curtailment reason for settlement purposes. Calculation process The calculation has not been performed. failed No capacity constraint The market position is such as no capacity constraint is applied to limit the impact on the market cross border exchanges. Special Condition Special condition need to be fulfilled. Unverified Missing or not validated data. Verified Data has successfully passed the verification process.

CGM inconsistency Describes an element which was not found in the CGM. Network dictionary Describes an element which was not found in the network dictionary. inconsistency Capacity reduced by TSO Describes a capacity that was reduced by a TSO. Overload Describes an N-k or N state overload. GLSK limitation Describes a situation in which there is not enough power adjustment in the GLSK file to cover the capacity. Voltage constraint Describes an N-k or N state voltage violation. Angle constraint Describes an N-k or N state angle violation. Stability Describes a situation in which the dynamic behaviour of the network violated. Loadflow divergence Describes a network situation in which the provided capacity values are part of a load flow divergence situation. Exclusion for SoS This is the adjustment applied to the capacity of a branch to have a minimum reasons RAM (Remaining Available Margin) available for commercial exchanges. Constraint by the market A constraint due to market restrictions. Ordinary The contingency is ordinary (methodology for coordinating operational security analysis, article 6). Exceptional The contingency is exceptional (methodology for coordinating operational security analysis, article 6). Out of range The contingency is out of range (methodology for coordinating operational security analysis, article 6). Internal congestion A temporary congestion within a bidding zone or scheduling area. Operational security Operational security constraints identified by TSOs. constraints Estimated value Describes a situation where a calculation process has failed and extrapolated or interpolated values will be applied.

Balancing Activated for balancing purposes. Values shared Values of this time series are also valid for counter-party. Outside price limits The offered price is not within the valid limits. Previous timeframe data In case of processing issue, sent data are based on the previous timeframe. MOL merging succesful The merging of the Merit Order List has been successfully processed. MOL merging failed The merging of the Merit Order List has failed. Because of redispatching Because of redispatching according to Article 2(26) of Commission Regulation (EU) 543/2013 Because of Because of countertrading according to Article 2(13) of Commission Regulation countertrading (EU) 543/2013 Because of other Not available because of any remedial action. remedial action Insufficiency of The reserve capacity is not sufficient. reserves Unavailability of The unit providing the reserve is subject to technical unavailability. reserve providing unit Unavailability of Unavailability of tools to detect predetermined system conditions that have a automatic protection high probability of causing unusual stress on the power system, for which pre- systems planned remedial action is considered necessary or for which automatic protective actions may be triggered such as special protection schemes or automatic load shedding. Physical cable or Limitation due to physical cable or converter restrictions. converter restrictions Constraints in Limitation due to constraints in controller systems. controller systems Adjusted because of The capacity is adjusted because of an expected violation of operational expected violation of security limits of physical transmission assets. operational security Adjusted because already The capacity is adjusted because the remedial actions were assessed as not considered remedial sufficient or ineffective to avoid the expected violation of operational security actions are assessed as limit(s). not sufficient Time series partially The time series is partially accepted.

accepted Demand fully netted Demand was fully netted against other demand(s). Bid activated in same One or several bids were activated in the same direction within the same direction uncongested area as the demand. Optimization in progress The activation optimisation framework is being executed. Wrong energy market The given energy market timeframe is invalid or wrong. timeframe Message partially The detailed transactions of the received document are partially accepted. It is accepted necessary to look at the detailed (transaction) level to determine if the transaction is accepted, rejected etc. (Explicit) Permission Based on the regulation (EU) 2016/679 (GDPR) Article 6, Lawfulness of from the customer processing: The data subject has given consent to the processing of his or her personal data for one or more specific purposes. In execution of a Based on the regulation (EU) 2016/679 (GDPR) Article 6, Lawfulness of contract with the processing: Processing is necessary for the performance of a contract to which customer (Execution of the data subject is party or in order to take steps at the request of the data contract) subject prior to entering into a contract. Legal obligations Based on the regulation (EU) 2016/679 (GDPR) Article 6, Lawfulness of processing: Processing is necessary for compliance with a legal obligation to which the controller is subject. In protection of vital Based on the regulation (EU) 2016/679 (GDPR) Article 6, Lawfulness of interests of the processing: Processing is necessary in order to protect the vital interests of the customer data subject or of another natural person. In order to perform a Based on the regulation (EU) 2016/679 (GDPR) Article 6, Lawfulness of duty of common processing: Processing is necessary for the performance of a task carried out (national) interest in the public interest or in the exercise of official authority vested in the controller. Justified interest of Based on the regulation (EU) 2016/679 (GDPR) Article 6, Lawfulness of the dataprocessee processing: Processing is necessary for the purposes of the legitimate interests pursued by the controller or by a third party, except where such interests are overridden by the interests or fundamental rights and freedoms of the data subject which require protection of personal data, in particular where the data subject is a child Fulfilment of purpose The purpose of the object or action has been fulfilled. Reach of end timestamp The object or action has reached its end timestamp.

Revocation Revocation of the action or object. Termination Termination of the action or object. No valid grid access There is no valid grid access contract for a specific metering point. There is no contract valid grid access contract for a specific metering point. Customer move-out A customer has moved out.

BusinessTypeList ESMPEnumerations The exact business nature identifying the principal characteristic of a time series. Production The nature of the business being described is production details. Internal trade The nature of the business being described is internal trade details. External trade explicit The nature of the business being described is external trade details between capacity two areas with limited capacity requiring a capacity agreement identification. Consumption The nature of the business being described is consumption details. External trade total The nature of the business being described is external trade total. External trade without The nature of the business being described is external trade details between explicit capacity two areas without requiring capacity allocation information. Net Production / Net production/consumption - where signed values will be used. Consumption With the following rules: In area=Out area, In party=Out party, + means production and - means consumption. Net internal trade Net internal trade - where the direction from out party (seller) to in party (buyer) is positive and the opposite direction is negative (with minus signs). IPP (Independent Power A time series concerning the production schedule from an IPP. Producer) Tertiary control A time series concerning tertiary reserve. Primary control A time series concerning primary reserve. Secondary control A time series concerning secondary reserve.

Load profile A time series concerning a load profile as calculated by a metered data aggregator. Aggregated energy data A time series concerning adjusted metered readings received from a metered data collector and aggregated and validated by a metered data aggregator. Losses A time series concerning losses that have been calculated for a tieline or an area. Transits (CBT) A time series concerning inter area transit flows determined for CBT requirements. Settlement deviation A time series concerning the imbalance energy calculated by an imbalance settlement responsible. Technical constraint A time series defining the imbalance between schedules accepted by the deviation system operator due to technical constraints and schedules declared by the balance responsible party. Balance energy deviation A time series defining the imbalance between the schedule of a balance responsible party that has been corrected by the system operator after using balance energy bids and the schedule that was accepted by the system operator due to technical constraints. Imbalance volume A time series defining the imbalance between the actual meter readings and the schedule of the balance responsible party corrected by the system operator after using balance energy bids. Unintended energy A timeseries concerning the volume of an unintended cross-border exchange of energy. Frequency control A time series concerning primary and secondary reserve. Balance management A time series concerning energy balancing services. Total trade A time series concerning the total of both the internal and external trades. General Capacity A time series providing the total capacity available on a TSO border. Information Available transfer Available transfer capacity for cross-border exchanges. capacity (ATC) Net transfer capacity Net transfer capacity for cross-border exchanges. (NTC) Control Area Program A time series providing the total exchanges between two TSOs (including the

commercial transactions, the compensation program and the losses compensation program). Note this definition might change when UCTE brings forward its coding requirements. Already allocated The already allocated capacity is the total amount of allocated transmission capacity (AAC) rights. Internal inter area A trade that occurs between internal areas within a market balance area. trade Offered Capacity The time series provides the offered capacity. Capacity transfer The time series provides information concerning the notification of the transfer notification of capacity to another market participant. Authorised AAC The time series in question provides the amount of transmission capacity rights to be nominated. Capacity rights The time series in question provides the capacity rights allocated for a given border. Minimum authorised AAC The time series in question provides the minimum amount of transmission capacity rights to be nominated. Maximum authorised AAC The time series in question provides the maximum amount of transmission capacity rights to be nominated. Installed generation The time series in question provides the installed generation. Available generation The time series in question provides the available generation. Interconnection Trade The Time series in question provides the designation of the ITR that may Responsible Designation nominate the capacity in question. Released AAC The already allocated capacity (AAC) that has been released for resale. Requested capacity (with The time series in question provides information concerning the requested price) capacity including price information. Requested capacity The time series in question provides information concerning the requested (without price) capacity but excludes price information. Compensation program Compensation of unintentional deviation is performed by exporting to / importing from the interconnected system during the compensation period by means of schedules as calculated during the accounting of unintentional deviations.

Schedule activated The cross border or internal reserves that are to be activated through schedule reserves nomination. System Operator The cross border redispatching between System Operators that are to be redispatching activated through schedule nomination. Market capacity price The price of the capacity offered on a given market. Market capacity price The difference between the price of capacity in a Market Balance Area differential receiving the capacity (In Area) and the price of capacity in a Market Balance Area providing the capacity (Out Area), i.e. In Area Price - Out Area price. Inflow The volume of water that flows into a reservoir in a given interval. Water extraction The volume of water that can be extracted from a reservoir in a given interval. Turbined water The volume of water that can be turbined in a plant in a given interval. Water spillage The volume of water that is not turbined going through the spillway in a given interval. Planned maintenance Maintenance has been planned for the object in question with a forecast ending date. Unplanned outage An unplanned outage has occurred on the object in question. Use it Or Sell it The time series provides information on the capacity resold in the "use it or sell (UIOSI) pricing it" process and its corresponding price. Compensation for auction The time series provides information on the compensation of the capacity for cancellation where resale following an auction cancellation. capacity is for resale Resale pricing For each Physical Transmission Rights holder, this document contains the resold capacity and its corresponding price. Curtailed capacity The time series provides information to compensate a party when curtailment is compensation applied on the capacity obtained in a previous auction, resale or transfer. Use it Or Sell it The time series provides information on the compensation for the capacity (UIOSI) compensation following an auction cancellation. Minimum possible The time series provides a schedule of minimum possible values for a Resource Object. The nature of the flow could be defined by the attribute Direction.

Maximum available The time series provides a schedule of maximum available values for a Resource Object. The nature of the flow could be defined by the attribute Direction. Spot price The time series provides the market spot prices from an auction. Minimum ATC The Available Transmission Capacity that must be guaranteed because of regulatory constraints. Meter Measurement data The data as provided for a meter measurement source. Accounting Point The metered data that is to be considered relevant for accounting purposes. Relevant data Energy flow Energy flow information. Power plant energy Energy flow scheduled for a power plant. Schedule Compensation The time series provides the compensation requirements for a given Requirements for the compensation period. compensation period Market coupling results The time series provides the results of a market coupling auction. Production, unavailable Production capacity that normally would be available, but due to maintenance or similar is temporarily unavailable. Supplementary available The supplementary generation that is available. generation Interruptible The consumption that may be interrupted on request. consumption Summarised Market A time series providing the total exchanges based on commercial transactions Balance Area Schedule between two Market Balance Areas. Load Frequency Control A time series providing the schedule information for the Load Frequency Program Schedule Control Program. Timeframe Independent A time series providing the total exchanges of Timeframe Independent Schedule Schedules between two System Operators. Consumption curtailment A time series providing the amount of voluntary consumption curtailed by the energy supplier of an end-consumer.

Production, dispatchable The nature of the business being described is dispatchable production details, i.e. generation output that can be changed by a request (activation order) of the TSO according with the applicable Market Rules. Consumption, The nature of the business being described is dispatchable consumption dispatchable details, i.e. consumption output that can be changed by a request (activation order) of the TSO according with the applicable Market Rules. Production, non- The nature of the business being described is non-dispatchable production dispatchable details, i.e. generation output that cannot be modified by an activation order. Consumption, non- The nature of the business being described is non-dispatchable consumption dispatchable details, i.e. consumption output that cannot be modified by an activation order. Total Transfer Capacity The Total Transfer Capacity is the maximum exchange program between two (TTC) areas compatible with operational security standards applicable at each system if future network conditions, generation and load patterns were perfectly known in advance. Mutual Emergency The cross border Mutual Emergency Assistance Service between System Assistance Service Operators that are to be activated through schedule nomination. (MEAS) Auction cancelation The time series covers auction cancellation right. Nomination curtailment The time series covers nomination curtailment rights Internal redispatch Redispatch to relieve Market Balance Area internal congestion. Control area balance A sum of secondary, tertiary control as well as other energy that was used to energy balance a control area. Balancing energy price Price of energy used to balance. Economised secondary The activated secondary reserve that had been economised due to pooled reserve reserve management. Spinning reserve The extra generating capacity that is available by increasing the production of generators that are already connected to the power system. Solar The business being described concerns solar power. positive forecast margin The business being described concerns a positive forecast margin. Negative forecast margin The business being described concerns a negative forecast margin.

Wind generation The business being described concerns wind generation. Solar generation The business being described concerns solar generation. Frequency containment The business being described concerns frequency containment reserve. reserve Automatic frequency The business being described concerns automatic frequency restoration restoration reserve reserve. Manual frequency The business being described concerns manual frequency restoration reserve. restoration reserve Replacement reserve The business being described concerns replacement reserve. Financial information The business being described concerns financial information. Interconnector network The business being described concerns interconnector network evolution. evolution Interconnector network The business being described concerns interconnector network dismantling. dismantling Counter trade The business being described concerns counter trades. Congestion costs The business being described concerns congestion costs. Capacity allocated The business being described concerns capacity allocation and includes price (including price) information. DC link constraint The business being described concerns DC link constraints. Auction revenue The business being described concerns auction revenue. Total nominated capacity The business being described concerns the total nominated capacity. Net position The business being described concerns net position. Congestion income The business being described concerns congestion income. Production unit The business being described concerns a production unit. Rounded market coupling Rounded outputs of the market coupling to be sent to TSOs and Market results Participants.

Allocation Revenue The time series provides information on the revenue generated by the allocations. Production deviation A time series concerning the imbalance energy between the metered production and the schedules calculated by an imbalance settlement responsible. Consumption deviation A time series concerning the imbalance energy between metered consumption and the forecasted consumption calculated by an imbalance settlement responsible. Transmission asset The business being described concerns a transmission asset. Consumption unit The business being described concerns a consumption unit. In-feed ATC Available Transfer Capacity at the in-feed side of a DC tieline. Out-feed ATC Available Transfer Capacity at the out-feed side of a DC tieline. Balance up regulation A time series concerning balance regulation market prices for up regulation. price Balance down regulation A time series concerning balance regulation market prices for down regulation. price Main direction A time series concerning the direction of balance regulations. Consumption imbalance A time series concerning imbalance prices for consumption. price Production sales A time series concerning imbalance prices for production sales. imbalance price Production purchase A time series concerning imbalance prices for production purchase. imbalance price Average balance price A time series concerning the average prices between Market Balance Areas. between MBAs Pumped A time series concerning the electricity consumption related to pumping. Large installation A time series concerning consumption from large installation. consumption Metering Grid Area (MGA) A time series concerning imbalance between reported consumption, production imbalance and exchange in a Metering Grid Area.

HVDC Link settings The time series in question provides HVDC Link settings. Transmission Reliability A time series concerning Transmission Reliability Margin (TRM). Margin (TRM) Imbalance component for This information is used to provide to a pool manager the combined imbalance a pool of all the pool participants. Area Control error (ACE) The sum of the instantaneous difference between the actual and the set-point value of the measured total power value and Control Program including Virtual Tie-Lines for the power interchange of a LFC Area or a LFC Block and the frequency bias given by the product of the K-Factor of the LFC Area or the LFC Block and the Frequency Deviation. Area Control Error after A time series concerning the Area Control Error after applying the imbalance Imbalance Netting netting energy correction. Implicit and explicit The sum of cross border schedules based on implicit and explicit trades trade total including long term, yearly, monthly, weekly, daily processes. Production units own The consumption of one or more production units. consumption Constraint situation The timeseries describes the constraint situation for a given TimeInterval. A constraint situation can be: - composed of a list of network elements in outage associated for each outage to a list of network elements on which remedial actions have been carried out accordingly to contingency process - or it can be an external constraint. Initial domain The timeseries describe the full flow based domain for a given TimeInterval. Critical network elements are displayed in details and their impact on the market is quantified. Flow based domain The timeseries describe the full flow based domain for a given TimeInterval adjusted to long term adjusted to the latest update of the schedules. schedules Critical network elements are displayed in details and their impact on the market is quantified. Network element The timeSeries describes limiting elements which are overloaded. constraint Calculation opposition The timeSeries describes a party who is opposed to the calculation result and (Red Flag) imposes its transfer capacity value. Base case proportional The GSK or LSK are proportional to the base case generation or load. shift key Proportional to The GSK or LSK are proportional to the participation factors. participation factors

shift key Proportional to the The GSK is proportional to the remaining available capacity. remaining capacity shift key Merit order shift key The GSK is proportional to a merit order list. Wind speed The TimeSeries provides information on the wind speed. Wind direction The TimeSeries provides information on the wind direction. Solar irradiance The TimeSeries provides information on the power per unit area produced by the sun in the form of electromagnetic radiation. Air temperature The TimeSeries provides information on the air temperature. Cloudiness The TimeSeries provides information on the cloudiness, i.e. the level of coverage of the sky with clouds. Air humidity The TimeSeries provides information on the level of humidity of the air. Atmospheric pressure The TimeSeries provides information on the atmospheric pressure. Precipitation The TimeSeries provides information on the amount of rain, snow, etc. that falls on the ground. Network constraint The TimeSeries describes the network elements, that constraints the market, to situation that be taken into account to simulate a network constraint during the network load constraints the market flow studies. The network situation includes the contingencies, the remedial actions, the monitored network elements and the potential additional constraints. Contingency The TimeSeries describes the network elements part of the contingency to be simulated for a given TimeInterval. Remedial Action The TimeSeries describes a set of remedial actions for a given TimeInterval. Monitored Network The TimeSeries describes the network elements to be monitored during the Element network load flow studies. Busbar The TimeSeries describes the network elements that composed a busbar. Network Element The TimeSeries describes network elements.

SPS The TimeSeries describes the network elements managed by a Special Protection System (automation). Aggregated netted The aggregated netted external market schedules for a given border. external market schedule Aggregated netted The aggregated netted external TSO schedules for a given border. external TSO schedule Aggregated netted The aggregated netted external schedules for a given border. external schedule Netted area AC position The AC position for a given area. Netted area position The netted aggregation of all AC external schedules of an area plus the aggregated External Netted Schedules of related HVDC links of an area. Interconnection shift The shift key series describes the amount of power to be shifted from a border key area. DC flow with losses DC flow with losses refers to the values at the importing end of the DC line. DC flow without losses DC flow without losses refers to the values at the exporting end of the DC line. minimum value of netted That value which a netted area position must not fall below for a given area. area position maximum value of netted That value which a netted area position must not exceed for a given area position optimisation area. maximum value of DC flow That value which a balanced DC flow must not exceed for a given DC line on exporting end. When aligning DC flows CGMA algorithm will respect this constraint. minimum value of DC flow That value which a balanced DC flow must not fall below for a given DC line on exporting end. Currently this business type is only included for consistency reasons. It is always set to 0. This constraint might, however, be used in future. When aligning DC flows the CGMA algorithm will respect this constraint. indicative AC flow It is the hypothetical flow on the aggregate of all AC tie lines of an electrical border between two optimisation areas. It results from the adjustments to the preliminary netted area positions of all optimisation areas made by the CGMA algorithm. Indicative AC flows are an artefact of the CGMA algorithm, and do not correspond to physical flows Offer The time series provides an offer to provide reserves. Need The time series provides a requirement for reserves.

Opportunity costs or The time series describes any opportunity costs or benefits. benefits Financial compensation The time series describes any financial compensation or penalties or penalties Global radiation The total short-wave radiation from the Global radiation is the total short-wave radiation from the sky falling onto a horizontal surface on the ground. It includes both the direct solar radiation and the diffuse radiation resulting from reflected or scattered sunlight. Diffuse radiation Radiation resulting from reflected or scattered sunlight. Direct solar radiation Radiation resulting from direct sunlight Outage (OUT) Outage process: Element is out of operation due to planned maintenance or due to an unplanned/forced outage. Outage may be used as a synonym for unavailability. Special switching state Outage Process: This state applies to grid elements which are in operation in (SSS) an exceptional state (e.g. separated nodes operation). Testing (TEST) Outage process: TESTING means any element status is possible - ON or OUT. This status applies either between first connection and final commissioning of the relevant asset, or directly following maintenance of the relevant asset. Auxiliary busbar Outage process: Element is in operation but connected via auxiliary busbar operation Automatic reclosing Outage process: Protection function Automatic reclosing is switched off for electric line Busbar protection Protection function busbar protection is switched off Phase Shift Angle The maximum phase shift angle allowed between two network elements. Base Case Network The TimeSeries describes the network elements to be taken into account to Situation simulate a base case network situation during the network load flow studies, without any contingency. Inter-TSO assistance Cross border assistance schedule between TSOs not interconnected directly. FlexibleNeed The business type indicates that the need is optional. GLSK Limitation A constraint related to a GLSK maximum or minimum limitation in the production or/and consumption shift.

Capacity ramping A constraint related to a ramping limitation on the capacity offered at a given limitation border. interconnector capacity The maximum capacity that can be exchanged on an interconnector, excluding external factor on both ends. Must Run A time series concerning must run generation. Procured capacity An accepted offer of balancing capacity. Used capacity The used cross-zonal balancing capacity. Estimated costs Estimated costs of the process. Estimated benefits Estimated benefits of the process. Load Shedding A time series concerning a load shedding used to avoid failure of the power system. Remaining Capacity A time series concerning the remaining capacity. Indicator of generation Indicator of adequacy, it indicates if there is final generation remaining capacity capacity adequacy after SMTA calculation. Income from price The time series describes income due to price divergence without congestion divergence without between bidding zones. congestions Push-button The cross-border Push-button service between System Operators. Intertripping The cross-border Intertripping service between System Operators. Emergency instruction The cross-border Emergency instruction service between System Operators. Ramp management The schedule resulting from cross-border Ramp management service between System Operators. Profile smoothing The schedule resulting from cross-border Profile smoothing service between System Operators. Emergency reallocation The schedule resulting from cross-border Emergency reallocation deselection deselection service between System Operators.

SO-SO-trade The generic cross border trade between System Operators. Production reduction A time series providing the volume of production reduced by an energy provider / producer / supplier. Maximum power exchange The timeseries provides the maximum admissible power flow between two bidding zones respecting operational security limits taking into account N-1 criterion. Maximum power exchange The timeseries provides the maximum admissible power flow between two after remedial actions bidding zones after remedial actions. Network constraint The TimeSeries describes the network elements, that cannot limit the market, situation that cannot to be taken into account to simulate a network constraint during the network limit the market load flow studies. The network situation includes the contingencies, the remedial actions, the monitored network elements and the potential additional constraints. Flat participation for Flat GSK factors of all generators or loads, independently of the size. all generators or loads Proportional to Generators participate relative to their maximum (installed) capacity (MW). installed capacity of generators Market price and total A time series concerning market price and total volume. volume Import price A time series concerning import price (the volume-weighted price average of all accepted bids). Capacity allocated The business being described concerns capacity allocation and excludes price (excluding price) information. Common Grid Model The timeseries provides equipment related to the Common Grid Model (CGM). Equipment Exchanged balancing The balancing reserve capacity exchanged between areas. reserve capacity Shared balancing reserve The balancing reserve capacity shared between areas. capacity Share of reserve A time series concerning the share of reserve capacity. capacity Actual reserve capacity A timeseries concerning actual reserve capacity. K-factor K-factor as stated in the SO GL Art. 2 (45). It is also known as Frequency Bias.

Frequency Containment FCR-N is a reserve that is automatically activated in both directions around a Reserve-Normal (FCR-N) set point when the frequency varies between 50.10 Hz and 49.90 Hz after an imbalance. Frequency Containment FCR-D is a reserve that is automatically activated when the frequency falls Reserve-Disturbance below 49.90 Hz or rises above 50.1 Hz after an imbalance. (FCR-D) Internal trade A time series concerning internal trade difference, within an area, such as a difference Bidding Zone or Scheduling Area. The internal trade difference is the difference between trades reported from an out party (seller) and an in party (buyer). Small scale production Production from small scale production plants. System price The system price is an unconstrained market clearing reference price. It is calculated without any congestion restrictions by setting capacities to infinity. Wind gust An increase in the speed of the wind lasting for a short period. Area imbalance A time series concerning imbalance between planned consumption, production and exchange in an Area. Unintended energy price A timeseries concerning the price of the unintended cross-border exchange of energy. Frequency containment A timeseries containing the volume of energy resulting from the frequency process energy containment process. Frequency containment A timeseries containing the energy price from the frequency containment process energy price process. Ramping period energy A timeseries containing the volume of energy exchanged as a result of ramping between different ANES values. Ramping period energy A timeseries containing the price of the energy exchanged as a result of price ramping between different ANES values. Frequency deviation A timeseries concerning the difference between the actual and the nominal frequency of a synchronous area. Day-Ahead market price A timeseries concerning Day-Ahead market prices. Conditional bid Standard product bid that is conditional on bids submitted outside of common platform. Thermal limit The current causing a given network element to work outside of the range of

safe operating temperatures. Frequency Limit A constraint related to the containment of frequency deviations within a given area. Voltage limit The maximum or minimum permissible voltage within normal operation state of a given network element. Current limit The maximum permissible current within normal operation state of a given network element. Short circuit current The maximum permissible short-circuit current within normal operation state of limit a given network element. Dynamic stability limit A maximum permissible load ensuring the control of oscillations in the grid and avoiding the loss of synchronism. Disconnection A timeseries describing disconnection of a TSO from a common platform. Intended energy with A timeseries concerning the amount of intended energy with prices higher than positive price zero (and including zero). Intended energy with A timeseries concerning the amount of intended energy with prices lower than negative price zero (excluding zero). Decopuling A time series describing decoupling of an area. Resource capacity unit A timeseries containing information about resource capacity units. Resource entry capacity A timeseries containing the resource capacity that can be allocated to an data eligible resource capacity operator from another area. Resource capacity A timeseries containing the resource capacity operator obligation to guarantee obligation data delivery. Available energy A timeseries concerning the available energy. Production curtailment A timeseries concerning the curtailment of production. Rounding error A timeseries describing a rounding error. Metered frequency The timeseries provides information about metered frequency. Adjusted TTC to the The exchange program between two areas which guarantees that the Margin nominal criteria

Available for Cross-Zonal Trade (MACZT) fulfils the nominal criteria at least on the most limiting Critical Network Element with Contingency (CNEC) which limits the transfer capacity. Adjusted TTC to the The exchange program between two areas which allows the Margin Available nominal criteria with for Cross-Zonal Trade (MACZT) on, at least, the most limiting Critical Network TSOs limitation Element with Contingency (CNEC) to get closer to the nominal criteria fulfilment with a limited impact on the rest of the network. Frequency deviation Total time in which the absolute value of the instantaneous frequency deviation larger than standard was larger than the standard frequency deviation. deviation Frequency deviation Total time in which the absolute value of the instantaneous frequency deviation larger than maximum was larger than the maximum instantaneous frequency deviation. deviation Frequency deviation not Number of events in which the absolute value of the instantaneous frequency returned to 50% deviation of the synchronous area exceeded 200 % of the standard frequency deviation as stated in SO GL (EU) regulation Art 131.1.a.vi. Frequency deviation not Number of events in which the absolute value of the instantaneous frequency returned to restoration deviation of the synchronous area exceeded 200 % of the standard frequency range deviation. Frequency deviation Number of events for which the absolute value of the instantaneous frequency outside recovery range deviation was outside of the frequency recovery range. Frequency A time series describing measurement frequency. Mean value A time series describing mean values. Standard deviation A time series describing standard deviation. Percentile A time series describing percentiles. Measured frequency A time series describing the resolution of a measured frequency. resolution Accuracy A time series describing measurement accuracy. FRCE outside level 1 The number of time intervals in which the average value of the FRCE was range outside the Level 1 FRCE range as stated in SO GL. (EU) regulation Art 131.1.b.i. FRCE outside level 2 The number of time intervals in which the average value of the FRCE was range outside the Level 2 FRCE range as stated in SO GL (EU) regulation. Art 131.1.b.i.

FRCE exceeded 60% of FRR The number of events for which the FRCE exceeded 60 % of the reserve capacity capacity on FRR as stated in SO GL (EU) regulation Art 131.1.b.ii. FRCE exceeded steady The number of events for which the absolute value of the FRCE exceeded the state deviation maximum steady-state frequency deviation. FRCE calculation and A time series describing how FRCE is calculated and its accuracy. accuracy descriptor Forecasted capacity A time series describing forecasted capacity. Minimum available A time series describing minimum available capacity. capacity Average available A time series describing average available capacity. capacity Maximum available A time series describing maximum available capacity. capacity Frequency and accuracy A time series describing how system frequency and accuracy are determined. descriptor Long-Term internal A time series describing long-term redispatch to relieve Scheduling Area redispatch internal congestion. Other unavailability This is an unplanned unavailability. Not considered by market participants as a planned maintenance. Faster than standard FAT Bids that can support a ?Full Activation Time? (FAT) that is faster than standard FAT. Fast activation can be done for bids with activation time shorter than the minimum requirement for the standard product. When circumstances call for it, the TSO can order activation of such bids on a shorter notice. Faster than standard Fast deactivation can be done for bids with activation time shorter than the deactivation time minimum requirement for the standard product. When circumstances call for it, the TSO can order activation of such bids on a shorter notice. Slower than standard FAT Bids that can support a Full Activation Time (FAT) that is slower than standard FAT. Remedial action cost A timeseries summarizing all incurred costs and/or revenues per party related summary to the activated remedial actions eligible to Cost Sharing. Settlement result A timeseries representing the cost sharing settlement results between parties (i.e. which party will be paying and/or receiving money for the concerned period).

Reserved cross zonal The reserved cross zonal capacity for system operator needs (EBGL art. 38(1) capacity (b) and art. 41.). Energy reserves A timeseries describing energy reserves. Combined dynamic The time series describes a combined dynamic constraint, being defined as a constraint limit on the sum of power flows on a set of network elements or partial flows on a set of network elements for the purpose to respect dynamic stability limits. Anonymised constraint The time series describes an anonymised constraint. Synchronous condenser Activates a number of generators or other units in synchronous condenser activation mode. Permission A timeseries describing permission details such as authorisations, licenses or administration details related regulatory requirements. Maximum capacity value Maximum flow on an AC border between two areas (capacity) of AC border flow Initial forecast value Initial flow on an AC border between two areas (forecast) of AC border flow

DirectionTypeList ESMPEnumerations The coded identification of the direction of energy flow. UP Up signifies that the available power can be used by the Purchasing area to increase energy. DOWN Down signifies that the available power can be used by the Purchasing area to decrease energy. UP and DOWN Up and Down signifies that the UP and Down values are equal. Stable The direction at a given instant in time is considered to be stable.

StatusTypeList ESMPEnumerations The condition or position of an object with regard to its standing. Intermediate The document is in a non finalized state. Final The document is in a definitive state.

Deactivated The object being reported has been deactivated. Reactivated The object being reported has been reactivated. Active The object being reported is currently active. Available The volumes (one or more) are available. Activated The quantities in the time series have been activated. In process The quantities in the time series are in the process of activation (an activation request has been made). Cancelled The tender indicated in the time series has been completely cancelled. In this case the resources are no longer available to all Acquiring System Operators Ordered The quantities in the time series are to be activated. Unavailable The volumes (one or more) are unavailable. RGCE agreed The information has been agreed within the ENTSO-E Regional Group Continental Europe process. Withdrawn The information or action has been withdrawn by the submitter. Creation The action requested to be carried out is the creation of a new object. Update The action requested to be carried out is the update an existing object. Deactivation The action requested to be carried out is to deactivate an existing object. Reactivation The action requested to be carried out is to reactivate a previously deactivated object. Preventive The remedial action is applied to prevent an outage. Curative The remedial action is applied after an outage has occurred, in order to maintain the operational security. Automatic The remedial action being described is applied by an automation when an outage occurs.

Open The action being described consists of disconnecting the network element to the transmission network. Close The action being described consists of connecting the network element to the transmission network. Stop The action being described consists of stopping the production or consumption connected to a network element. Start The action being described consists of starting the production or consumption connected to a network element. Relative The quantity being described is a relative value to an initial state. Absolute The quantity being described is an absolute value. Curative or preventive The remedial action can be applied to prevent an outage or after an outage has occurred in order to maintain the operational security. Unshared bid Used to indicate that the bid cannot be shared. Pre Processed to be process Substituted Substituted pre-processing data. Modified Modified pre-processing data by RSC or CGMA platform. Result Result Not satisfied The need described in the time series cannot be satisfied. Rejected The document rejected by one or more parties. Preliminary Indicative information only for initial planning purposes. Planned Is planned. Confirmed The status is confirmed. Shall Be Used The object defined in the series shall be used. Could Be Used The object defined in the series could be used.

Proposed The status of the information is proposed. Individual Network Data The network data provided in the document or series concerns the the unique TSO area describes by the document or series. Common Network Data The network data provided in the document or series concerns the whole area describes by the document or series. Setpoint schedule The code for the power setpoint mode of operation of the HVDC link. Proportional external The code for the proportional external signal mode of operation of the HVDC signal link. AC emulation The code for the AC emulation mode of operation of the HVDC link. Importing element An importing network element in which the flow measurement enters. Exporting element An exporting network element from which the flow measurement comes out. To be optimized Describes an element which needs to be optimized by an optimization process. To be monitored Describes an element which needs to be monitored by an optimization process. To be included in Describes an element which needs to be taken into account in a capacity capacity calculation calculation process. Relative to previous The quantity being described is a relative value to a previous point in time. point in time For flow optimization Describes an element which needs to be optimized by a flow optimization process. For voltage optimization Describes an element which needs to be optimized by a voltage optimization process. Presolved Describes an active constraint that limits the exchanges. It is part of the presolved domain. Not available if linked Bid not available if linked bid activated. bid activated Not available if linked Bid not available if linked bid rejected. bid rejected

Not available for DA if Bid not available for direct activation if linked bid subject to direct activation. linked bid subject to DA Not available for DA if Bid not available for direct activation if linked bid subject to scheduled linked bid subject to SA activation. Not available if linked Bid not available if linked bid subject to scheduled activation. bid subject to SA Not available if linked Bid not available if linked bid subject to direct activation. bid subject to DA Primary market A value is traded for the first time. Secondary market A value is traded for the second or next times between two parties. Interesting Describes an asset which is considered as interesting. Relevant Describes an asset which is considered as relevant. Conditionally available Bid available as long as none of the conditions associated with the linked bids materialise. Conditionally Bid unavailable as long as none of the conditions associated with the linked unavailable bids materialise. Available if linked bid Bid available if linked bid activated. activated Available if linked bid Bid available if linked bid rejected. rejected Available if linked bid Bid available if linked bid subject to scheduled activation. subject to SA Available if linked bid Bid available if linked bid subject to direct activation. subject to DA Available for DA if Bid available for direct activation if linked bid subject to direct activation. linked bid subject to DA Available for DA if Bid available for direct activation if linked bid subject to scheduled activation. linked bid subject to SA Delta Describes a status representing a difference between two values. Validated The object or action was validated.

Invalid The object or action was invalid. Timed out The object or action was timed out.

UnitOfMeasureTypeList ESMPEnumerations (synonym MeasurementUnit) The unit of measure that is applied to a quantity. The measurement units shall be in compliance with UN/ECE Recommendation 20. ampere The unit of electrical current in the International system of Units (SI) equivalent to one Coulomb per second. megavolt-ampere MVA unit as per UN/CEFACT recommendation 20. One A unit for dimensionless quantities, also called quantities of dimension one. cubic hectometres A unit of volume equal to one million cubic metres. kilovolt kV unit as per UN/CEFACT recommendation 20. kilometre km unit as per UN/CEFACT recommendation 20. kilovolt ampere reactive A unit of electrical reactive power represented by a current of one thousand amperes flowing due to a potential difference of one thousand volts where the sine of the phase angle between them is 1. The unity power factor is expressed in thousands of a volt ampere reactive. gigawatt hour GWh unit as per UN/CEFACT recommendation 20. kilowatt hour A total amount of electrical energy transferred or consumed in one hour. kilowatt A unit of bulk power flow, which can be defined as the rate of energy transfer /consumption when a current of 1000 amperes flows due to a potential of 1000 volts at unity power factor expressed in thousands of a watt. megavolt ampere reactive Total amount of reactive power across a power system. hours megavolt ampere reactive A unit of electrical reactive power represented by a current of one thousand amperes flowing due to a potential difference of one thousand volts where the sine of the phase angle between them is 1. megawatt A unit of bulk power flow, which can be defined as the rate of energy transfer /consumption when a current of 1000 amperes flows due to a potential of 1000

volts at unity power factor expressed in millions of a watt. cubic metres per second The volume flow rate of cubic metre per second. cubic metre A Cubic metre. metre The length of a metre. megawatt hours The total amount of bulk energy transferred or consumed. percent A unit of proportion equal to 0.01. gigawatt GW unit as per UN/CEFACT recommendation 20. watt The watt is the International System of Units (SI) standard unit of power (energy per unit time), the equivalent of one joule per second. meter per second A unit of measurement of the speed expressed in m/s. degree (unit of angle) A unit of measurement of angles expressed in a 0 to 360 degree gradient. watt per square meter A unit of measurement of the density of heat flow rate expressed in watt per square meter. Celsius A unit of measurement of temperature expressed in degree Celsius. OKTA unit A unit of measurement of the cloudiness expressed in OKTA or OCTA, i.e. A unit of count defining the number of eighth-parts as a measure of the celestial dome cloud coverage. hectopascal A unit of measurement of the pressure expressed in hectopascal. millimeter A unit of measurement of length expressed in millimeter. K (Kelvin) Temperature unit refer ISO 80000-5 (Quantities and units, Part 5: Thermodynamics). Hertz HTZ unit as per UN/CEFACT recommendation 20. Megawatt per Hertz A unit of energy expressed as the load change in million watts that will cause a frequency shift of one hertz. millihertz A unit of frequency equal to 0.001 cycle per second.

minute A period of time equal to sixty seconds. second A period of time equal to one second.

Compound types

ESMP_DateTimeInterval ESMPDataTypes This datatype enables to express the start date and time, and the end date and time of a time interval with a specific pattern. This pattern is the YYYY-MM-DDThh:mmZ. start 1..1 YMDHM_DateTime The start date and time of the interval with a minute resolution. end 1..1 YMDHM_DateTime The end date and time of the interval with a minute resolution.

Datatypes

YMDHM_DateTime ESMPDataTypes In ESMP, the date and time is expressed as "YYYY-MM-DDThh:mmZ", which conforms with ISO 8601 UTC time zone. This date and time is without the seconds. value 1..1 DateTime The date and time as "YYYY-MM- DDThh:mmZ", which conforms with the ISO 8601 UTC time zone.

ESMP_DateTime ESMPDataTypes In ESMP, the dateTime shall be expressed in UTC as YYYY-MM-DDThh:mm:ssZ. value 1..1 DateTime Main Core value Space.

AreaID_String ESMPDataTypes The coded identification of a domain, i.e. balance area, grid area, etc. In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for

domain identification. codingScheme 1..1 CodingSchemeTypeList DomainQualification. value 1..1 String Main Core value Space.

ID_String ESMPDataTypes A code to uniquely distinguish one occurrence of an entity from another. In the ESMP context, the code is defined either by: - an emitting company that provides an agreed identification unique within a business context such as capacity auction identification, market agreement identification, etc. - a party (originator of the exchange) that provides a unique identification in the framework of a business exchange such as document identification, time series identification, bid identification, ... value 1..1 String Main Core value Space.

ESMPVersion_String ESMPDataTypes In ESMP, the coded value is restricted to digits. A code that distinguishes one evolution of an identified object from another. Information about a specific object may be sent several times, each transmission being identified by a different version number. value 1..1 String Main Core value Space.

ProcessKind_String ESMPDataTypes The coded identification of the nature of process. value 1..1 ProcessTypeList Main Core value Space.

MarketRoleKind_String ESMPDataTypes The identification of the role played by a party. value 1..1 RoleTypeList Main Core value Space.

PartyID_String ESMPDataTypes

The identification of an actor in the energy market. In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for market participant identification. codingScheme 1..1 CodingSchemeTypeList DomainQualification. value 1..1 String Main Core value Space.

MessageKind_String ESMPDataTypes The coded type of a document. value 1..1 MessageTypeList Main Core value Space.

Position_Integer ESMPDataTypes An integer value, this value is used as a sequential value representing the relative position of an entity within a space such as a time interval. value 1..1 Integer Main Core value Space.

Quality_String ESMPDataTypes The coded identification of the quality of the information. value 1..1 QualityTypeList Main Core value Space.

ReasonCode_String ESMPDataTypes The coded motivation of an act. value 1..1 ReasonCodeTypeList Main Core value Space.

ReasonText_String ESMPDataTypes The textual explanation of an act as a string of characters. value 1..1 String Main Core value Space.

BusinessKind_String ESMPDataTypes The coded identification of the business type. value 1..1 BusinessTypeList Main Core value Space.

DirectionKind_String ESMPDataTypes The coded identification of the direction of energy flow. value 1..1 DirectionTypeList Main Core value Space.

Status_String ESMPDataTypes The identification of the status of an object. value 1..1 StatusTypeList Main Core value Space.

MeasurementUnitKind_String ESMPDataTypes The coded identification of a unit of measure that is applied to a quantity. The measurement units shall be in compliance with UN/ECE Recommendation 20. value 1..1 UnitOfMeasureTypeList Main Core value Space.

ResourceID_String ESMPDataTypes The identification of a resource object in the energy market. In the ESMP context, it is an authorized issuing office that provides an agreed identification coding scheme for resources (generator, lines, substations, etc.) identification. codingScheme 1..1 CodingSchemeTypeList DomainQualification. value 1..1 String Main Core value Space.